1	Introduction	2

1.1	Purpose and Scope	2

2	Relationship diagram	2

3	Tables	3

3.1	Global Settings Table	3

3.2	Agent Look Up Table	5

3.3	IAP Table	6

3.4	Dial Out ISP Service Table	7

3.5	Secure Socket Table	10

3.6	VPN Service Record Table	10

3.7	Virtual Bearer Table	11

3.8	LAN Service Table	11

3.9	WLAN Service Table	13

3.10	WCDMA Packet Service Table (Outgoing and Incoming GPRS Table)	13

3.11	CDMA2000 Packet Service Table	15

3.12	Default WCDMA Table	18

3.13	Default CDMA2000 Settings Table	19

3.14	Dial In ISP Service Table	20

3.15	Connection Preference Table	21

3.16	Network Table	21

3.17	Modem Bearer Table	22

3.18	LAN Bearer Table	24

3.19	Location Table	25

3.20	Chargecard Table	25

3.21	Proxies Table	25

3.22	WAP Access Point Table	26

3.23	WAP IP Bearer Table	26

3.24	WAP SMS Bearer Table	26

3.25	Access Point Table (CCDAccessPointRecord)	26

3.26	IAP Priority Selection Policy Table (CCDIAPPrioritySelectionPolicyRecord)	27

4	Further Information	28

4.1	People	28

4.2	References	28

4.3	Open Issues	28

4.4	Glossary	28

4.5	Document History	28

# Introduction

## Purpose and Scope

Briefly (i.e. 3-5 lines):

- What this document is for;

- In what situations is it used;

- Who must use it

# Relationship diagram

# Tables

## Global Settings Table

The new entries that will be stored in the Global Settings table are presented in table 2-1. They represent CDMA SimpleIP and MobileIP capability parameters and cannot be overwritten through provisioning.

| Defined Constant | Type | Possible values | Description |
| --- | --- | --- | --- |
| WAP_ACCESS_POINT | TUint32 |  | Identifier of default WAP Access Point |
| REDIAL_ATTEMPTS | TUint32 |  | Number of times to attempt redial when ISP number is busy |
| SMS_BEARER | TUint32 |  | The bearer for sending SMS. |
| GPRS_ATTACH_MODE | TUint32 |  | The GPRS attach mode, i.e.attach on boot up of device or at a later time. |
| ACCEPT_INCOMING_GPRS | TUint32 |  | Whether or not to allow incoming requests for PDP context activation to be automatically accepted (when there is a waiting application). |
| GPRS_CLASS_C_BEARER | TUint32 |  | The preferred bearer when device is forced into GPRS Class C operation. |
| CONNECTION_ATTEMPTS | TUint32 |  | The number of connection attempts (preferences) to make. This is limited to a maximum of 2. |
| MODEM_DATA_FAX | TUint32 |  | Specifies the id of the modem record to be used for data transactions. |
| MODEM_PHONE_SERVICES_SMS | TUint32 |  | Specifies the id of the modem record to be used for phone services such as sms. |
| LOCATION_DATA_FAX | TUint32 |  | Specifies the location to be used with the default modem for data transactions. |
| LOCATION_PHONE_SERVICES_SMS | TUint32 |  | Specifies the location to be used with default modem for phone services such as sms. |
| SMS_RECEIVE_MODE | TUint32 |  | Takes the values of RMobileSmsMessaging::TMobileSmsReceiveMode |
| DEFAULT_NETWORK | Tuint32 |  | Identifier of the network that is the users preferred |
| BEARER_AVAILABILITY_CHECK_TSY | Text |  | Name of the tsy used by default to check for availability of services |
| CDMA_OP_CAPABILITY | TUint32 |  | Operation capability bitmap to indicate which operations are supported by the mobile station. The possible values are presented in tables 2.2 and 2.3 |
| CDMA_SIMIP_MAX_NUM_NAI | TUint32 |  | Maximum SimpleIP NAI entries |
| CDMA_SIMIP_MAX_NAI_LENGTH | TUint32 |  | Maximum SimpleIP NAI length (min 72 octets) |
| CDMA_SIMIP_MAX_SS_LENGTH | TUint32 |  | Maximum length of SimpleIP shared secret data. |
| CDMA_SIMIP_AUTH_SUPPORTED | TUint32 |  | SimpleIP authentication algorithm bitmap supported by the mobile station. Possible values are presented in tables 2-4 and 2-5. |
| CDMA_MIP_MAX_NUM_NAI | TUint32 |  | Maximum MobileIP NAI entries |
| CDMA_MIP_MAX_NAI_LENGTH | TUint32 |  | Maximum MobileIP NAI length (min 72 octets) |
| CDMA_MIP_MAX_MN_AAA_SS_LENGTH | TUint32 |  | Maximum length of the shared secret data (MN-AAA) |
| CDMA_MIP_MN_AAA_AUTH_ALGORITHM | TUint32 |  | Authentication algorithm bitmap (MN-AAA) supported by the mobile station. Possible values are presented in tables 2-6 and 2-7. |
| CDMA_MIP_MAX_MN_HA_SS_LENGTH | TUint32 |  | Maximum length of the shared secret data (MN-HA) |
| CDMA_OP_CAPABILITY | TUint32 | Bit fields(1-8) SimpleIP(Value = 1) ECommDbCdmaOpCapabilitySimpleIp SimpleIP supported MobileIP(2) ECommDbCdmaOpCapabilityMobileIp MobileIP supported MobileIP with SimpleIP fallback(4) ECommDbCdmaOpCapabilityFallback MobileIP with SimpleIP fallback supported Reserved 5 higher bits are reserved | Operation capability bitmap to indicate which operations are supported by the mobile station. The possible values are presented in tables 2.2 and 2.3 |
| CDMA_SIMIP_MAX_NUM_NAI | TUint32 |  | Maximum SimpleIP NAI entries |
| CDMA_SIMIP_MAX_NAI_LENGTH | TUint32 |  | Maximum SimpleIP NAI length (min 72 octets) |
| CDMA_SIMIP_MAX_SS_LENGTH | TUint32 |  | Maximum length of SimpleIP shared secret data. |
| CDMA_SIMIP_AUTH_SUPPORTED | TUint32 | Bit fields(1-8) PPP_CHAP(1) ECommDbCdmaSimpIpCapabilityChap CHAP supported PPP_PAP(2) ECommDbCdmaSimpIpCapabilityPap PPP supported Reserved 6 higher bits are reserved | SimpleIP authentication algorithm bitmap supported by the mobile station. Possible values are presented in tables 2-4 and 2-5. |
| CDMA_MIP_MAX_NUM_NAI | TUint32 |  | Maximum MobileIP NAI entries |
| CDMA_MIP_MAX_NAI_LENGTH | TUint32 |  | Maximum MobileIP NAI length (min 72 octets) |
| CDMA_MIP_MAX_MN_AAA_SS_LENGTH | TUint32 |  | Maximum length of the shared secret data (MN-AAA) |
| CDMA_MIP_MN_AAA_AUTH_ALGORITHM | TUint32 | Bit fields(1-8) MD5(1) ECommDbCdmaMIpMd5 MD5 authentication supported (RFC 3012) Reserved 7 7 higher bits are reserved | Authentication algorithm bitmap (MN-AAA) supported by the mobile station. Possible values are presented in tables 2-6 and 2-7. |
| CDMA_MIP_MAX_MN_HA_SS_LENGTH | TUint32 |  | Maximum length of the shared secret data (MN-HA) |
| CDMA_MIP_MN_HA_AUTH_ALGORITHM | TUint32 |  | Authentication algorithm bitmap (MN-HA) supported by the mobile station. Possible values are presented in tables 2-8 and 2-9. |
| DEFAULT_AP | TUint32 |  | Default access point on an access point aware system. |
| PROMPT_FOR_AP | TBool | FALSE / TRUE | Access point prompting switch. |

Table 3-1: Global settings table entries

## Agent Look Up Table

| Defined Constant | Type | Null? | Possible values | Description |
| --- | --- | --- | --- | --- |
| AGENT_FRIENDLY_NAME | Text | Yes |  | Agent Friendly Name |
| AGENT_FILENAME | Text | Yes |  | Agent Filename |

## IAP Table

| Defined Constant | Type | Null? | Possible values | Description |
| --- | --- | --- | --- | --- |
| IAP_SERVICE | TUint32 | no |  | Identifier of service in this IAP |
| IAP_SERVICE_TYPE | Text | no |  | Name of service table in this IAP |
| IAP_BEARER | TUint32 | no |  | Identifier of bearer in this IAP |
| IAP_BEARER_TYPE | Text | no |  | Name of bearer table in this IAP |
| IAP_NETWORK | TUint32 | No |  | Identifier of the network which this IAP connects to |
| IAP_NETWORK_WEIGHTING | TUint32 | No |  | Weighting of IAP in relation to other IAPs in same network |
| IAP_LOCATION | TUint32 | yes |  | Identifier of location in this IAP |

## Dial Out ISP Service Table

| Defined Constant | Type | Null? | Possible values |  | Description |
| --- | --- | --- | --- | --- | --- |
| ISP_DESCRIPTION | Text | yes |  |  | Application’s description of ISP |
| ISP_TYPE | TUint32 | yes |  | Value | Enum value indicating ISP type, e.g. Internet ISP, WAP ISP. Selected from TcommsDbIspType |
|  |  |  | EIspTypeInternetOnly | 0 | ISP internet only |
|  |  |  | EIspTypeWAPOnly | 1 | ISP WAP only |
|  |  |  | EIspTypeInternetAndWAP | 2 | ISP internet and WAP |
| ISP_DEFAULT_TEL_NUM | Text | yes |  |  | Default phone number |
| ISP_DIAL_RESOLUTION | TBool | no |  |  | Perform dialling resolution for default phone number? |
| ISP_CHARGECARD | TUint32 | yes |  |  | Identifier of the chargecard in this ISP |
| ISP_USE_LOGIN_SCRIPT | TBool | no |  |  | Use login script? |
| ISP_LOGIN_SCRIPT | Long Text | yes |  |  | Login script (used if Use Login Script is true) |
| ISP_PROMPT_FOR_LOGIN | TBool | no |  |  | Prompt user for username and password? |
| ISP_LOGIN_NAME | Text | yes |  |  | Login name |
| ISP_LOGIN_PASS | Text | yes |  |  | Login Password |
| ISP_DISPLAY_PCT | TBool | yes |  |  | Display PCT (without scanning script for READ command)? |
| ISP_IF_NAME | Text | no |  |  | Interface name, e.g. "PPP" or "SLIP" |
| ISP_IF_ PARAMS | Text | yes |  |  | Interface parameter string |
| ISP_IF_NETWORKS | Text | yes |  |  | Comma separated list of network protocols, e.g. "PPP" |
| ISP_IF_PROMPT FOR AUTH | TBool | no |  |  | Prompt user for authentication username and password? |
| ISP_IF_AUTH NAME | Text | yes |  |  | Authentication username used by PPP |
| ISP_IF_AUTH PASS | Text | yes |  |  | Authentication password used by PPP |
| ISP_IF_AUTH_RETRIES | TUint32 | yes |  |  | Number of times to retry authentication if it fails |
| ISP_IF_CALLBACK_ENABLED | TBool | yes |  |  | Callback enabled? |
| ISP_IF_CALLBACK_TYPE | TUint32 | yes |  | Value | Enum value indicating type of callback (if enabled). Selected from TcallbackAction. |
|  |  |  | IETF0 | 0 |  |
|  |  |  | IETF1 | 1 |  |
|  |  |  | IETF2 | 2 |  |
|  |  |  | IETF3 | 3 |  |
|  |  |  | IETF4 | 4 |  |
|  |  |  | IETF5 | 5 |  |
|  |  |  | MSCBCPCLIENTNUM | 1001 |  |
|  |  |  | MSCBCPSERVERNUM | 1002 |  |
|  |  |  | MSCBCPOVERRRIDESERVERNUM | 1003 |  |
| ISP_IF_CALLBACK_INFO | 8-Bit Text | yes |  |  | Info for callback request (if enabled) |
| ISP_CALLBACK_TIMEOUT | TUint32 | yes |  |  | Time to wait for callback in microseconds (if enabled) |
| ISP_IF_SERVER_MODE | TBool | yes |  |  | When PPP is configured as a server. |
| ISP_IP_ADDR_FROM_SERVER | TBool | no |  |  | Get IP address from server? |
| ISP_IP_ADDR | Text | yes |  |  | IP address of interface |
| ISP_IP_NET MASK | Text | yes |  |  | IP net mask of interface |
| ISP_IP_GATEWAY | Text | yes |  |  | IP address of gateway |
| ISP_IP_DNS_ADDR_FROM_SERVER | TBool | no |  |  | Get DNS addresses from server? |
| ISP_IP_NAME_SERVER1 | Text | yes |  |  | IP Address of primary name server |
| ISP_IP_NAME_SERVER2 | Text | yes |  |  | IP Address of secondary name server |
| ISP_IP6_DNS_ADDR_FROM_SERVER | TBool | yes |  |  | Get IP6 DNS addresses from server? |
| ISP_IP6_NAME_SERVER1 | Text | yes |  |  | IP6 Address of primary name server |
| ISP_IP6_NAME_SERVER2 | Text | yes |  |  | IP6 Address of secondary name server |
| ISP_IP_ADDR_LEASE_VALID_FROM | Text | yes |  |  | IP address valid from this time, used to store dynamically assigned address lease info |
| ISP_IP_ADDR_LEASE_VALID_TO | Test | yes |  |  | IP address valid for use until this time, used to store dynamically assigned address lease info |
| ISP_CONFIG_DAEMON_MANAGER_NAME | Text | yes |  |  | Name of the ECOM configuration daemon manager component. This component interfaces with the server identified in ISP_CONFIG_DAEMON_NAME. If specified, ISP_CONFIG_DAEMON_NAME should also be specified. |
| ISP_CONFIG_DAEMON_NAME | Text | yes |  |  | Name of the configuration daemon server. This server is used to provide further configuration for a connection, e.g. dynamic IP address assignment. If specified, ISP_CONFIG_DAEMON_MANAGER_NAME should also be specified. |
| ISP_ENABLE_IP_HEADER_COMP | TBool | yes |  |  | Enable IP header compression? |
| ISP_ENABLE_LCP_EXTENSIONS | TBool | yes |  |  | Enable LCP extensions? |
| ISP_DISABLE_PLAIN_TEXT_AUTH | TBool | yes |  |  | Disable plain text authentication? |
| ISP_ENABLE_SW_COMP | TBool | yes |  |  | Enable software compression (e.g. Microsoft, STAC or Predictor compression)? |
| ISP_BEARER_NAME | TUint32 | yes |  |  | Enum value specifying the name of the bearer, e.g. Asynchronous Modem. Selected from RbasicGsmCall:: TbearerName |
| ISP_BEARER_SPEED | TUint32 | yes |  |  | Enum value specifying the bearer speed from RBasicGsmCall::TbearerSpeed |
| ISP_BEARER_CE | TUint32 | yes |  |  | Enum value specifying the bearer CE, e.g. non-transparent. Selected from RbasicGsmCall:: TBearerCE |
| ISP_INIT_STRING | 8-bit Text | yes |  |  | Modem initialisation string to be used when this ISP is in use. This string is copied into the MODEM_ISP_INIT_STRING field by NetDial. |
| ISP_BEARER_TYPE | TUint32 | yes |  |  | Enum value indicating connection type, e.g. Standard GSM, HSCSD. Selected from TcommsDbBearerType |
| ISP_CHANNEL_CODING | TUint32 | yes |  |  | Enum value specifying HSCSD channel coding. Selected from RbasicGsmCall:: TaiurCodingsCaps |
| ISP_AIUR | TUint32 | yes |  |  | Enum value specifying AIUR for HSCSD. Selected from RbasicGsmCall:: Taiur |
| ISP_REQUESTED_TIME_SLOTS | TUint32 | yes |  |  | Requested number of time slots for HSCSD. |
| ISP_MAXIMUM_TIME_SLOTS | TUint32 | yes |  |  | Maximum number of time slots for HSCSD which could be requested during this connection. |
| ISP_BEARER_SERVICE | TUint32 | yes |  |  | Enum of type RMobileCall::TmobileCallDataService |
| ISP_BEARER_PROTOCOL | TUint32 | yes |  |  | Enum of type RMobileCall::TmobileCallDataProtocol |
| ISP_RLP_VERSION | TUint32 | yes |  |  | Enum of type RMobileCall::TMobileCallDataRLPVersion |
| ISP_IWF_TO_MS | TUint32 | yes |  |  | Integer specifying network to MS window size |
| ISP_MS_TO_IWF | TUint32 | yes |  |  | Integer specifying MS to network window size |
| ISP_ACK_TIMER | TUint32 | yes |  |  | Integer specifying rlp ack timeout in microseconds |
| ISP_RETRANSMISSION_ATTEMPTS | TUint32 | yes |  |  | Integer specifying rlp retransmission attempts |
| ISP_RESEQUENCE_PERIOD | TUint32 | yes |  |  | Integer specifying resequencing period |
| ISP_V42_COMPRESSION | TUint32 | yes |  |  | Enum of type TMobileCallDataV42bis |
| ISP_V42_CODEWORDS | TUint32 | yes |  |  | Integer specifying V42bis codewords |
| ISP_V42_MAX_LENGTH | TUint32 | yes |  |  | Integer specifying V42bis maximum string length |
| ISP_ASYMMETRY | TUint32 | yes |  |  | Enum of type TmobileCallAsymmetry for HSCSD |
| ISP_USER_INIT_UPGRADE | TBool | Yes |  |  | HSCSD parameter - user init upgrade |
| ISP_USE_EDGE | TBool | Yes |  |  | EDGE parameter used by GSM |

## Secure Socket Table

| Defined Constant | Type | Null? | Possible values | Description |
| --- | --- | --- | --- | --- |
| SS_PROTO_NAME | Text |  |  | ProtocolName |
| SS_PROTO_LIBRARY | Text |  |  | ProtoLibrary |

## VPN Service Record Table

| Defined Constant | Type | Null? | Possible values | Description |
| --- | --- | --- | --- | --- |
| VPN_SERVICE_POLICY | Text |  |  | Policy |
| VPN_SERVICE_IAP | TUint32 |  |  | HomeIAP |
| VPN_SERVICE_NETWORKID | TUint32 |  |  | HomeNetwork |
| VPN_SERVICE_ENABLE_LLMNR | TBool |  |  | LLMNR |

Links:

VPN_SERVICE_IAP

VPN_SERVICE_NETWORKID

## Virtual Bearer Table

| Defined Constant | Type | Null? | Possible values | Description |
| --- | --- | --- | --- | --- |
| VIRTUAL_BEARER_AGENT | TUInt32 |  |  | AGENT_NAME |
| VIRTUAL_BEARER_NIF | Text |  |  | IF_NAME |
| LAST_SOCKET_ACTIVITY_TIMEOUT | TUInt32 |  |  | LastSocketActivityTimeout |
| LAST_SESSION_CLOSED_TIMEOUT | TUInt32 |  |  | LastSessionClosedTimeout |
| LAST_SOCKET_CLOSED_TIMEOUT | TUInt32 |  |  | LastSocketClosedTimeout |

## LAN Service Table

| Defined Constant | Type | Null? | Possible values | Description |
| --- | --- | --- | --- | --- |
| LAN_IF_NETWORKS | Text | yes |  | Comma separated list of network protocols, e.g. "PPP" |
| LAN_IP_NETMASK | Text | yes |  | IP net mask of interface |
| LAN_IP_GATEWAY | Text | yes |  | IP address of gateway |
| LAN_IP_ADDR_FROM_SERVER | TBool | no |  | Get IP address from server? |
| LAN_IP_ADDR | Text | yes |  | IP address of interface |
| LAN_IP_DNS_ADDR_FROM_SERVER | TBool | no |  | Get DNS addresses from server? |
| LAN_IP_NAME_SERVER1 | Text | yes |  | IP Address of primary name server |
| LAN_IP_NAME_SERVER2 | Text | yes |  | IP Address of secondary name server |
| LAN_IP6_DNS_ADDR_FROM_SERVER | TBool | yes |  | Get IP6 DNS addresses from server? |
| LAN_IP6_NAME_SERVER1 | Text | yes |  | IP6 Address of primary name server |
| LAN_IP6_NAME_SERVER2 | Text | yes |  | IP6 Address of secondary name server |
| LAN_IP_ADDR_LEASE_VALID_FROM | Text | yes |  | IP address valid from this time, used to store dynamically assigned address lease info |
| LAN_IP_ADDR_LEASE_VALID_TO | Test | yes |  | IP address valid for use until this time, used to store dynamically assigned address lease info |
| LAN_CONFIG_DAEMON_MANAGER_NAME | Text | yes |  | Name of the ECOM configuration daemon manager component. This component interfaces with the server identified in ISP_CONFIG_DAEMON_NAME. If specified, ISP_CONFIG_DAEMON_NAME should also be specified. |
| LAN_CONFIG_DAEMON_NAME | Text | Yes |  | Name of the configuration daemon server. This server is used to provide further configuration for a connection, e.g. dynamic IP address assignment. If specified, ISP_CONFIG_DAEMON_MANAGER_NAME should also be specified. |
| LAN_SERVICE_EXTENSION_TABLE_NAME | Text | Yes |  |  |
| LAN_SERVICE_EXTENSION_TABLE_RECORD_ID | TUint32 | Yes |  |  |
| LAN_SERVICE_ENABLE_LLMNR | TBool | Yes |  |  |

## WLAN Service Table

| Defined Constant | Type | Null? | Possible values | Description |
| --- | --- | --- | --- | --- |
| WLAN_SERVICE_CONNECTION_MODE | TUInt32 |  |  | Wlan_Connection_Mode |
| WLAN_SERVICE_SSID | Text |  |  | Wlan_SSID |
| WLAN_SERVICE_CHANNELID | TUInt32 |  |  | Wlan_Channel_ID |
| WLAN_SERVICE_SECURITY_MODE | TUInt32 |  |  | Wlan_Security_Mode |
| WLAN_SERVICE_WEPKEY1 | Text |  |  | Wlan_Wep_Key1 |
| WLAN_SERVICE_WEPKEY2 | Text |  |  | Wlan_Wep_Key2 |
| WLAN_SERVICE_WEPKEY3 | Text |  |  | Wlan_Wep_Key3 |
| WLAN_SERVICE_WEPKEY4 | Text |  |  | Wlan_Wep_Key4 |
| WLAN_SERVICE_DEFAULT_WEPKEY | TUInt32 |  |  | Wlan_Wep_Default_Key |
| WLAN_SERVICE_AUTHENTICATION_MODE | TUInt32 |  |  | Wlan_Authentication_Mode |
| WLAN_SERVICE_WPA_PRESHARE_KEY | Text |  |  | Wlan_Wpa_PreShared_Key |
| WLAN_SERVICE_WPAKEY_LENGTH | TUInt32 |  |  | Wlan_Wpa_Key_Length |
| WLAN_SERVICE_EAPLIST | Text |  |  | Wlan_Eap_List |
| WLAN_SERVICE_WPA_PRESHARE_KEY_USED | TUInt32 |  |  | Wlan_Use_Wpa_PreShared_Key |

## WCDMA Packet Service Table (Outgoing and Incoming GPRS Table)

| Defined Constant | Type | Null? | Possible values |  | Description |
| --- | --- | --- | --- | --- | --- |
| GPRS_APN | Text | yes |  |  | Access point name |
| GPRS_PDP_TYPE | TUint32 | no |  |  | PDP type |
| GPRS_PDP_ADDRESS | Text | yes |  |  | PDP address of phone |
| GPRS_REQ_PRECEDENCE | TUint32 | yes |  |  | Requested quality of service precedence class |
| GPRS_REQ_DELAY | TUint32 | yes |  |  | Requested quality of service delay class |
| GPRS_REQ_RELIABIITY | TUint32 | yes |  |  | Requested quality of service reliability class |
| GPRS_REQ_PEAK_THROUGHPUT | TUint32 | yes |  |  | Requested quality of service peak throughput class |
| GPRS_REQ_MEAN_THROUGHPUT | TUint32 | yes |  |  | Requested quality of service mean throughput class |
| GPRS_MIN_PRECEDENCE | TUint32 | yes |  |  | Minimum quality of service precedence class |
| GPRS_MIN_DELAY | TUint32 | yes |  |  | Minimum quality of service delay class |
| GPRS_MIN_RELIABIITY | TUint32 | yes |  |  | Minimum quality of service reliability class |
| GPRS_MIN_PEAK_THROUGHPUT | TUint32 | yes |  |  | Minimum quality of service peak throughput class |
| GPRS_MIN_MEAN_THROUGHPUT | TUint32 | yes |  |  | Minimum quality of service mean throughput class |
| GPRS_DATA_COMPRESSION | TBool | yes |  |  | Data compression on? |
| GPRS_HEADER_COMPRESSION | TBool | yes |  |  | IP header compression on? |
| GPRS_USE_EDGE | TBool | yes |  |  | EDGE parameter |
| GPRS_ANONYMOUS_ACCESS | TBool | yes |  |  | Use anonymous access on? |
| GPRS_IF_PARAMS | Text | yes |  |  | Interface parameter string |
| GPRS_IF_NETWORKS | Text | yes |  |  | Comma separated list of network protocols, e.g. "PPP" |
| GPRS_IF_PROMPT FOR AUTH | TBool | no |  |  | Prompt user for authentication username and password? |
| GPRS_IF_AUTH NAME | Text | yes |  |  | Authentication username used by PPP |
| GPRS_IF_AUTH PASS | Text | yes |  |  | Authentication password used by PPP |
| GPRS_IF_AUTH_RETRIES | TUint32 | yes |  |  | Number of times to retry authentication if it fails |
| GPRS_IP_NET MASK | Text | yes |  |  | IP net mask of interface |
| GPRS_IP_GATEWAY | Text | yes |  |  | IP address of gateway |
| GPRS_IP_ADDR_FROM_SERVER | TBool | no |  |  | Get IP addresses (for EPOC) from server? |
| GPRS_IP_ADDR | Text | yes |  |  | IP address of EPOC |
| GPRS_IP_DNS_ADDR_FROM_SERVER | TBool | no |  |  | Get DNS addresses from server? |
| GPRS_IP_NAME_SERVER1 | Text | yes |  |  | IP Address of primary name server |
| GPRS_IP_NAME_SERVER2 | Text | yes |  |  | IP Address of secondary name server |
| GPRS_IP6_DNS_ADDR_FROM_SERVER | TBool | yes |  |  | Get IP6 DNS addresses from server? |
| GPRS_IP6_NAME_SERVER1 | Text | yes |  |  | IP6 Address of primary name server |
| GPRS_IP6_NAME_SERVER2 | Text | yes |  |  | IP6 Address of secondary name server |
| GPRS_IP_ADDR_LEASE_VALID_FROM | Text | yes |  |  | IP address valid from this time, used to store dynamically assigned address lease info |
| GPRS_IP_ADDR_LEASE_VALID_TO | Test | yes |  |  | IP address valid for use until this time, used to store dynamically assigned address lease info |
| GPRS_CONFIG_DAEMON_MANAGER_NAME | Text | yes |  |  | Name of the ECOM configuration daemon manager component. This component interfaces with the server identified in ISP_CONFIG_DAEMON_NAME. If specified, ISP_CONFIG_DAEMON_NAME should also be specified. |
| GPRS_CONFIG_DAEMON_NAME | Text | yes |  |  | Name of the configuration daemon server. This server is used to provide further configuration for a connection, e.g. dynamic IP address assignment. If specified, ISP_CONFIG_DAEMON_MANAGER_NAME should also be specified. |
| GPRS_ENABLE_LCP_EXTENSIONS | TBool | yes |  |  | Enable LCP extensions? |
| GPRS_DISABLE_PLAIN_TEXT_AUTH | TBool | yes |  |  | Disable plain text authentication? |
| GPRS_AP_TYPE | TUint32 | yes |  | Value | Service supports Internet only, WAP only or both |
|  |  |  | EIspTypeInternetOnly | 0 |  |
|  |  |  | EIspTypeWAPOnly | 1 |  |
|  |  |  | EIspTypeInternetAndWAP | 2 |  |
| GPRS_QOS_WARNING_TIMEOUT | TUint32 | Yes |  |  | If the requested QOS can not be satisfied warn the user after this time in microseconds. Set to 0xffffffff to disable. |
| GPRS_SERVICE_ENABLE_LLMNR | TBool | Yes |  |  |  |

## CDMA2000 Packet Service Table0

| Defined Constant | Type | Null? | Possible values | Description |
| --- | --- | --- | --- | --- |
| CDMA_FCH_MUX | TUint32 |  |  |  |
| CDMA_SCH_MUX | TUint32 |  |  |  |
| CDMA_IWF_NAME | Text | yes |  | IWF name |
| CDMA_SERVICE_OPTIONS | TUint32 | no |  | Mask indicating valid service options |
| CDMA_PDP_TYPE | TUint32 | no |  | PDP type. (IPv4 or IPv6) |
| CDMA_PDP_ADDRESS | Text | yes |  | PDP address of phone |
| CDMA_REQ_FWD_PRIORITY | TUint32 | yes |  | Requested forward priority |
| CDMA_REQ_REV_PRIORITY | TUint32 | yes |  | Requested reverse priority |
| CDMA_REQ_FWD_BITRATE | TUint32 | yes |  | Requested forward bitrate |
| CDMA_REQ_REV_BITRATE | TUint32 | yes |  | Requested reverse bitrate |
| CDMA_REQ_FWD_LOSS | TUint32 | yes |  | Requested forward frame loss rate |
| CDMA_REQ_REV_LOSS | TUint32 | yes |  | Requested reverse frame loss rate |
| CDMA_REQ_FWD_MAXDELAY | TUint32 | yes |  | Requested forward delay |
| CDMA_REQ_REV_MAXDELAY | TUint32 | yes |  | Requested reverse delay |
| CDMA_MIN_FWD_BITRATE | TUint32 | yes |  | Minimum acceptable forward bitrate |
| CDMA_MIN_REV_BITRATE | TUint32 | yes |  | Minimum acceptable reverse bitrate |
| CDMA_ACCPT_FWD_LOSS | TUint32 | yes |  | Maximum acceptable forward frame loss rate |
| CDMA_ACCPT_REV_LOSS | TUint32 | yes |  | Maximum acceptable forward frame loss rate |
| CDMA_ACCPT_FWD_MAXDELAY | TUint32 | yes |  | Maximum acceptable forward delay |
| CDMA_ACCPT_REV_MAXDELAY | TUint32 | yes |  | Maximum acceptable reverse delay |
| CDMA_DATA_COMPRESSION | TBool | yes |  | Data compression on? |
| CDMA_HEADER_COMPRESSION | TBool | yes |  | IP header compression on? |
| CDMA_ANONYMOUS_ACCESS | TBool | yes |  | Use anonymous access on? |
| CDMA_IF_NAME | Text | no |  | Interface name, e.g. "PPP" or "SLIP" |
| CDMA_IF_PARAMS | Text | yes |  | Interface parameter string |
| CDMA_IF_NETWORKS | Text | yes |  | Comma separated list of network protocols, e.g. "PPP" |
| CDMA_IF_PROMPT_FOR_AUTH | TBool | no |  | Prompt user for authentication username and password? |
| CDMA_IF_AUTH_NAME | Text | yes |  | Authentication username used by PPP |
| CDMA_IF_AUTH_PASS | Text | yes |  | Authentication password used by PPP |
| CDMA_IF_AUTH_RETRIES | TUint32 | yes |  | Number of times to retry authentication if it fails |
| CDMA_IP_NET MASK | Text | yes |  | IP net mask of interface |
| CDMA_IP_GATEWAY | Text | yes |  | IP address of gateway |
| CDMA_IP_ADDR_FROM_SERVER | TBool | no |  | Get IP addresses (for EPOC) from server? |
| CDMA_IP_ADDR | Text | yes |  | IP address of EPOC |
| CDMA_IP_DNS_ADDR_FROM_SERVER | TBool | no |  | Get DNS addresses from server? |
| CDMA_IP_NAME_SERVER1 | Text | yes |  | IP Address of primary name server |
| CDMA_IP_NAME_SERVER2 | Text | yes |  | IP Address of secondary name server |
| CDMA_IP6_DNS_ADDR_FROM_SERVER | TBool | yes |  | Get IP6 DNS addresses from server? |
| CDMA_IP6_NAME_SERVER1 | Text | yes |  | IP6 Address of primary name server |
| CDMA_IP6_NAME_SERVER2 | Text | yes |  | IP6 Address of secondary name server |
| CDMA_IP_ADDR_LEASE_VALID_FROM | Text | yes |  | IP address valid from this time, used to store dynamically assigned address lease info |
| CDMA_IP_ADDR_LEASE_VALID_TO | Test | yes |  | IP address valid for use until this time, used to store dynamically assigned address lease info |
| CDMA_CONFIG_DAEMON_MANAGER_NAME | Text | yes |  | Name of the ECOM configuration daemon manager component. This component interfaces with the server identified in ISP_CONFIG_DAEMON_NAME. If specified, ISP_CONFIG_DAEMON_NAME should also be specified. |
| CDMA_CONFIG_DAEMON_NAME | Text | yes |  | Name of the configuration daemon server. This server is used to provide further configuration for a connection, e.g. dynamic IP address assignment. If specified, ISP_CONFIG_DAEMON_MANAGER_NAME should also be specified. |
| CDMA_ENABLE_LCP_EXTENSIONS | TBool | yes |  | Enable LCP extensions? |
| CDMA_DISABLE_PLAIN_TEXT_AUTH | TBool | yes |  | Disable plain text authentication? |
| CDMA_AP_TYPE | TUint32 | yes |  | Service supports Internet only, WAP only or both |
| CDMA_QOS_WARNING_TIMEOUT | TUint32 | yes |  | Notify user if the requested QOS can not be satisfied for this time in microseconds. Set to 0xffffffff to disable. |
| CDMA_RLP_MODE | TUint32 | no |  | Transparent or non-transparent radio link protocol. (RPacketQoS::TRLPMode) |
| CDMA_NAI_TYPE | TUint32 | No | ECommDbCdmaNaiSimpleIp(Value=0) SimpleIP only ECommDbCdmaNaiMobileIp(1) MobileIP with SimpleIP fallback | Defines the type of NAI this record refers to (SimpleIP or MobileIP). Possible values are presented in table 2-14. |
| CDMA_SIMIP_AUTH_ALGORITHM | TUint32 | yes |  | SimpleIP authentication algorithm. Possible values are presented in table 2-15. |
| CDMA_SIMIP_PAP_SS_HANDLE | TUint32 | yes |  | SimpleIP PAP shared secret handle |
| CDMA_SIMIP_CHAP_SS_HANDLE | TUint32 | yes |  | SimpleIP CHAP shared secret handle |
| CDMA_MIP_T_BIT | TUint32 | yes |  | Reverse-tunnelling required flag. |
| CDMA_MIP_HOME_ADDRESS | Text | yes |  | Home address associated with the Mobile Station |
| CDMA_MIP_PRIMARY_HOME_AGENT | Text | yes |  | IP address of the mobile node’s primary home agent |
| CDMA_MIP_SECONDARY_HOME_AGENT | Text | yes |  | IP address of the mobile node’s secondary home agent |
| CDMA_MIP_MN_AAA_AUTH_ALGORITHM | TUint32 | yes | ECommDbCdmaSimpIpAuthNone(Value=0) No authentication ECommDbCdmaSimpIpAuthChap(1) PPP CHAP authentication ECommDbCdmaSimpIpAuthPap(2) PPP PAP authentication ECommDbCdmaSimpIpAuthFallback(3) PPP CHAP to PAP fallback | Authentication algorithm (MN-AAA). Possible values are presented in table 2-16 |
| CDMA_MIP_MN_AAA_SPI_INDICATOR | TBool | yes |  | Security Parameter Index Indicator (MN-AAA) |
| CDMA_MIP_MN_AAA_SPI | TUint32 | yes |  | Security Parameter Index (MN-AAA) |
| CDMA_MIP_MN_HA_AUTH_ALGORITHM | TUint32 | yes | ECommDbCdmaMIpAuthNone(0) No authentication ECommDbCdmaMIpAuthMd5(1) MD5 authentication (IETF RFC 2002) | Authentication algorithm (MN-HA). |
| CDMA_MIP_MN_HA_SPI_INDICATOR | TBool | yes |  | Security Parameter Index Indicator (MN-HA) |
| CDMA_MIP_MN_HA_SPI | TUint32 | yes |  | Security Parameter Index (MN-HA) |
| CDMA_MIP_MN_AAA_SS_DATA | Text | yes |  | MobileIP MN-AAA shared secret data |
| CDMA_MIP_MN_HA_SS_DATA | Text | Yes |  | MobileIP MN-HA shared secret data |
| CDMA_SERVICE_ENABLE_LLMNR | TBool | Yes |  |  |
| SERVICE_ENABLE_SW_COMP | TUint32 | yes |  |  |
| SERVICE_IF_EXTERN_IP_CONFIG_ALWAYS_REJECT_AUTH | TBool | No | If this field is not set or absent, the result is the same as when it is set to TRUE. | When using external IP configuration (MobileIP or DHCP), always reject PPP peer authentication requests. If external IP configuration is used and this field is set to TRUE then peer authentication requests are always rejected. If external IP configuration is used and this field is set to FALSE then PPP allows authentication to proceed (although it may still fail later). |

## Default WCDMA Table

| Defined Constant | Type | Null? | Possible values | Description |
| --- | --- | --- | --- | --- |
| DEFAULT_GPRS_USAGE | TUint32 | yes |  | Enum to specify usage of this default. |
| DEFAULT_GPRS_APN | Text | no |  | Default access point name |
| DEFAULT_GPRS_PDP_TYPE | TUint32 | no |  | Default PDP type |
| DEFAULT_GPRS_PDP_ADDRESS | Text | no |  | Default PDP address |
| DEFAULT_GPRS_ PRECEDENCE | TUint32 | yes |  | Default quality of service precedence class |
| DEFAULT_GPRS_DELAY | TUint32 | yes |  | Default quality of service delay class |
| DEFAULT_GPRS_RELIABILITY | TUint32 | yes |  | Default quality of service reliability class |
| DEFAULT_GPRS_PEAK_THROUGHPUT | TUint32 | yes |  | Default quality of service peak throughput class |
| DEFAULT_GPRS_MEAN_THROUGHPUT | TUint32 | yes |  | Default quality of service mean throughput class |
| DEFAULT_GPRS_MIN_PRECENDENCE | TUint32 | yes |  | Default minimum quality of service precedence class |
| DEFAULT_GPRS_MIN_DELAY | TUint32 | yes |  | Default minimum quality of service delay class |
| DEFAULT_GPRS_MIN_RELIABILITY | TUint32 | yes |  | Default minimum quality of service reliability class |
| DEFAULT_GPRS_MIN_PEAK_THROUGHPUT | TUint32 | yes |  | Default minimum quality of service peak throughput class |
| DEFAULT_GPRS_MIN_MEAN_THROUGHPUT | TUint32 | yes |  | Default minimum quality of service mean throughput class |
| DEFAULT_GPRS_DATA_COMPRESSION | TBool | yes |  | Data compression on/off by default |
| DEFAULT_GPRS_HEADER_COMPRESSION | TBool | yes |  | IP header compression on/off by default |
| DEFAULT_GRRS_USE_EDGE | TBool | yes |  | EDGE parameter |
| DEFAULT_GPRS_ANONYMOUS_ACCESS | TBool | yes |  | Use anonymous access on/off by default |

## Default CDMA2000 Settings Table0

| Defined Constant | Type | Null? | Possible values | Description |
| --- | --- | --- | --- | --- |
| DEFAULT_CDMA_USAGE | Tuint32 | yes |  | Enum specifying usage of this default |
| CDMA_IWF_NAME | Text | no |  | IWF name |
| CDMA_SERVICE_OPTIONS | Tuint32 | no |  | Mask indicating valid service options |
| CDMA_PDP_TYPE | TUint32 | no |  | PDP type. (IPv4 or IPv6) |
| CDMA_PDP_ADDRESS | Text | yes |  | PDP address of phone |
| CDMA_REQ_FWD_PRIORITY | TUint32 | yes |  | Requested forward priority |
| CDMA_REQ_REV_PRIORITY | TUint32 | yes |  | Requested reverse priority |
| CDMA_REQ_FWD_BITRATE | TUint32 | yes |  | Requested forward bitrate |
| CDMA_REQ_REV_BITRATE | TUint32 | yes |  | Requested reverse bitrate |
| CDMA_REQ_FWD_LOSS | TUint32 | yes |  | Requested forward frame loss rate |
| CDMA_REQ_REV_LOSS | TUint32 | yes |  | Requested reverse frame loss rate |
| CDMA_REQ_FWD_MAXDELAY | TUint32 | yes |  | Requested forward delay |
| CDMA_REQ_REV_MAXDELAY | TUint32 | yes |  | Requested reverse delay |
| CDMA_MIN_FWD_BITRATE | TUint32 | yes |  | Minimum acceptable forward bitrate |
| CDMA_MIN_REV_BITRATE | TUint32 | yes |  | Minimum acceptable reverse bitrate |
| CDMA_ACCPT_FWD_LOSS | TUint32 | yes |  | Maximum acceptable forward frame loss rate |
| CDMA_ACCPT_REV_LOSS | TUint32 | yes |  | Maximum acceptable forward frame loss rate |
| CDMA_ACCPT_FWD_MAXDELAY | TUint32 | yes |  | Maximum acceptable forward delay |
| CDMA_ACCPT_REV_MAXDELAY | TUint32 | yes |  | Maximum acceptable reverse delay |
| CDMA_DATA_COMPRESSION | TBool | yes |  | Data compression |
| CDMA_HEADER_COMPRESSION | TBool | yes |  | IP header compression |
| CDMA_ANONYMOUS_ACCESS | TBool | yes |  | Use anonymous access |
| CDMA_OP_MODE | TUint32 | yes | ECommDbCdmaOpSimpleIp(0) SimpleIP only ECommDbCdmaOpFallback(1) MobileIP with SimpleIP fallback ECommDbCdmaOpMobileIp (2) MobileIP only | Defines the active operation mode in the mobile station. Possible values are presented in table 2-11. |
| CDMA_MIP_MAX_NUM_RETRY | TUint32 | yes |  | Maximum number of retries for MobileIP registration |
| CDMA_MIP_FIRST_RETRY_TIMEOUT | TUint32 | yes |  | Amount of time between the first and second MobileIP registration requests, while the Mobile Station did not receive a Registration Reply (units of 250ms). |
| CDMA_MIP_REREG_THRESHOLD | TUint32 | Yes |  | Re-registration threshold (time in minutes before the expiration of the registration lifetime) |
| CDMA_SERVICE_ENABLE_LLMNR | TBool | Yes |  |  |

## Dial In ISP Service Table

| Defined Constant | Type | Null? | Possible values | Description |
| --- | --- | --- | --- | --- |
| ISP_DESCRIPTION | Text | yes |  | Application’s description of ISP |
| ISP_TYPE | TUint32 | yes |  | Enum value indicating ISP type, e.g. Internet ISP, WAP ISP |
| ISP_USE_LOGIN_SCRIPT | TBool | yes |  | Use login script? |
| ISP_LOGIN_SCRIPT | Long Text | yes |  | Login script (used if Use Login Script is true) |
| ISP_AUTHENTICATION | TUint32 | yes |  | UNUSED (will eventually point to some authentication table/group) |
| ISP_IF_AUTH_RETRIES | TUint32 | yes |  | Number of times to retry authentication if it fails |
| ISP_IF_ PARAMS | Text | yes |  | Interface parameter string |
| ISP_IF_NETWORKS | Text | yes |  | Comma separated list of networkprotocols, e.g. "PPP" |
| ISP_IP_ADDR_FROM_SERVER | TBool | no |  | Get IP address from server? |
| ISP_IP_ADDR | Text | yes |  | IP address of interface |
| ISP_IP_NET MASK | Text | yes |  | IP net mask of interface |
| ISP_IP_GATEWAY | Text | yes |  | IP address of gateway |
| ISP_IP_DNS_ADDR_FROM_SERVER | TBool | no |  | Get DNS addresses from server? |
| ISP_IP_NAME_SERVER1 | Text | yes |  | IP Address of primary name server |
| ISP_IP_NAME_SERVER2 | Text | yes |  | IP Address of secondary name server |
| ISP_IP6_DNS_ADDR_FROM_SERVER | TBool | yes |  | Get DNS addresses from server? |
| ISP_IP6_NAME_SERVER1 | Text | yes |  | IP Address of primary name server |
| ISP_IP6_NAME_SERVER2 | Text | yes |  | IP Address of secondary name server |
| ISP_ENABLE_IP_HEADER_COMP | TBool | yes |  | Enable IP header compression? |
| ISP_ENABLE_LCP_EXTENSIONS | TBool | yes |  | Enable LCP extensions? |
| ISP_DISABLE_PLAIN_TEXT_AUTH | TBool | yes |  | Disable plain text authentication? |
| ISP_ENABLE_SW_COMP | TBool | yes |  | Enable software compression (e.g. Microsoft, STAC or Predictor compression)? |
| ISP_BEARER_NAME | TUint32 | yes |  | Enum value specifying the name of the bearer, e.g. Asynchronous Modem |
| ISP_BEARER_SPEED | TUint32 | yes |  | Enum value specifying the bearer speed, e.g. |
| ISP_BEARER_CE | TUint32 | yes |  | Enum value specifying the bearer CE, e.g. non-transparent |
| ISP_INIT_STRING | 8-bit Text | Yes |  | Modem initialisation string to be used when this ISP is in use. This string is copied into the MODEM_ISP_INIT_STRING field by NetDial. |
| ISP_USE_EDGE | TBool | Yes |  | EDGE parameter used by GSM |

## Connection Preference Table

| Defined Constant | Type | Null? | Possible values | Description |
| --- | --- | --- | --- | --- |
| CONNECT_PREF_RANKING | TUint32 | no |  | Ranking of this connection preference |
| CONNECTION_PREF_DIRECTION | TUint32 | no |  | Enum indicating the direction of the connection. |
| CONNECT_PREF_BEARER_SET | TUint32 | yes |  | Bitwise ORed value representing the bearers which can be used for this connection |
| CONNECT_PREF_DIALOG_PREF | TUint32 | no |  | Enum indicating whether or not the user should be prompted or warned on connection. |
| CONNECT_PREF_IAP | TUint32 | yes |  | Identifier of preferred IAP to use |

## Network Table

The Network table simply holds network identifiers. This enables connections based upon network rather than connection preference possible. It a client queries the user for their preferred network then it is possible to create a view on the IAP table with the specified network and thus present the user with a  list of IAPs to choose from all of which will be on the chosen network.

The network table only contains the default fields found in all records. This is because at present the network table’s sole purpose is as a reference point identifying possible existing networks.

| Defined Constant | Type | Null? | Possible values | Description |
| --- | --- | --- | --- | --- |
| HOST_NAME | Text | no |  | Name of the host |

## Modem Bearer Table

| Defined Constant | Type | Null? | Possible values | Description |
| --- | --- | --- | --- | --- |
| MODEM_AGENT | Tuint32 | No |  | Identifier of the Agent to load for this modem |
| MODEM_NIF_NAME | Text | no |  | Name of the Network Interface to load for this modem |
| MODEM_PORT_NAME | Text | yes |  | Comm port name |
| MODEM_TSY_NAME | Text | yes |  | TSY name |
| MODEM_CSY_NAME | Text | yes |  | CSY name |
| LAST_SOCKET_ACTIVITY_TIMEOUT | TUint32 | yes |  | Time (in seconds) to stay online when all socket activity has ceased |
| LAST_SESSION_CLOSED_TIMEOUT | TUint32 | yes |  | Time (in seconds) to stay online when session has closed |
| LAST_SOCKET_CLOSED_TIMEOUT | TUint32 | yes |  | Time (in seconds) to stay online when socket has closed |
| MODEM_DATA_BITS | TUint8 | yes |  | Data bits |
| MODEM_STOP_BITS | TUint8 | yes |  | Stop bits |
| MODEM_PARITY | TUint8 | yes |  | Parity |
| MODEM_RATE | TUint32 | yes |  | Baud rate |
| MODEM_HANDSHAKING | TUint32 | yes |  | Handshaking |
| MODEM_SPECIAL_RATE | TUint32 | yes |  | Special rate |
| MODEM_XON_CHAR | TUint8 | yes |  | Xon character |
| MODEM_XOFF_CHAR | TUint8 | yes |  | Xoff Character |
| MODEM_FAX_CLASS_PREF | TUint8 | yes |  | Preferred fax class |
| MODEM_SPEAKER_PREF | TUint8 | yes | Field TypeId Value | Preferred speaker mode |
|  |  |  | EModemSpeakerSettingNever 0 | Speaker always off. |
|  |  |  | EModemSpeakerSettingUntilCallAnswered 1 | Speaker on until the call has been answered |
|  |  |  | EModemSpeakerSettingAlways 2 | Speaker always on. |
|  |  |  | EModemSpeakerSettingAfterDialUntilAnswer 3 | Speaker on after dialing until answer |
| MODEM_SPEAKER_VOL_PREF | TUint8 | yes | Field TypeId Value | Preferred speaker volume |
|  |  |  | EModemSpeakerVolumeQuiet, 0 | Speaker off. |
|  |  |  | EModemSpeakerVolumeMedium 1 | Speaker volume set to medium |
|  |  |  | EModemSpeakerVolumeLoud 2 | Speaker volume set to loud |
| MODEM_MODEM_INIT_STRING | 8-Bit Text | yes |  | General Modem initialisation string |
| MODEM_DATA_INIT_STRING | 8-Bit Text | yes |  | Data Initialisation string |
| MODEM_FAX_INIT_STRING | 8-Bit Text | yes |  | Fax Initialisation string |
| MODEM_ISP_INIT_STRING | 8-Bit Text | yes |  | ISP Initialisation string. This field is written by NetDial from the ISP_INIT_STRING. |
| MODEM_DIAL_PAUSE_LENGTH | 8-Bit Text | yes |  | Command to modify the pause created during dialling using the comma character (‘,’). |
| MODEM_CARRIER_TIMEOUT | 8-Bit Text | yes |  | Command to set the time out the modem uses when establishing a link before giving up and returning to command mode |
| MODEM_AUTO_ANSWER_RING_COUNT | 8-Bit Text | yes |  | Command to set the number of rings before modem auto answers. |
| MODEM_SPEAKER_VOL_CONTROL_LOW | 8-Bit Text | yes |  | Command to set the modem speaker volume to low. |
| MODEM_SPEAKER_VOL_CONTROL_MEDIUM | 8-Bit Text | yes |  | Command to set the modem speaker volume to medium. |
| MODEM_SPEAKER_VOL_CONTROL_HIGH | 8-Bit Text | yes |  | Command to set the modem speaker volume to high. |
| MODEM_SPEAKER_ALWAYS_OFF | 8-Bit Text | yes |  | Command to set the modem speaker off. |
| MODEM_SPEAKER_ON_UNTIL_CARRIER | 8-Bit Text | yes |  | Command to set the modem speaker on until the carrier. |
| MODEM_SPEAKER_ALWAYS_ON | 8-Bit Text | yes |  | Command to set the modem speaker on. |
| MODEM_SPEAKER_ON_AFTER_DIAL_UNTIL_CARRIER | 8-Bit Text | yes |  | Command to set the modem speaker on except during dialling. |
| MODEM_DIAL_TONE_WAIT_MODIFIER | 8-Bit Text | yes |  | The dial command modifier to wait for Dial Tone. |
| MODEM_CALL_PROGRESS_1 | 8-Bit Text | yes |  | Disable busy and dial tone detection. |
| MODEM_CALL_PROGRESS_2 | 8-Bit Text | yes |  | Dial tone detection enabled, Busy detection disabled. |
| MODEM_CALL_PROGRESS_3 | 8-Bit Text | yes |  | Dial tone detection disabled, Busy detection enabled. |
| MODEM_CALL_PROGRESS_4 | 8-Bit Text | yes |  | Dial tone and busy detection enabled |
| MODEM_ECHO_OFF | 8-Bit Text | yes |  | Switch echo mode off |
| MODEM_VERBOSE_TEXT | 8-Bit Text | yes |  | Switch verbose mode on |
| MODEM_QUIET_OFF | 8-Bit Text | yes |  | Switch quiet mode off |
| MODEM_QUIET_ON | 8-Bit Text | yes |  | Switch quiet mode on |
| MODEM_DIAL_COMMAND_STATE_MODIFIER | 8-Bit Text | yes |  | Dial command modifier to return to command mode after dialling. |
| MODEM_ON_LINE | 8-Bit Text | yes |  | Enter on-line mode from on-line command mode. |
| MODEM_RESET_CONFIGURATION | 8-Bit Text | yes |  | Reset the modem configuration. |
| MODEM_RETURN_TO_FACTORY_DEFS | 8-Bit Text | yes |  | Return the modem configuration to its factory defaults. |
| MODEM_DCD_ON_DURING_LINK | 8-Bit Text | yes |  | Command the modem to only assert DCD when a carrier is actually detected, i.e. while the link is up. |
| MODEM_DTR_HANG_UP | 8-Bit Text | yes |  | Command the modem to hang up the current call when the DTE drops the DTR line. |
| MODEM_DSR_ALWAYS_ON | 8-Bit Text | yes |  | Command the modem to always assert DSR. |
| MODEM_RTS_CTS_HANDSHAKE | 8-Bit Text | yes |  | Command the modem to use RTS/CTS flow control. |
| MODEM_XON_XOFF_HANDSHAKE | 8-Bit Text | yes |  | Command the modem to use software flow control. |
| MODEM_ESCAPE_CHARACTER | 8-Bit Text | yes |  | The character used by the DTE to return to command mode from on-line mode. |
| MODEM_ESCAPE_GUARD_PERIOD | 8-Bit Text | yes |  | Command the modem to use a particular escape sequence guard period. |
| MODEM_FAX_CLASS_INTERROGATE | 8-Bit Text | yes |  | Ask the modem which Fax modes are supported |
| MODEM_FAX_CLASS | 8-Bit Text | yes |  | Set the Fax mode |
| MODEM_NO_DIAL_TONE | 8-Bit Text | yes |  | No dial tone detected |
| MODEM_BUSY | 8-Bit Text | yes |  | Busy tone detected |
| MODEM_NO_ANSWER | 8-Bit Text | yes |  | No answer detected |
| MODEM_CARRIER | 8-Bit Text | yes |  | Carrier report message |
| MODEM_CONNECT | 8-Bit Text | yes |  | Connection report message |
| MODEM_COMPRESSION_CLASS_5 | 8-Bit Text | yes |  | Compression Class 5 report message |
| MODEM_COMPRESSION_V42BIS | 8-Bit Text | yes |  | Compression V.42 bis report message |
| MODEM_COMPRESSION_NONE | 8-Bit Text | yes |  | No compression report message |
| MODEM_PROTOCOL_LAPD | 8-Bit Text | yes |  | LAPD protocol report message |
| MODEM_PROTOCOL_ALT | 8-Bit Text | yes |  | ALT protocol report message |
| MODEM_PROTOCOL_ALTCELLULAR | 8-Bit Text | yes |  | ALT-CELLULAR report message |
| MODEM_PROTOCOL_NONE | 8-Bit Text | yes |  | No protocol report message |
| MODEM_MESSAGE_CENTRE_NUMBER | Text | yes |  | Phone number of the SMS centre |
| MODEM_MESSAGE_VALIDITY_PERIOD | TUint32 | yes |  | Valdity period of an SMS (in minutes) |
| MODEM_MESSAGE_DELIVERY_REPORT | TBool | Yes |  | Make a delivery report on SMS? |
| MODEM_MIN_SIGNAL_LEVEL | TUint32 | Yes |  | Minimum signal level required for a connection to succeed. The connection will fail if below this level. Setting the column to NULL will turn off this functionality. The value must be in dBm with an offset of 10000 added to it. i.e. Value=dBm+10000 |
| MODEM_COMM_ROLE | TUint32 | Yes | Field TypeId Value | Modem role: as DTE or DCE |
|  |  |  | KModemCommRoleDCE 0 | Open comm port as DTE |
|  |  |  | KModemCommRoleDCE 1 | Open comm. port as DCE |
| MODEM_CONTROL_CHANNEL_PORT_NAME | TUint32 | Yes |  |  |
| MODEM_SIR_SETTINGS | TUint32 | yes |  |  |

## LAN Bearer Table

| Defined Constant | Type | Null? | Possible values | Description |
| --- | --- | --- | --- | --- |
| LAN_BEARER_AGENT | Tuint32 | no |  | Identifier of the Agent to load for this modem |
| LAN_BEARER_NIF_NAME | Text | no |  | Name of the Network Interface to load for this modem |
| LAN_BEARER_LDD_FILENAME | Text | yes |  |  |
| LAN_BEARER_LDD_NAME | Text | no |  | Name of the logical driver to load for this LAN |
| LAN_BEARER_PDD_FILENAME | Text | yes |  |  |
| LAN_BEARER_PDD_NAME | Text | no |  | Name of the Physical driver to load for this LAN |
| LAN_BEARER_PACKET_DRIVER_NAME | Text | yes |  |  |
| LAST_SOCKET_ACTIVITY_TIMEOUT | TUint32 | yes |  | Time (in seconds) to stay online when all socket activity has ceased |
| LAST_SESSION_CLOSED_TIMEOUT | TUint32 | yes |  | Time (in seconds) to stay online when session has closed |
| LAST_SOCKET_CLOSED_TIMEOUT | TUint32 | yes |  | Time (in seconds) to stay online when socket has closed |

## Location Table

| Defined Constant | Type | Null? | Possible values | Description |
| --- | --- | --- | --- | --- |
| LOCATION_INTL_PREFIX_CODE | Text | yes |  | International prefix code |
| LOCATION_NAT_PREFIX_CODE | Text | yes |  | National prefix code |
| LOCATION_NAT_CODE | Text | yes |  | National code |
| LOCATION_AREA_CODE | Text | yes |  | Area code |
| LOCATION_DIAL_OUT_CODE | Text | yes |  | Number to dial for an outside line |
| LOCATION_DISABLE_CALL_WAITING_CODE | Text | yes |  | Code to dial to disable call waiting facility |
| LOCATION_MOBILE | TBool | no |  | Mobile phone? |
| LOCATION_USE_PULSE_DIAL | TBool | no |  | Use pulse dialling? |
| LOCATION_WAIT_FOR_DIAL_TONE | TBool | no |  | Wait for the dial tone? |
| LOCATION_PAUSE_AFTER_DIAL_OUT | TUint32 | no |  | Pause time after dial out |

## Chargecard Table

| Defined Constant | Type | Null? | Possible values | Description |
| --- | --- | --- | --- | --- |
| CHARGECARD_ACCOUNT_NUMBER | Text | yes |  | Account number |
| CHARGECARD_PIN | Text | yes |  | PIN number |
| CHARGECARD_LOCAL_RULE | Text | yes |  | Order of dialling account number, PIN and .phone number for local calls |
| CHARGECARD_NAT_RULE | Text | yes |  | Order of dialling account number, PIN and phone number for national calls |
| CHARGECARD_INTL_RULE | Text | yes |  | Order of dialling account number, PIN and phone number for international calls |

## Proxies Table

This table has no COMMDB_NAME field.

| Defined Constant | Type | Null? | Possible values | Description |
| --- | --- | --- | --- | --- |
| PROXY_ISP | TUint32 | no |  | Identifier of the associated ISP (or GPRS) record |
| PROXY_SERVICE_TYPE | Text | yes |  | Name of the table from which the ISP (or GPRS) ID is taken |
| PROXY_USE_PROXY_SERVER | TBool | no |  | Use proxy server? |
| PROXY_SERVER_NAME | Long Text | yes |  | Name of the host |
| PROXY_PROTOCOL_NAME | Text | no |  | Name of the protocol for which this proxy can be used |
| PROXY_PORT_NUMBER | TUint32 | yes |  | Port number |
| PROXY_EXCEPTIONS | Long Text | yes |  | “;” separated list of the addresses for which the proxy server should not be used. |

## WAP Access Point Table

| Defined Constant | Type | Null? | Possible values | Description |
| --- | --- | --- | --- | --- |
| WAP_CURRENT_BEARER | Text | no |  | Text string indicating the name of the table from which to read the bearer information. |
| WAP_START_PAGE | Text | yes |  | WAP start page URL |

## WAP IP Bearer Table

This table has no COMMDB_NAME field.

| Defined Constant | Type | Null? | Possible values | Description |
| --- | --- | --- | --- | --- |
| WAP_ACCESS_POINT_ID | TUint32 | no |  | ID of the the WAP Access Point record to which this bearer information refers. |
| WAP_GATEWAY_ADDRESS | Text | yes |  | WAP Gateway address, in this case an IP address |
| WAP_IAP | TUint32 | yes |  | Identifier of a record in the IAP table to be used |
| WAP_PROXY_PORT | TUint32 | no |  | Proxy port number. Required for WAP2.0 only. |
| WAP_PROXY_LOGIN_NAME | Text | yes |  | Proxy login name. Required for WAP2.0 only. |
| WAP_PROXY_LOGIN_PASS | Text | yes |  | Proxy login password. Required for WAP2.0 only. |

## WAP SMS Bearer Table

This table has no COMMDB_NAME field.

| Defined Constant | Type | Null? | Possible values | Description |
| --- | --- | --- | --- | --- |
| WAP_ACCESS_POINT_ID | TUint32 | no |  | ID of the the WAP Access Point record to which this bearer information refers. |
| WAP_GATEWAY_ADDRESS | Text | yes |  | WAP Gateway address, in this case an IP address |
| WAP_SERVICE_CENTRE_ADDRESS | Text | yes |  | SMS Centre Address |
| WAP_WSP_OPTION | TUint8 | yes |  | Enum value indicating whether connection-oriented or connectionless API should be used |
| WAP_SECURITY | TBool | yes |  | Attempt secure WTLS connection to the gateway |

## Access Point Table (CCDAccessPointRecord)

| Defined Constant | Type | Null? | Possible values | Description |
| --- | --- | --- | --- | --- |
| NAME | Text | no |  | Access Point’s name |
| ACCESS_POINT_POLICY | TUint32 | yes |  | Link to a selection policy |
| ACCESS_POINT_GID | TUint32 | No |  | Id of a group (usually a layer) this Access Point belongs to. |

## IAP Priority Selection Policy Table (CCDIAPPrioritySelectionPolicyRecord)

| Defined Constant | Type | Null? | Possible values | Description |
| --- | --- | --- | --- | --- |
| NAME | Text | no |  | Policy’s name |
| SELECTION_POLICY_IAP1 | TUint32 | yes |  | Link to an IAP of priority 1 |
| SELECTION_POLICY_IAP2 | TUint32 | yes |  | Link to an IAP of priority 2 |
| SELECTION_POLICY_IAP3 | TUint32 | yes |  | Link to an IAP of priority 3 |
| SELECTION_POLICY_IAP4 | TUint32 | yes |  | Link to an IAP of priority 4 |
| SELECTION_POLICY_IAP5 | TUint32 | yes |  | Link to an IAP of priority 5 |
| SELECTION_POLICY_IAP6 | TUint32 | yes |  | Link to an IAP of priority 6 |
| SELECTION_POLICY_IAP7 | TUint32 | yes |  | Link to an IAP of priority 7 |
| SELECTION_POLICY_IAP8 | TUint32 | yes |  | Link to an IAP of priority 8 |
| SELECTION_POLICY_IAP9 | TUint32 | yes |  | Link to an IAP of priority 9 |
| SELECTION_POLICY_IAP10 | TUint32 | yes |  | Link to an IAP of priority 10 |
| SELECTION_POLICY_IAP11 | TUint32 | yes |  | Link to an IAP of priority 11 |
| SELECTION_POLICY_IAP12 | TUint32 | yes |  | Link to an IAP of priority 12 |
| SELECTION_POLICY_IAP13 | TUint32 | yes |  | Link to an IAP of priority 13 |
| SELECTION_POLICY_IAP14 | TUint32 | yes |  | Link to an IAP of priority 14 |
| SELECTION_POLICY_IAP15 | TUint32 | yes |  | Link to an IAP of priority 15 |

# Further Information

The following section should always be the last section of the document as it is essentially information about the document rather than its content.

## People

| Role | Person / People |
| --- | --- |
| Reviewers | List of people the document was sent out to for review |
| Contributors | List of people who contributed to the production of or reviewed the document |
| Distribution | List of people the document is to be distributed to or database/location where the document is to be stored. |

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [R1] |  |  |  |
| [R2] |  |  |  |

## Open Issues

## Glossary

| Term | Definition |
| --- | --- |

## Document History

| Date | Version | Status | Author | Description |
| --- | --- | --- | --- | --- |
| 04-08-2005 | 0.2 | Draft | <Name> | Added Access Point table by MZ |
| dd-mm-yyyy | 0.1 | Draft | <Name> | First draft based on SGL.PPS001.500 Lightweight Document Template Rev 8.5 |