Contents

1	Introduction	3

2	Overview	3

3	Configuring Tables in the Comms Database for a PSD Connection	3

3.1	Network	3

3.2	Location	3

3.3	ModemBearer	3

3.4	OutGoingGPRS	3

3.5	Chargecard	3

3.6	IAP	3

3.7	GlobalSettings	3

3.8	ConnectionPreferences	4

4	Sample configuration file	4

[GlobalSettings]	4

# COMMDB_ID = 1	4

Name=GlobalSettingsRecord	4

[ConnectionPreferences]	5

# COMMDB_ID = 1	5

Name=First Choice	5

[Network]	5

# COMMDB_ID = 1	5

Name=Office Intranet	5

[Location]	5

# COMMDB_ID = 1	5

Name=Office Direct Dial	5

[ModemBearer]	6

# COMMDB_ID = 1	6

Name=Dacom Surfer	6

[Chargecard]	7

# COMMDB_ID = 1	7

Name=Dummy Chargecard Record	7

[OutgoingGPRS]	7

# COMMDB_ID = 1	7

[IAP]	8

# COMMDB_ID = 1	8

Name=PSD: Outgoing GPRS connection	8

5	Further Information	9

5.1	References	9

5.2	Glossary	9

5.3	Document History	9

# Introduction

This document explains one particular use case for the Comms database: configuring an outbound GPRS connection, i.e., configuring a packet-switched data (PSD) connection.

The Comms database stores configuration data that supports network connections. See How to Configure the Comms Database to Support Network Connections [R1] for an overview and for other references.

# Overview

This document explains how to configure an outbound connection over a GPRS network.

You must configure at least one record in each of the tables described below. You may need to move between tables, but we suggest configuring the tables in the order below

# Configuring Tables in the Comms Database for a PSD Connection

## Network

This table contains the configuration settings for each available network. Each record identifies one available network used for multihoming (maintaining multiple connections, sometimes over multiple technologies).  You must have one record in this table and it is important to note that two network records will be used to setup a VPN connection. One record will refer to VPN and the other one will address to real network.

## Location

This table holds the details of device's location information. It is used to configure a record to support outgoing connections from the user’s country, area code, etc.

## ModemBearer

This table configures a record to support outgoing modem-based connections over GPRS network. The Modem Bearer table contains information about modems and physical connections to networks.

## OutGoingGPRS

This table is used to configure a record with the settings appropriate to your GPRS network for outbound connection.

## Chargecard

If necessary, configure a record with the data for the credit card or charge card that will pay for the GPRS service

## IAP

The IAP table defines a number of IAPs and configure a record for this network connection. This IAP record must refer to the above records in Network, Location, ModemBearer, and OutGoingGPRS.

## GlobalSettings

GlobalSettings can have only one record. Make sure that:

- ModemForDataAndFax contains the record id of the appropriate record in the ModemBearer table.

- LocationforDataandFax contains the record id of the appropriate record in the Location table.

- DefaultNetwork refers contains the record id of the appropriate record in the Network table.

## ConnectionPreferences

The Comms database has multiple IAP’s representing possible network connection choices. This table allows the device to connect to another network if the attempt to connect to the first network fails. . The ConnectionPreferences table must contain at least one record.  No more than two records can have a Ranking of 1 or 2. Other records can have a ranking of zero. The highest priority connection should be given the ranking 1 and the next priority network connection should be given ranking 2

Make sure that at least one of the ranked records has the following settings:

- Direction=OUTGOING

- BearerSet=PSD

- DialogPref=DONOTPROMPT (or another valid value)

- IAP contains the record id of the appropriate record in the IAP table.

# Sample configuration file

Below is a configuration file that will generate a Comms database that will support PSD (GPRS) connections. Fields values that refer to other tables or other records are hyperlinked.

Notes:

- For all tables not configured below, CED will generate tables with no records.

- Not all the fields below are relevant for a PSD connection. Some may refer to records that are not defined in this sample configuration file.

# [GlobalSettings]

ADD_SECTION

## # COMMDB_ID = 1

## Name=GlobalSettingsRecord

# ConnectionAttempts must be set to one or two

ConnectionAttempts=2

RedialAttempts=3

# This value for SMSBearer is regarded as sensible for packet switched #connections, whereby data is only sent circuit switched if packet

#switched is not available. 1 denotes ESMSBearerPacketPreferred for this #field

SmsBearer= 1

SmsReceiveMode=2

# The GPRS fields configure packet switched connections.

GPRSAttachMode=1

# AcceptIncomingGprs is set to ETRUE

AcceptIncomingGprs=1

GPRSClassCBearer=GSM

# General settings, will differ when the record IDs are different

ModemForDataAndFax=1

ModemForPhoneServicesAndSMS=1

LocationForDataAndFax=1

LocationForPhoneServicesAndSMS=1

# Default connection to the network specified in record one from the

#Network table

DefaultNetwork=1

MaxMBufHeap=2

BearerAvailabilityCheckTSY=mm

FIELD_COUNT=15

END_ADD

# [ConnectionPreferences]

ADD_SECTION

## # COMMDB_ID = 1

## Name=First Choice

# Ranking setting must be set to one if connection to this IAP is to be # made default.

Ranking=1

Direction=OUTGOING

# BearerSet must be set to PSD; We need a bearer capable of supporting #packet switched connections

BearerSet=PSD

DialogPref=DONOTPROMPT

# IAP record one is set as it offers GPRS services

IAP=1

FIELD_COUNT=6

END_ADD

# [Network]

ADD_SECTION

## # COMMDB_ID = 1

## Name=Office Intranet

FIELD_COUNT=1

END_ADD

# [Location]

ADD_SECTION

## # COMMDB_ID = 1

# These are general settings, which may be used for both circuit and #packet switched connections and only differ if obviously the location #is different.

## Name=Office Direct Dial

IntlPrefixCode=00

NatPrefixCode=0

NatCode=44

AreaCode=207

Mobile=FALSE

UsePulseDial=FALSE

WaitForDialTone=FALSE

PauseAfterDialout=0

FIELD_COUNT=9

END_ADD

# [ModemBearer]

ADD_SECTION

## # COMMDB_ID = 1

# 	These are general settings which can safely set for both circuit and    #	packet switched connections, apart from field ‘Agent’ which will differ #	depending on whether a CSD or a PSD agent is in use.

#	Most fields are Hayes commands and can only be set to one value for that #     particular field

#	TSY and CSY and IfName identify libraries to be used for this connection

#	In the table, set PortName to a suitable COM port. For COM 1, set PortName to COMM::0. For COM 2 use COMM::1, for COM 3 use COMM::2 etc.

## Name=Dacom Surfer

IfName=PPP    #The interface name that is used to implement a network #layer protocol such as PPP

PortName=COMM::0 # The Comm port name used by the modem

TSYName=DUMMY

CSYName=ECUART

LastSocketActivityTimeout=6

LastSessionClosedTimeout=180

LastSocketClosedTimeout=10

DataBits=8   #Number of data bits used for serial port configuration

StopBits=1

Parity=NONE

Rate=57600	#The baud rate of the modem.

Handshaking=244

SpecialRate=0

XonChar=0

XoffChar=0

FaxClassPref=AUTO	#Preferred fax modem class supported by the fax client

SpeakerPref=AFTERDIALUNTILANSWER

SpeakerVolPref=QUIET

ModemInitString=AT&F

DataInitString=AT

FaxInitString=AT&d2

DialPauseLength=S8=   #Initialsation string specific to a particular ISP

SpeakerVolContorlLow=L0

SpeakerVolControlMedium=L1

SpeakerVolControlHigh=L2

SpeakerAlwaysOff=M0

SpeakerOnUntilCarrier=M1

SpeakerAlwaysOn=M2

SpeakerOnAfterUntilCarrier=M3

DialToneWaitModifier=W

CallProgress1=X1	#Disable busy and dial tone detection

CallProgress2=X2	#Dial tone detection is enabled and busy detection is #disabled

CallProgress3=X3	#Dial tone detection is disabled and busy detection is #enabled

CallProgress4=X4	#Both dial tone and busy detection are enabled

EchoOff=E0

VerboseText=V1

QuietOff=Q0

QuietOn=Q1

DialCommandStateModifier=;	#Dial command modifier- used to return to #command mode after dialling.

OnLine=O

ResetConfiguration=Z

ReturnToFactoryDefs=&F

DCDOnDuringLink=&C1

DTRHangUp=&D2

DSRAlwaysOn=&S0

RTSCTSHandshake=&K3

XonXoffHandshake=&K4	#Command the modem to use software flow control

EscapeCharacter=+

EscapeGuardPeriod=S12	#Command the modem to use a particular escape #sequence guard period.

NoDialTone=NO DIAL TONE

Busy=BUSY

NoAnswer=NO ANSWER

Carrier=CARRIER

Connect=CONNECT

CompressionClass5=COMPRESSION:CLASS 5

CompressionV42bis=COMPRESSION :V.42 bis

CompressionNone=COMPRESSION :NONE

ProtocolLAPD=PROTOCOL:LAPD

ProtocolALT=PROTOCOL:ALT

ProtocolALTCELLULAR=PROTOCOL:ALT-CELLULAR

ProtocolNone=PROTOCOL:NONE

MessageValidityPeriod=0	#Validity period for SMS in minutes

MessageDeliveryReport=FALSE

Agent=DummyPSD.agt	#The file name of the agent used

FIELD_COUNT=65

END_ADD

# [Chargecard]

ADD_SECTION

## # COMMDB_ID = 1

## Name=Dummy Chargecard Record

# Dummy example of a Chargecard record

AccountNumber=0500800800,,12345678

Pin=****

LocalRule=HG

NatRule=J,K,0FG

IntlRule=HEFG

FIELD_COUNT=6

END_ADD

# [OutgoingGPRS]

ADD_SECTION

## # COMMDB_ID = 1

# This table only needs to be configured for packet switched connections

Name=Test GPRS

APN=Test

PDPType=IPV4

#These 10 QoS fields below have been deprecated, and may be removed in the future releases of the OS

ReqPrecedence=2

ReqDelay=4

ReqReliability=3

ReqPeakThroughput=3

ReqMeanThroughput=31

MinPrecedence=2

MinDelay=4

MinReliability=3

MinPeakThroughput=3

MinMeanThroughput=31

DataCompression=FALSE

HeaderCompression=FALSE

GprsUseEdge=FALSE

AnonymousAccess=FALSE

IfNetworks=ip

IfPromptForAuth=FALSE

IfAuthName=RasUser

IfAuthPass=pass

AuthRetries=1

IpAddrFromServer=TRUE

IpDNSAddrFromServer=TRUE

EnableLCPExtension=FALSE

DisablePlainTextAuth=FALSE

GprsAccessPointType=0

QosWarningTimeout=1000000

FIELD_COUNT=28

END_ADD

# [IAP]

ADD_SECTION

## # COMMDB_ID = 1

#This IAP is chosen as it offers GPRS services, most appropriate when #setting up packet switched connections

## Name=PSD: Outgoing GPRS connection

IAPServiceType=OutgoingGPRS

# Record one is used from the servicing table ‘OutgoingGPRS’

IAPService=1

IAPBearerType=ModemBearer

# Record one is used from the ModemBearer table

IAPBearer=1

# This IAP will connect to the network specified in record one of the Network table

IAPNetwork=1

IAPNetworkWeighting=0

Location=1

FIELD_COUNT=8

END_ADD

Here is a ready made configuration file. After saving onto your local machine, rename the file to “Psd.cfg”, since after you open it, the file name would have been changed to “PKGXX” where X denotes some automated number.

# Further Information

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [R1] | How to Configure the Comms Database to Support Network Connections |  |  |

## Glossary

| Term | Definition |
| --- | --- |
| LAN | Local Area Network |
| PSD | Packet Switched Data |
| IAP | Internet Access Points |
| ISP | Internet Service Provider |
| GPRS | General Packet Radio Service |

## Document History

| Date | Version | Status | Description |
| --- | --- | --- | --- |
| 12.11.06 | 0.1 | Draft | This document was further amended and has been further investigated ensuring accuracy of the documentation. |
| 17.11.06 | 0.2 | Awaiting review | This document was written as a guide to how to configure PSD connections. |
| 21.07.06 | 1.0 | Issued | This document was written as a guide to how to configure PSD connections. |
| 02.07.07 | 1.1 | Issued | This document was written as a guide to how to configure PSD connections. |