Mobile IP to Simple IP Fallback Mechanism Architecture Analysis

| Security Classification: | Internal-Symbian |
| --- | --- |
| Status: | Draft |
| Revision: | 0.3 |
| Last Revised Date: | Feb 26, 2004 |
| Team/Department : | CDMA/Networking |
| Author(s): | Voicu Tulai, Victor Minkovski |
| Owner(s): | CMDA TA |

| Approver(s): | Philippe Gabriel | Distribution: | Philippe Gabriel, Dan Fandrich, Bill Gordon |
| --- | --- | --- | --- |

Table of Contents

1	Introduction	3

2	Analysis	3

3	Options for selecting the Simple IP fallback record in the service view	7

3.1	Using CNifAgentRef	7

3.2	Using NetCon	9

3.3	Using existing Nifman connection fallback mechanism	10

3.4	Combining Mobile IP and Simple IP settings	12

4	Conclusion	13

Table of Figures

Figure 1: Nifman Mobile IP to Simple IP fallback sequence	4

Figure 2: Modified CNifAgentRef state machine	5

Figure 3: Activity times	6

Figure 4: Option 1 activity timing	7

Figure 5: Option 2 activity timing	11

Summary Tables

Table 1: Option 1 summary	8

Table 2: Option 2 summary	9

Table 3: Option 3 summary	11

# Introduction

The Mobile IP to Simple IP fallback mechanism allows MS to retry the connection by using Simple IP when a Mobile IP registration attempt fails.

# Analysis

When a connection is attempted Netcon is used to select an IAP and the current CommDb views are updated with the service and bearer records corresponding to the selected IAP. An agent is loaded based on the bearer record and a NIF is loaded and initialized based on the service record.

In the fallback scenario the Mobile IP connection identified by the current service record fails and a new Simple IP connection has to be initiated. It is assumed that the Simple IP connection is defined in a separate service record.

Actions to implement the fallback:

- Detect the Mobile IP failure

- Stop the NIF

- Select the new Simple IP service record

- Re-start the NIF

These actions are detailed in the sequence diagram below. Description:

- When the first ESOCK sub-session is started CNifSession::Start is called

- CNetworkControllerBase::SelectAgent is called to select an IAP based on the settings in the ConnectionPreferences table

- An IAP is selected and the CommDB views are updated with the corresponding records in the service and bearer tables

- SelectComplete is called to indicate the end of selection and the CNifAgentRef instance is created and started

- Connect is called to initialize the agent

- ConnectComplete indicates that the agent was initialized successfully

- The PPP NIF is started and it signals the completion of the negotiation phase through LinkLayerUp. Note that PPP is aware not to negotiate an IP address by looking at the current service record, which specifies that MobileIP is used

- When PPP completes the Mobile IP daemon is asked to register through ConfigureNetwork

- The daemon fails and an error is reported back with the KLinkLayerOpen progress notification. Note that the error can be either an operational error from the daemon, a timeout error from CNifAgentRef itself or a “server died” error if the daemon is killed while processing the configuration request

- In order to execute the fallback PPP has to be re-initialized and it should re-negotiate all parameters (LCP, NCP, etc) for a Simple IP connection. The most important differences in between the Mobile IP and the Simple IP negotiations is the addition of the IP address negotiation option for IPCP and the authentication option.

- PPP is stopped and when done (LinkLayerDown) the Simple IP service record is selected in CommDb.

- PPP is re-started and this time will negotiate the Simple IP parameters.

- When PPP finishes a CNifNullConfiguration instance is created (as specified by the current service record) and the ConfigureNetwork call is completed right away with no error

Refer to  figures 1 & 2 (below) for the fallback sequence  in Nifman, and a state machine required to execute this sequence.

Refer to figure 3  (below) for the approximate fallback activity times.

Figure 1: Nifman Mobile IP to Simple IP fallback sequence

New CNifAgentRef states

Existing CAgentRef states

Connecting

Stopping

Started

Disconnecting

Configuring

Link Up

Falling back to Simple IP

Start

Agent Connected /Start NIF

Reconnect Complete

Link Layer Down / Disconnect

Link Layer Down / Reconnect

Link Layer Down/

Reconnect

Link Layer Down / Disconnect

Stop

Stop

Link Layer Up /

ConfigureNetwork

Progress(KLinkLayerOpen, Error) / Stop

Success / Start NIF

Link Layer Down / Disconnect

Failure/ Disconnect

Agent Stopped

Progress(KLinkLayerOpen, KErrNone) /

Start Timer

Timer Complete / Stop Timer

Link Layer Down /

Fall Back

Packet Activity

Reconnecting

Figure 2: Modified CNifAgentRef state machine

Figure 3: Activity times

| Activity | Approximate time, seconds |
| --- | --- |
| Bring up the air interface t1 | 3 |
| Establish PPP connection t2 | 2 |
| Perform Mobile IP registration t3 | 2 |
| Detect Mobile IP registration attempt failure t4 | 2 |
| Perform Simple IP negotiation t5 | Very short |
| Tear down the PPP connection | Negligible |
| Tear down the air interface | Negligible |

# Options for selecting the Simple IP fallback record in the service view

## Using CNifAgentRef

The Simple IP fallback record is specified when the Mobile IP record is selected. This can be implemented by adding a field to the service table that would allow a Mobile IP record to be statically linked with the corresponding Simple IP fallback record if the current mode of operation is “fallback” (this is defined using an existing field in DefaultCDMA2000SettingsTable). When the Mobile IP connection fails this field is used to select the new service record without Netcon being involved. This option requires changes to Nifman and CommDb and ensures that the same agent is used for the Simple IP connection. If required, the “link” field can be overridden programmatically so that another Simple IP record is selected at runtime. Refer to figure 2 for required changes to Nifman’s CNifAgentRef.

A drawback of this option is the possibility of Mobile IP negotiation failure due to a failure of the air interface. Under this scenario, Simple IP negotiation is attempted over a non-functioning air interface, resulting in an unnecessary delay.

The fallback timing is illustrated below:

Mobile Station

Base Station

Home Agent

Bring up the air interface

t1

Establish PPP connection

t2

Perform Mobile IP registration

t3

Detect failure of Mobile IP registration attempt

t4

Negotiate Simple IP

t5

Time

Establish PPP connection

t2

Figure 4: Option 1 activity timing

Approximate time to execute the connection and the fallback is:

T =  t1 + t2 + t3 + t4 + t2 + t5

=   3 +  2 +  2 +  2 +  2 +  0

= ~ 11 seconds

See fig.3 for the suggested changes to CNifAgentRef state machine. Note the new states introduced by the fallback mechanism.

Table 1: Option 1 summary

| Factor | Comments |
| --- | --- |
| Functionality | Implemented |
| Development cost | High |
| Schedule | Long |
| Performance: approximate time to execute the fallback | 11 seconds |
| Risk | Introduction of defects due to changes to Nifman and CommdDB |

## Using NetCon

The Simple IP service record is selected using Netcon. New functionality will have to be implemented in Netcon so it knows to filter out the service records that do not have the same agent as the original Mobile IP record and only consider the Simple IP records. This option requires changes to Nifman, Netcon and CommDb.

As in Option 1, Mobile IP negotiation may fail due to a failure of the air interface, resulting in an unnecessary delay.

Approximate time to execute the connection and the fallback is as in Option 1 (See fig. 2):

T =  t1 + t2 + t3 + t4 + t2 + t5

=   3 +  2 +  2 +  2 +  2 +  0

= ~ 11 seconds

Table 2: Option 2 summary

| Factor | Comments |
| --- | --- |
| Functionality | Implemented |
| Development cost | Highest |
| Schedule | Longest |
| Performance: approximate time to execute the fallback | 11 seconds |
| Risk | Introduction of defects due to changes to Nifman , CommdDB and Netcon |

## Using existing Nifman connection fallback mechanism

The Simple IP service record is selected using the existing Nifman connection fallback mechanism and a special configuration of CommDB. 
The Connection Preferences Table in CommDB assigns the IAPs a “Ranking” value. By default, Nifman chooses the IAPs according to this ranking. If a connection attempt using a certain IAP fails, Nifman retries using another IAP with the next highest ranking.
Therefore, the IAP records in Connection Preferences Table can be ranked so that a Simple IP-based IAP is used by Nifman if a Mobile IP-based IAP causes a failure.

No changes to existing Nifman, NetCon, CommDB code are necessary, because the desired fallback behavior is achieved via CommDB configuration only.

A drawback of this approach is that Nifman’s CNifAgentRef is destroyed and the air interface is brought down the after the first connection attempt fails. They have to be reestablished, resulting in performance penalty.  On the other hand, bringing down the air interface avoids the unnecessary delay due to a Simple IP negotiation attempt over a non-functioning interface, if the problems with the air interface caused Mobile IP negotiation failure in the first place.

The only issue with this option is that using the existing CommDB framework with a second preference for a Simple IP-based IAP might not offer a sufficient level of transparency. In a case of a third-party application, which uses the RConnection::ProgressNotification API, a failure notification (On the Mobile IP-based IAP) may be received by the application and presented to the user. Meanwhile, in the background the second IAP preference is being successfully attempted.

As of February 25, 2004 it is believed that failure progress notifications are being queued by the NetCon until the last IAP connection attempt fails. Therefore, the third party applications should not be affected.

Please see next page for the discussion of the fallback time per this option.

Mobile Station

Base Station

Home Agent

Bring up the air interface

t1

Establish PPP connection

t2

Perform Mobile IP registration

t3

Detect failure of Mobile IP registration attempt

t4

Negotiate Simple IP

t5

Time

Bring up the air interface

t1

Establish PPP connection

t2

Figure 5: Option 2 activity timing

Approximate time to execute the connection and the fallback is:

T =  t1 + t12 + t3 + t4 + t1 + t2 + t5

=   3 +  2 +  2 +  2 +  3 +  2  + 0

= ~ 14 seconds

Table 3: Option 3 summary

| Factor | Comments |
| --- | --- |
| Functionality | Implemented |
| Development cost | Lowest |
| Schedule | Shortest |
| Performance: approximate time to execute the fallback | 14 seconds |
| No Risk | No risk of introduction of defects due to changes to Nifman , CommdDB, Netcon |
| Risk | Low risk due to RConnection::ProgressNotification issue |

## Combining Mobile IP and Simple IP settings

Mobile IP and Simple IP settings are combined in a single CommDB service table.
Instead of providing separate CommDB service tables for Mobile IP and Simple IP based connection, a single service table is used, where Simple IP fields are added to the Mobile IP record, with a “Fallback Enabled” field.  This single unified service table is accessed by Nifman in order to execute the fallback to Simple IP.

Under this scenario, some CommDB fields are duplicated.

This option is similar to option 1. It requires changes to Nifman and CommDB and is associated with the same risks as option 1.

# Conclusion

It is optimal to pursue Option 3 – Using existing Nifman connection fallback mechanism, due to the following considerations:

- This option requires the minimal number of changes, resulting in minimal effort, shortest schedule, and lowest risk.

- This option is potentially robust, as only the existing and verified mechanisms are used.

- The performance penalty incurred is not critical: the fallback from Mobile IP to Simple IP takes 3 seconds longer to execute, taking 14 seconds instead of 11 seconds. (27% longer.)

- This option resolves the issue of attempting to negotiate Simple IP over a non-functioning air interface in cases where problems with the air interface caused Mobile IP negotiation failure.

- Assuming that the progress notification issue is resolved, alternative options have no functional advantage.