Ser-Comms v9.5 Subsystem Architectural Description

| Status: | Issued |
| --- | --- |
| Version: | 1.4 |
| Team/Department : | Converged Comms/Core Development |

Contents

1	Introduction	3

1.1	Purpose and Scope	3

2	Subsystem Overview and Background	4

3	Ser-Comms Architecture and APIs	5

3.1	The Subsystem components	5

3.2	Ser-Comms/C32	5

3.2.1	Ser-Comms/C32 Client APIs	7

4	System Architectural Requirements	8

5	Security	9

5.1	Data caging	9

5.2	Backup and restore	9

5.3	Capability Assignments	9

5.3.1	Capability required for using Ser-Comms services	9

5.3.2	Capabilities required for the Ser-Comms subsystem to function	9

5.4	Other Platform Security Changes	10

5.4.1	Deferring capability checking to CSYs	10

5.4.2	Name change to C32 Server	11

5.4.3	Removal of Globally Named Kernel Objects	11

6	Further Information	13

6.1	References	13

6.2	Open Issues	13

6.3	Glossary	13

6.4	Document History	13

# Introduction

## Purpose and Scope

The Ser-Comms v9.5 Architectural Description details the architecture and APIs exposed by the Ser-Comms v9.5 Subsystem.

NOTE: This Architectural Description does not cover the USB component.  For details of this component see [R1].

For a description of the functionality contained within this subsystem please refer to [R4].

# Subsystem Overview and Background

This section provides a general overview of the technology area, including relevant Symbian and external specifications and background information.

The following diagram shows the Ser-Comms components interaction with other Symbian OS Comms components.  For the full Symbian OS Comms Architectural layout see [R2].

ETel.DLL

MM.TSY

C32.DLL

FaxSvr.DLL

PPP.NIF

ECUART.CSY

IrComm.CSY

EComm.LDD

Physical Device Driver

…….Ser-Comms

…….Telephony

…….Networking

…….PAN

…….Base

Legend

IrTranP.DLL

BtComm.CSY

GSMBTT.TSY

Apex.DLL

AtPduMapMM.DLL

ECACM.CSY

EUSBC.LDD

EUSBCC.DLL

UsbMan.DLL

Figure 1 Ser-Comms Component Organisation

# Ser-Comms Architecture and APIs

## The Subsystem components

The Ser-Comms subsystem consists of the following components:

- Ser-Comms\C32

- Ser-Comms\USB*

*As mentioned in the previous section the USB component is not covered in this document, refer to [R1] for more details.

## Ser-Comms/C32

The C32 component itself consists of two sub-components

- Ser-Comms\C32\C32 – implemented as C32.dll

- Ser-Comms\C32\ecuart – implemented as ecuart.csy.

C32 is the Symbian OS Comms server. The component contains the client side API and server side code which is run as a thread in the C32 Comms process.

Manage

C32 Process

Root Server Thread

Root Server Client Library

SymbianOS client/server IPC

C32start.exe

Etel Srv

Phbksync Srv

Esock Srv

c32 Srv

c32 client library API

SymbianOS client/server IPC

Figure 2 C32 Process Layout

The C32 process, also called RootServer process contains its main thread, C32 thread and a number of other threads, namely ESock, ETel and Phonebook Synchroniser. The management of this process is performed by the process’s main thread, the root server thread. From v9.5 onwards, C32 is a multithreaded server.

Rootserver Process

WorkerId 3

C32 Player

C32 Player

Client Process

C32 Dealer CPM

Main C32 thread

(WorkerId 0)

WorkerId 2

ECUARTECACM CSY

C32 Player

WorkerId 1

ECACM

*

Figure 3 Typical c32 server configuration

Figure 3 shows a typical C32 server configuration. C32 contains two distinct roles.

- A Dealer implements an IPC server for an C32 session. The Dealer provides an interface to a number of CSYs to a client. It will receive requests from client sessions and in most cases pass them (in form of RMessage2) to the appropriate Player. Thus the Dealer is a very light-weight entity meant to very quickly and efficiently pass on the IPC requests.

- A Player hosts one or more CSYs by means of .CSY plug-in DLL files. The Player performs the actual work for processing a request from a client and it is responsible for completing the RMessage2 when ready.

The C32 server thread (referred to by its thread name “CommServer”) contains the Dealer (and possibly also a Player if this is configured via its .CMI file -  refer to [R6] ) with the server name “!CommServer” to which client sessions connect.

All other “Worker Threads” can contain a Player. Messages are passed between threads via Comms Channels (the message queue abstraction offered by the CommsFw component), except in the case of a Dealer and a co-resident Player when synchronous function calls are made..

The C32 client API consists of RCommServ, which gives the client a session to the server and RComm, which provides an API to a serial port. RCommServ represents a session to the comms server and it provides a connection to the server for loading and unloading different comms modules (CSYs) and services for finding out the name and numbers of the available ports.

The C32 component also provides the base implementation classes for serial communication modules (CSYs).  It provides two classes CPort and CSerial which serial protocol modules such as ECUART derive from.

A serial communications service is implemented using a polymorphic DLL whose extension is conventionally .CSY. RCommServ is responsible for loading and unloading the CSY modules and for finding out the names and numbers of the available ports. Symbian provides two modules as standard.

- ECUART drives the machines built-in serial port on ARM in RS232 mode.

- IRCOMM drives the infra-red port on ARM in infrared mode.

Other serial services may be used: their clients must ask the communications server to load them.

### Ser-Comms/C32 Client APIs

The following APIs are exposed by the Ser-Comms/C32 architecture:

| API Name | Broad Purpose | Intended Users |
| --- | --- | --- |
| RCommServ | Provides a session with the comms server. | Base, Bluetooth, Infra-red, Telephony and Networking, Comms-infras subsystems |
| RComm | RComm represents a serial port. | Base, Bluetooth, Infra-red, Telephony, Networking, Ser-comms, Java subsystems |

For a detailed description of the C32 APIs see [R3].

#### API Purpose - Further Elaboration

##### RCommServ

This API provides methods for connection to the server and for loading and unloading different comms modules (CSY) as well as services for finding out the name and numbers of the available ports.

##### RComm

This API provides all the necessary functions for communicating via a port, including functions for opening, closing, reading, writing, together with port configuration and capability checking.

#### Key External Relationships and Dependencies

The C32 APIs are used by the following components

| Subsystem | Component |
| --- | --- |
| app-framework | uikon |
| Bluetooth | bluetooth |
| comms-infras | esock, nifman |
| commsui_techview | Bluetoothui, phoneui |
| conn-framework | mrouter |
| Graphics | pdrstore |
| infra-red | irda |
| Messaging | biomsg |
| Networking | csdagt, ftp, ppp |
| ser-comms | usb |
| Syslibs | stdlib |
| Telephony | etel, fax, mmtsy, phbksync |
| tools | debugger_stub, metrotrk |

# System Architectural Requirements

The C32 server provides the link between a transport medium such as USB, serial, IR and protocol data being sent over the medium e.g. USB as a transport for PPP.  [R1] gives a use case demonstrating the interaction between the PPP protocol module, C32 and the USB serial communications module (CSY).

A second example use-case for the C32 component is within a two-box solution, where a TSY communicates with an external modem using AT commands.

The following diagram shows the flow of requests initiating from a client request to perform some telephony operation.  The request will pass through the Telephony server to the TSY which holds a session with the C32 server.  The TSY generates an AT command which is passed to the C32 server, (c32 Dealer) which forwards the request to appropriate c32 Player (hosting the CSY) and sent through the CSY module corresponding to the chosen connection method (in this example the connection is a serial connection and will use ecuart.csy) to the ports logical device driver and physical device driver which will send the request to the external modem.

Telephony Client App

Etel Server

TSY

Etel client library

(etel.dll)

(etel.dll)

(mm.tsy, gprs.tsy, cdma.tsy)

Serial Comms Server

(c32.dll)

Logical Device Driver

(ecomm.ldd)

Physical Device Driver

(euart.pdd)

External Modem

C32 Process

Client Process

Kernel

Etel Srv Thread

c32 Srv Dealer Thread

User side

Kernel side

External

RPhone, RCall, Rline API

RBusLogicalChannel API

Client

Telephony

C32

Base

External

RCommServ, RComm API

c32 Srv Player Thread

Serial Comms Serverecuart.csy

(c32.dll)

Figure 4 Telephony use of C32

# Security

This section relates to the platform security work that has been introduced in a number of phases during release v8.1 and v9.0.

For a detailed description of the Symbian platform security architecture see [R5].

## Data caging

The files identified as requiring protection by data caging are .CMI files. Licensees can have a custom configuration of c32 threads. Symbian provides a default set namely

- c32.cmi

- c32_first.cmi

- c32_second.cmi

- c32_third.cmi

- c32_fourth.cmi

## Backup and restore

This area is not applicable to the Ser-Comms subsystem.

## Capability Assignments

### Capability required for using Ser-Comms services

As described in [R5] securing an executables services requires controlling the loading of library dll’s into processes and controlling the use of its API in a client/server architecture. This is achieved through the use of the capability model.

The range of capabilities required by a client wishing to use the services provided by the Ser-Comms subsystem is completely determined by the port they are attempting to communicate through.  For this reason the required capabilities can be classed as being “deferred to the CSY”. For a complete explanation see section 5.4.1.

For specific details of the security policy used to police an API see the relevant component design documents.

### Capabilities required for the Ser-Comms subsystem to function

Although the capabilities required to use a port are wholly dependent on which port is accessed it is necessary entrust some components with a set of capabilities in order that they may function correctly.  These are outlined below.

#### Library Dlls

The following components are library components in that they provide a client-side interface.

- c32.dll

As it cannot be known all the potential clients (and corresponding capabilities that they possess) which would want to use this library component it is necessary to assign it with the maximum capabilities available to user-side executables, namely All –Tcb capabilities.

#### C32 Comms loaded components

The following components are loaded and run from within the C32 comms process:

- c32.dll*

- Any CSYs (e.g. Ecuart.csy)

To satisfy the second rule of the capability model that a process cannot load a DLL with less capabilities than itself the components that are loaded into the C32 comms process must be entrusted with the capabilities of the C32 comms process in order that they may be successfully loaded.  These components are therefore assigned the following capabilities:

- ReadDeviceData

- WriteDeviceData

- ReadUserData

- WriteUserData

- NetworkControl

- CommDD

- PowerMgmt

- TrustedUI

- ProtServ

- NetworkServices

- LocalServices

*the c32.dll component contains both client and server code in the same binary and because of the requirements described in the previous section this component has already been entrusted with All -Tcb capabilities.

## Other Platform Security Changes

The following sections refer to other architectural changes that have been made to the subsystem in the v9.0 release as a result of platform security.

### Deferring capability checking to CSYs

Since a comm port can be used to gain access to the phone hardware and start a phone call or access a local network, this could potentially incur call charges or malicious access to network.

The architecture of the C32 server is such that at the server level it is not known what serial port is intended to be used until the port is actually accessed.  This is distinguished by what type of CSY has been loaded (e.g. Ecuart.csy for serial port. Ircomm.csy for the IR port, etc.).  The capabilities required for each of the serial ports depends entirely upon the nature of the external hardware that it communicates with. E.g. talking to the baseband should require NetworkServices, while talking to IR or Bluetooth devices, will require LocalServices. Conversely talking to a standard serial port (i.e. a physical serial port) will require no capabilities at all None, as it simply cannot be determined what hardware it is connected to.

When policing the client interface to the C32 server (RCommServ and RComm) it will therefore be necessary to defer the policing of IPC’s related to the opening of a port to the time when the CSY is being accessed.  This is what is being implied in the ‘Deferred to CSY’ item in section 5.3.1.  Therefore any application can access the RcommServ to load/unload CSY modules, enumerate ports etc but to access a specific serial port the client will need to have certain capabilities depending on what CSY the user wishes to access.

To allow the C32 server to retrieve the required capability information from the CSY a new query method is implemented in the CSY interface.

The following diagram describes how it works:

client

C32

CSY

LDD

User

Kernel

1) Client tries to open a Port

2) C32 asks the CSY for the capabilities required to open the port.

3) C32 compares the capabilities returned from the CSY with those of the client.

4) The client is either allowed or disallowed the functionality

Figure 5 Deferring capability checking to CSYs

A new API is added to the CPort interface which is derived from by CSY developers.  This API allows the C32 server to request the capabilities required to open a named port and compare this with the capabilities of the client attempting to open the port.

Ecuart.csy is a particular example of restricting access to certain ports in that it just provides a link to a serial port without knowing what is on the other end of the port.  If Ecuart.csy was being used to obtain a serial link to the phone’s base band (i.e. one-box solution) it would be possible to by-pass Etel and send commands (e.g. AT commands) through the port thus initiating a network connection.

To counter this Ecuart.csy polices the RComm::Open IPC request dependent on what port is being opened.  The Ecuart.csy will return to the C32 server a TSecurityPolicy structure detailing the capabilities a client will require to use the port.  This capability set will vary dependent on whether the port is it an external RS232 port or a port connecting to the base band.  Opening a port to the base band would then require a higher set of capabilities than opening an external port.

This means for the two-box scenario it is trusted to the user of the device to control how the external port is being used. i.e. Whether AT commands are being sent across the serial link.

### Name change to C32 Server

To prevent spoofing of key OS servers a special naming convention is adopted for Platform Security. The name of key servers now begins with ‘!’ and the kernel enforces that only processes with the ProtServ capability may create servers with such a name. C32 is a key OS server and has this change applied to it.

### Removal of Globally Named Kernel Objects

With the introduction of Platform Security Globally Named Kernel Objects (e.g. memory chunks, mutexes and semaphores) are removed because they exploit possibilities for rogue and malicious applications to hijack or block access to protected resources.

The following table summaries the utilisation of globally named kernel objects in the Ser-Comms subsystem and describes the solution for its removal.

| File | Method | Role and type of the object | Removal Solution |
| --- | --- | --- | --- |
| C32\scomm\cs_utl.cpp | ::StartCommServer() | Semaphore used as a global critical section. | Can be safely removed as not utilised. |
| C32\scomm\cs_utl.cpp | CCommSession::CreateThreadL() Executed on the server side when the following global static method is exercised. RCommServ::CreateThreadInCommProc() | Semaphore used as a global critical section. | Replaced by utilising RootServer API. |

# Further Information

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [R1] | Ser-Comms_USB_Subsystem_Architectural_Description.doc | 1.1 | PERFORCE@//EPOC/master/common/generic/ser-comms/documentation/Ser-Comms_USB_Subsystem_Architectural_Description.doc |
| [R2] | V7.0s 3G Comms Architecture.ppt | - | \\londata04\psiosoft\SW Engineering\Core Development\Public\Networking\Architecture\Comms Architecture Diagram |
| [R3] | C32API.doc | 1.0 | PERFORCE@//EPOC/master/common/generic/ser-comms/C32/documentation/C32API.doc |
| [R4] | SerComms_Subsystem_Functional_Specification.doc | 1.0 | PERFORCE@//EPOC/master/common/generic/ser-comms/documentation/SerComms_Subsystem_Functional_Specification.doc |
| [R5] | Platform Security Engineering Guide | 2.2 | http://smglinx.intra/twiki/pub/SMG/PlatSecTechnical/SGL.SM0007.001.Rev2.2.PlatformSecurityEngineeringGuide.doc |
| R6 | C32 HowTo doc | 0.2 | \\banfiler01\Software Engineering\Projects\GT\GT0280\Technical\PREQ890\ How To configure C32.doc |

## Open Issues

None.

## Glossary

The following technical terms and abbreviations are used within this document.

| Term | Definition |
| --- | --- |
| TSY | Telephony System Module |
| USB | Universal Serial Bus |
| Etel | Symbian Telephony Server |
| AT commands | Method to control external modems |
| CSY | Serial communications module |
| C32 | Serial communications server |

## Document History

| Date | Version | Status | Description |
| --- | --- | --- | --- |
| 01-03-2004 | 0.1 | Draft | First draft based on the Subsystem Architectural Description Template. |
| 08-03-2004 | 0.2 | On Review | Update after review comments |
| 25-03-2004 | 1.0 | Issued | Document Issued |
| 08-06-2004 | 1.1 | On Review | Updated Security section. |
| 29-06-2004 | 1.2 | Issued | Document Issued |
| 30-09-2004 | 1.3 | Issued | Updated for Symbian OS 9.0. |
| 30-03-2007 | 1.4 | Issued | Updated with C32 server as a multi-threaded for v9.5 |