Contents

1	Introduction	3

2	Overview	3

3	Configuring Tables in the Comms Database for a CSD Connection	3

3.1	Network	3

3.2	Location	3

3.3	ModemBearer	3

3.4	DialOutISP	3

3.5	Chargecard	3

3.6	IAP	3

3.7	GlobalSettings	3

3.8	ConnectionPreferences	4

4	Sample Configuration File	4

[GlobalSettings]	4

# COMMDB_ID = 1	4

Name=GlobalSettingsRecord	4

FIELD_COUNT=15	5

END_ADD	5

[ConnectionPreferences]	5

# COMMDB_ID = 1	5

Name=First Choice	5

[Network]	5

# COMMDB_ID = 1	5

Name=Office Intranet	5

[Location]	5

# COMMDB_ID = 1	5

Name= Direct Dial from Office in UK	5

[ModemBearer]	6

# COMMDB_ID = 1	6

Name= Dacom Surfer	6

[Chargecard]	7

# COMMDB_ID = 1	7

Name=Dummy Chargecard Record	7

[DialOutISP]	7

# COMMDB_ID = 1	7

Name=ISP in UK	7

[IAP]	8

#COMMDB_ID = 1	8

Name=CSD: From UK location, dial out to an ISP using a Modem	8

5	Further Information	9

5.1	References	9

5.2	Glossary	9

5.3	Document History	9

# Introduction

This document explains one particular use case for the Comms database: configuring an outbound dial-up connection over a circuit-switched telephone network, i.e., configuring a circuit-switched data (CSD) connection.

The Comms database stores configuration data that supports network connections. See How to Configure the Comms Database to Support Network Connections [R1] for an overview and for other references.

# Overview

This document explains how to configure an outbound dial-up connection over a circuit-switched telephone network. This use case could apply to a user of a wireless device who wants to connect to an internet service provider (ISP) over a public telephone network.

# Configuring Tables in the Comms Database for a CSD Connection

You must configure at least one record in each of the tables described below. You may need to move between tables, but we suggest configuring the tables in the order below

## Network

This table contains the configuration settings for each available network. Each record identifies one available network used for multihoming (maintaining multiple connections, sometimes over multiple technologies).  You must have one record in this table and it is important to note that two network records will be used to setup a VPN connection. One record will refer to VPN and the other one will address to real network.

## Location

This table holds the details of device's location information. It is used to configure a record to support outgoing connections from the user’s country, area code, etc.

## ModemBearer

This table configures a record to support outgoing modem-based connections over GPRS network. The Modem Bearer table contains information about modems and physical connections to networks.

## DialOutISP

This table is used to configure a record with the telephone number and other settings for the internet service provided by the ISP.

## Chargecard

If necessary, configure a record with the data for the credit card or charge card that will pay connection related services. Configure a record for this network connection.

## IAP

This IAP record must refer to the above records in Network, Location, ModemBearer, and DialoutISP tables.

## GlobalSettings

GlobalSettings can have only one record. Make sure that:

- ModemForDataAndFax has the id of the appropriate record in ModemBearer.

- LocationforDataAndFax has the id of the appropriate record in Location.

- DefaultNetwork refers to a valid record id in Network.

## ConnectionPreferences

ConnectionPreferences must have at least one record and no more than two records with a Ranking of 1 or 2.

Make sure that at least one of these records has the following settings:

- Direction=OUTGOING

- BearerSet=CSD

- DialogPref=DONOTPROMPT (or another valid value)

- IAP has the number of the appropriate record in the IAP table.

# Sample Configuration File

Below is a configuration file that will generate a Comms database that will support CSD connections. Fields values that refer to other tables or other records are hyperlinked.

Notes:

- For all tables not configured below, CED will generate tables with no records.

- Not all the fields below are relevant for a CSD connection. Some may refer to records that are not defined in this sample configuration file.

# [GlobalSettings]

ADD_SECTION

## # COMMDB_ID = 1

## Name=GlobalSettingsRecord

# ConnectionAttempts must be set to 1 or 2

ConnectionAttempts=2

RedialAttempts=3

# This value for SMSBearer is regarded as sensible for CSD connections,

# whereby data is only sent packet switched if circuit switched is not

# available.

# 3 is ESMSBearerCircuitPreferred

SmsBearer= 3

# 2 is EReceiveUnstoredClientAck

SmsReceiveMode= 2

# The following GPRS fields are considered less significant when dealing	# with CSD connections and can be regarded as placeholders for fields,

# which cannot contain null values.

# GPRSAttachMode is set to EAttachWhenNeeded

GPRSAttachMode= 1

# AcceptIncomingGprs is set to ETRUE

AcceptIncomingGprs=1

GPRSClassCBearer=GSM

# General settings, will differ when the record IDs are different

ModemForDataAndFax=1

ModemForPhoneServicesAndSMS=1

LocationForDataAndFax=1

LocationForPhoneServicesAndSMS=1

# Default connection to the network specified in record one from the

#Network table

DefaultNetwork=1

MaxMBufHeap=2

BearerAvailabilityCheckTSY=mm

# FIELD_COUNT=15

# END_ADD

# [ConnectionPreferences]

ADD_SECTION

## # COMMDB_ID = 1

## Name=First Choice

# Ranking setting must be set to one if connection to this IAP is to be

# made default.

Ranking=1

Direction=OUTGOING

# BearerSet must be set to CSD; We need a bearer capable of supporting

# circuit switched connections.

BearerSet=CSD

DialogPref=DONOTPROMPT

# IAP record one is set as it offers ‘dial up’ services

IAP=1

FIELD_COUNT=6

END_ADD

# [Network]

ADD_SECTION

## # COMMDB_ID = 1

## Name=Office Intranet

FIELD_COUNT=1

END_ADD

# [Location]

ADD_SECTION

## # COMMDB_ID = 1

## Name= Direct Dial from Office in UK

# These are general settings, which may be used for both circuit and #packet switched connections and only differ if obviously the location #is different.

IntlPrefixCode=00

NatPrefixCode=0

NatCode=44

AreaCode=207

Mobile=FALSE

UsePulseDial=FALSE

WaitForDialTone=FALSE

PauseAfterDialout=0

FIELD_COUNT=9

END_ADD

# [ModemBearer]

ADD_SECTION

## # COMMDB_ID = 1

# 	These are general settings which can safely set for both circuit and    #	packet switched connections, apart from field ‘Agent’ which will differ #	depending on whether a CSD or a PSD agent is in use.

#	Most fields are Hayes commands and can only be set to one value for that #     particular field

## Name= Dacom Surfer

IfName=PPP    #The interface name that is used to implement a network #layer protocol such as PPP

PortName=COMM::0 # The Comm port name used by the modem

TSYName=DUMMY

CSYName=ECUART

LastSocketActivityTimeout=6

LastSessionClosedTimeout=180

LastSocketClosedTimeout=10

DataBits=8   #Number of data bits used for serial port configuration

StopBits=1

Parity=NONE

Rate=57600	#The baud rate of the modem.

Handshaking=244

SpecialRate=0

XonChar=0

XoffChar=0

FaxClassPref=AUTO	#Preferred fax modem class supported by the fax client

SpeakerPref=AFTERDIALUNTILANSWER

SpeakerVolPref=QUIET

ModemInitString=AT&F

DataInitString=AT

FaxInitString=AT&d2

DialPauseLength=S8=   #Initialsation string specific to a particular ISP

SpeakerVolContorlLow=L0

SpeakerVolControlMedium=L1

SpeakerVolControlHigh=L2

SpeakerAlwaysOff=M0

SpeakerOnUntilCarrier=M1

SpeakerAlwaysOn=M2

SpeakerOnAfterUntilCarrier=M3

DialToneWaitModifier=W

CallProgress1=X1	#Disable busy and dial tone detection

CallProgress2=X2	#Dial tone detection is enabled and busy detection is #disabled

CallProgress3=X3	#Dial tone detection is disabled and busy detection is #enabled

CallProgress4=X4	#Both dial tone and busy detection are enabled

EchoOff=E0

VerboseText=V1

QuietOff=Q0

QuietOn=Q1

DialCommandStateModifier=;	#Dial command modifier- used to return to #command mode after dialling.

OnLine=O

ResetConfiguration=Z

ReturnToFactoryDefs=&F

DCDOnDuringLink=&C1

DTRHangUp=&D2

DSRAlwaysOn=&S0

RTSCTSHandshake=&K3

XonXoffHandshake=&K4	#Command the modem to use software flow control

EscapeCharacter=+

EscapeGuardPeriod=S12	#Command the modem to use a particular escape #sequence guard period.

NoDialTone=NO DIAL TONE

Busy=BUSY

NoAnswer=NO ANSWER

Carrier=CARRIER

Connect=CONNECT

CompressionClass5=COMPRESSION:CLASS 5

CompressionV42bis=COMPRESSION :V.42 bis

CompressionNone=COMPRESSION :NONE

ProtocolLAPD=PROTOCOL:LAPD

ProtocolALT=PROTOCOL:ALT

ProtocolALTCELLULAR=PROTOCOL:ALT-CELLULAR

ProtocolNone=PROTOCOL:NONE

MessageValidityPeriod=0	#Validity period for SMS in minutes

MessageDeliveryReport=FALSE

Agent=DummyPSD.agt	#The file name of the agent used

FIELD_COUNT=65

END_ADD

# [Chargecard]

ADD_SECTION

## # COMMDB_ID = 1

## Name=Dummy Chargecard Record

# Dummy example of a Chargecard record

AccountNumber=0500800800,,12345678

Pin=****

LocalRule=HG

NatRule=J,K,0FG

IntlRule=HEFG

FIELD_COUNT=6

END_ADD

# [DialOutISP]

ADD_SECTION

## # COMMDB_ID = 1

# These are general settings which can safely set in order to establish # a circuit switched connection

## Name=ISP in UK

Description=For documentation.

Type=INTERNETONLY

DefaultTelNum=+442079460224

DialResolution=TRUE

UseLoginScript=FALSE

LoginScript=New Record

PromptForLogin=FALSE

DisplayPCT=FALSE

IfNetworks=ip

IfPromptForAuth=FALSE

AuthRetries=0

IfCallbackEnabled=FALSE

CallbackTimeout=0

IpAddrFromServer=TRUE

IpDNSAddrFromServer=TRUE

EnableIPHeaderComp=TRUE

EnableLCPExtension=FALSE

DisablePlainTextAuth=FALSE

EnableSWComp=TRUE

BearerName=ASYNCHRONOUS

BearerSpeed=9600

BearerCE=UNSPECIFIED

BearerType=HSCSD

ChannelCoding=144

Aiur=1400

RequestedTimeSlots=1

MaximumTimeSlots=4

BearerService=1

BearerProtocol=V32

RlpVersion=0

IwfToMs=0

MsToIwf=0

AckTimer=0

RetransmissionAttempts=0

ResequencePeriod=0

V42Compression=0

V42Codewords=0

V42MaxLength=0

Asymmetry=0

UserInitUpgrade=FALSE

UseEdge=FALSE

FIELD_COUNT=42

END_ADD

# [IAP]

ADD_SECTION

## #COMMDB_ID = 1

#  This IAP is chosen as it offers ‘dial up’ services, most appropriate #  when setting up circuit switched connections

## Name=CSD: From UK location, dial out to an ISP using a Modem

IAPServiceType=DialOutISP

# Record one is used from the servicing table ‘DialOutISP’

IAPService=1

IAPBearerType=ModemBearer

# Record one is used from the ModemBearer table

IAPBearer=1

# This IAP will connect to the network specified in record one of the #Network table

IAPNetwork=1

IAPNetworkWeighting=0

Location=1

FIELD_COUNT=8

END_ADD

Here is a ready made configuration file. After saving onto your local machine, rename the file to “Csd.cfg”, since after you open it, the file name would have been changed to “PKGXX” where X denotes some automated number.

# Further Information

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [R1] | How to Configure the Comms Database to Support Network Connections | 1.0 |  |

## Glossary

| Term | Definition |
| --- | --- |
| LAN | Local Area Network |
| CSD | Circuit Switched Data |
| IAP | Internet Access Points |
| ISP | Internet Service Provider |

## Document History

| Date | Version | Status | Description |
| --- | --- | --- | --- |
| 09.12.06 | 0.1 | Draft | This document was written as a guide to showing how to configure CSD connections. |
| 12.12.06 | 1.0 | Awaiting review | This document was further modified and verified to ensure the details were correct and accurate. |
| 21.07.06 | 1.1 | Issued | This document was written as a guide to showing how to configure CSD connections. |