# BR1424 & BR1425 Migration How-To

| Security Classification: | RESTRICTED |
| --- | --- |
| Document Number | SGL.SWM051.504 |
| Status: | DRAFT |
| Version: | 0.1 |
| Team/Department : | Core Development/Networking Technologies |
| Author(s): | Remek Zajac |
| Owner(s): | Comms-Infras Technology Architect |

| Approver(s): | Comms-Infras Technology Architect | Distribution: | Networking Technologies Team |
| --- | --- | --- | --- |

# Purpose

This document outlines the migration procedure for Break Requests: BR1424 & BR1425. Both requests are tightly coupled and it is hence pointless to cover them independently.

# Summary

## BR1424

Title: Deprecate GPRS/UMTS R97/R98 QoS Parameters.

This Break Requests both:

- Deprecates the support for R97/R99 revision of 3GPP QoS parameters (as per 3GPP TS 23.107 v99).

- Introduces the support for R99/R4 revision of 3GPP QoS parameters (as per 3GPP TS 23.107 v.4).

### R97/R98

The R97/R98 parameters can be set in the comms database (CommsDAT) and are used (read as defaults) by PSD.Agent [2.2 BR1425]. The above are the only references in to the deprecated parameters.

The following items are subjects to deprecation:

GPRS_REQ_PRECEDENCE;

GPRS_REQ_DELAY;

GPRS_REQ_RELIABILITY;

GPRS_REQ_PEAK_THROUGHPUT;

GPRS_REQ_MEAN_THROUGHPUT;

GPRS_MIN_PRECEDENCE;

GPRS_MIN_DELAY;

GPRS_MIN_RELIABILITY;

GPRS_MIN_PEAK_THROUGHPUT;

GPRS_MIN_MEAN_THROUGHPUT;

The items are fields in IncomingGprsTable and OutgoingGprsTable of CommsDat.

### R99/R4

Rather than adding the new parameters directly to IncomingGprsTable and OutgoingGprsTable, BR1424 introduces a new table to list QoS parameters exclusively. IncomingGprsTable and OutgoingGprsTable are modified to refer a specific row in that table.

The new table is:

QOS_UMTS_R99_AND_ON_TABLE

{

GPRS_QOS_REQ_TRAFFIC_CLASS

GPRS_QOS_MIN_TRAFFIC_CLASS

GPRS_QOS_REQ_DELIVERY_ORDER

GPRS_QOS_MIN_DELIVERY_ORDER

GPRS_QOS_REQ_DELIVER_ERRONEOUS_SDU

GPRS_QOS_MIN_DELIVER_ERRONEOUS_SDU

GPRS_QOS_REQ_MAX_SDUSIZE

GPRS_QOS_MIN_ACCEPTABLE_MAX_SDU_SIZE

GPRS_QOS_REQ_MAX_UPLINK_RATE

GPRS_QOS_REQ_MIN_UPLINK_RATE

GPRS_QOS_REQ_MAX_DOWNLINK_RATE

GPRS_QOS_REQ_MIN_DOWNLINK_RATE

GPRS_QOS_REQ_BER

GPRS_QOS_MAX_BER

GPRS_QOS_REQ_SDU_ERROR_RATIO

GPRS_QOS_MAX_SDU_ERROR_RATIO

GPRS_QOS_REQ_TRAFFIC_HANDLING_PRIORITY

GPRS_QOS_MIN_TRAFFIC_HANDLING_PRIORITY

GPRS_QOS_REQ_TRANSFER_DELAY

GPRS_QOS_MAX_TRANSFER_DELAY

GPRS_QOS_REQ_GUARANTEED_UPLINK_RATE

GPRS_QOS_MIN_GUARANTEED_UPLINK_RATE

GPRS_QOS_REQ_GUARANTEED_DOWNLINK_RATE

GPRS_QOS_MIN_GUARANTEED_DOWNLINK_RATE

GPRS_QOS_SIGNALLING_INDICATOR

GPRS_QOS_IM_CN_SIGNALLING_INDICATOR

GPRS_QOS_SOURCE_STATISTICS_DESCRIPTOR

}

## BR1425

Title: Deprecate PSD Agent

This Break Requests deprecates the usage of PSD.Agent. The context is as follows:

- PSD.Agent expects R97/R98 QoS defaults in CommsDAT.

- Symbian’s 3GNif (coupled with Nullagent) replaces the UMTS functionality of PSD.Agent.

- The 3GNif supports R99/R4 QoS parameters.

- The Symbian Telephony deprecates their support for R97/98 QoS parameters.

Symbian Networking wants to encourage migration towards 3GNif and hence does not plan to modify PSD.Agent to migrate from R97/98 to R99/R4 QoS. Consequently PSD.Agent has got to be deprecated along with R97/R98 parameters.

## Migration Procedure

This section details all the steps necessary to:

- Migrate from R97/R98 to R99/R4 parameters

- Migrate from PSD.Agent to 3GNif (a.k.a. SPUD)

For each database configuration <file> (ced.exe input file), be it *.cfg or *.xml that is a subject to migration, i.e.: has at least one ModemBearerTable entry refering PSD.agt  as it's agent:

{

1) Migrate from PSD.Agt to SPUD, i.e.:

For each IAPTable <entry>, where the <entry>'s IAPBearer points at a

<ModemBearer record>, which refers PSD.AGT as its Agent.

{

In the <ModemBearer record>:

{

1.1) Change PSD.Agt to NULLAGT

1.2) Change IfName to refer SPUD.

}

In the table record pointed by the <entry>::IAPService

{

1.3) Add lowernif=<nif> to IfParams, where <nif> should refer

what was previously IfName (see (1.2)).

We assume <nif> can only be PPP or rawip.

}

1.4) Save the file

}

2) Convert QoS parameters from R97/R98 to R4/R99, i.e.:

2.1) build the binary database file by running 'ced.exe <file>'

2.2) perform the parameter conversion running 'ceddump -CR99'

ceddump will produce cedout.cfg output in where it was run.

2.3) In case the <file> is an *.xml file, the ceddump output can be converted

from *.cfg to *.xml using 'cfg2xml -mode:file cedout.cfg'

2.4) Replace the <file> with the output (either 2.2 or 2.3).

}