Contents

1	Introduction	3

1.1	Purpose and Scope	3

2	Background	3

3	RSubConnection API	4

3.1	Overview of RSubConnection Functionality	4

3.2	RSubConnection Characteristics	5

3.3	RSubConnection Class	6

3.4	RSocket Class	8

4	RSubConnection API Related Classes	9

4.1	RSubConParameterBundle	9

4.2	CSubConParameterFamily	10

4.3	CSubConGenericParameterSet	11

4.4	CSubConExtensionParameterSet	12

4.5	CSubConNotificationEvent	12

5	RSubConnection API Events	13

5.1	CSubConGenEventDataClientJoined, CSubConGenEventDataClientLeft	13

5.2	CSubConGenEventParamsGranted	14

5.3	CSubConGenEventParamsRejected	15

5.4	CSubConGenEventParamsChanged	16

5.5	CSubConGenEventSubConDown	16

6	QoS Family Classes	17

6.1	QoS Parameters Description	17

6.2	CSubConQosGenericParamSet	18

6.3	CSubConQosR99ParamSet	19

6.4	CSubConQosR5ParamSet	20

6.5	CSubConIMSExtParamSet	20

7	Authorisation Family Classes	21

7.1	CSubConAuthorisationGenericParamSet	21

7.2	CSubConSBLPR5ExtensionParamSet	21

8	Code Examples	22

8.1	Attaching to the default Sub-Connection	22

8.2	Creating a new sub-connection - Socket connected over the sub-connection	23

8.3	Creating a new sub-connection - Adding an already connected socket	24

8.4	Creating and setting properties for a Sub-Connection	25

8.5	Registering for events – Simple case	26

8.6	Registering for events – Using filters	27

8.7	Extracting information from received events	28

8.8	Working Code Examples	28

9	Further Information	29

9.1	Glossary	29

9.2	People	29

9.3	References	29

9.4	Open Issues	29

9.5	Document History	30

# Introduction

## Purpose and Scope

This document describes the RSubConnection API introduced into release 9.0 of the Symbian OS to replace the existing IP specific Quality of Service (QoS) RQosChannel API. The RSubConnection API provides the functionality to divide a Connection (provided by the RConnection API) into channels that have properties. An example of a set of parameters that could be used are QoS parameters such as bandwidth and latency.

There is an overlap between the RSubConnection API and the RConnection API as both provide sub-connection management. The RConnection API provides overall sub-connection management for an entire connection whereas the RSubConnection API allows the individual setting of properties of a sub-connection as well as the ability to associate sockets (RSocket) to sub-connections. The sockets are then able to take advantage of the sub-connections properties whilst they send and receive data.

This is not a design document, and does not go into details of the internal implementation of the API, but rather describes how the API is to be used by clients, and implications for the RSocket API. For this reason and for clarity private and protected members have been omitted from the class definitions found in later sections.

Examples code fragments demonstrating a variety of ways that the RSubConnection API can be used are located towards the end of this document.

# Background

The only API capable of providing Quality of Service functionality in earlier releases of Symbian OS was RQoSChannel. This API provides IP specific QoS services and would not work with other protocols such as Bluetooth. A product requirement was drawn up to introduce an API that would provide channels that sockets would share and these channels would have properties that included Quality of Service parameters. The RSubConnection API was created to fulfil this requirement allowing an application to create channels on a Connection (provided via the RConnection API), set properties and associate sockets with them.

The new RSubConnection API is provided by the Socket Server component ESock as it already provides the Connection and Socket APIs that are independent of protocol.

# RSubConnection API

## Overview of RSubConnection Functionality

The RSubConnection API will provide the following functionality:

- Create a sub-connection (channel), given the connection (RConnection) to place the channel on, and assign properties to it

- Assign Quality of Service Parameters, both generic and technology specific parameters

- Associate a number of sockets with the channel

- Obtain progress notification on a sub-connection

- Allow an application to connect to the default sub-connection and set properties.

Note: Attaching to the default sub-connection is not supported in Symbian OS v9.2 and previous releases.

- Move a socket from the default sub-connection of an interface to a specific sub-connection on the same interface

- Move a socket from a specific sub-connection to the default sub-connection of the same interface

- Close a sub-connection

The RSocket API has been added to, to allow it to be created with a sub-connection as well as a connection.  A client will be able to start a sub-connection via RSubConnection and associate individual RSocket objects with that sub-connection.  This will ensure that all data flowing over those objects will use the particular sub-connection previously started by the application.  This will be implemented using new RSocket::Open(…) method which will take an additional argument specifying the sub-connection to use for the object in question.

As well as opening sub-connections explicitly via RSubConnection, applications may also attach to the default sub-connection (e.g. In GPRS, the default could be the Primary PDP context without a TFT), and modify and retrieve its properties.

Several classes have been added to support the setting of properties on sub-connections. Related properties are collected together into either generic or extension parameter sets. A generic set must be technology independent, so that use of it could equally be applied regardless of the underlying technology. An extension parameter set should be used to contain properties that are specific to a technology (e.g. the IM CN Signalling Indicator flag). Parameter sets are grouped together into technology independent families (e.g. QoS and Authorisation) represented by the CSubConParameterFamily class (section 4.2), which in turn are contained in a parameter bundle represented by the RSubConParameterBundle class (section 4.1). A parameter family can contain three types of each generic or extension set. These types are requested (ERequested), acceptable (EAcceptable), and granted (EGranted). The requested and acceptable types are used to request properties in a given range, and the granted type is used to identify the current properties on the sub-connection. These are described more in-depth in section 4.2. The containment (and ownership) of instantiations of each of these classes can be seen in Figure 1.

Note: Start(),Start(TRequestStatus &aStatus) and Stop() methods are not currently in use.

Figure 1 – Containment of sub-connection property classes

## RSubConnection Characteristics

Some general characteristics of RSubConnection objects are described:

- RSubConnection objects are implemented as sub-sessions to ESOCK, in the same way as RConnection,  RSocket and RHostResolver objects.  Consequently, they need to be opened on an existing ESOCK session.

- Rather like RSocket/RHostResolver objects, an individual RSubConnection object is not designed to be used by multiple clients simultaneously (ie: have multiple requests outstanding on it from different clients).

- Although RSubConnection objects are automatically closed when the corresponding ESOCK session is closed, appropriate use of CleanupClosePushL(RSubConnection&) is more likely to ensure proper operation.

- There is no restriction on the number of RSubConnection objects per ESOCK session.  It is also possible for several RSubConnection objects to refer to the same underlying default sub-connection.

- As mentioned earlier, RSocket objects can be associated with an RSubConnection object via additions to their respective APIs (described later).  In this case, data will flow over the same underlying interface as the RSubConnection to which they have been associated.  For this to work, the RSocket/RSubConnection/RConnection objects must all be on the same ESOCK session, and the RSubConnection object must have been previously opened.

## RSubConnection Class

### Description

The Open() method is used to open an RSubConnection object on an ESOCK session. The RConnection used for this must already be active. A sub-connection type is passed as a parameter to this method to select whether to attach to the default sub-connection (the RConnection), or create a new one. As with most of the calls in the RSubConnection API a system wide error code is returned, returning KErrNone upon success.

The Add() method is used to move a socket from the default sub-connection to the RSubConnection. The Remove() method will return the socket to the default sub-connection. Although these methods both return system wide error codes, the error code only represents the success or failure of the request and initialisation to perform the action. The action of moving the socket is completed asynchronously, and the socket must be connected in order for the call to complete. It is not permitted to move a socket from one sub-connection to another, nor is it permitted to move a socket to a sub-connection that has been opened on a different RConnection. When the socket has been added, or has been removed from a sub-connection, the events CSubConGenEventDataClientJoining, or CSubConGenEventDataClientLeaving respectively will be notified.

The GetParameters() and SetParameters() methods are used to retrieve and set bundles of properties on the sub-connection. GetParameters() will return KErrNotReady if no properties have been negotiated. This may not happen until the sub-connection has been used. The SetParameters() method, like the Add() and Remove() methods returns an error code indicating the success or failure of the request to perform the action. Upon negotiation of the properties either the CSubConGenEventParamsGranted or CSubConGenEventParamsRejected event will be notified for each family within the parameter bundle. A call to the SetParameters() method may not result in any negotiation until such time as the sub-connection is used.

With 9.3, there are some non-intuitive behaviours of GetParameters() and SetParameters() when there are multiple RSubConnections attached to the default sub-connection. These are:

- GetParameters() will return KErrNotReady until parameters for the default sub-connection are negotiated either as a result of the network status changing or new parameters are requested through SetParameters().

- SetParameters() will cause CSubConGenEventParamsGranted or –Rejected events. These events are reported to all RSubConnections that are attached to the default connection, not just the RSubConnection that requested the parameters be set.

- In the parameter bundles retrieved by GetParameters() any granted parameters will be the current granted parameters and be the same for all RSubConnection objects attached to the default sub-connection. Any other parameters, including acceptable and requested parameters, are not shared between multiple RSubConnections attached to the default sub-connection and will be those from the last call to SetParameters() on that RSubConnection.

The EventNotification() methods are used to asynchronously register for event notifications. The methods support filtering of events in two different ways, via a boolean flag to receive notification of generic or all events, and via an array of TEventFilter’s. These are described more in depth later with the code examples (see sections 8.5-8.7). In order to avoid missing any notifications it is recommended to re-register for notifications before handling the received notification. Registration for event notifications is cancelled with the CancelEventNotification() method.

Generic control of the sub-connection is provided by the Control() method. Use of this method is dependant upon support provided by the underlying sub-connection provider technology.

### C++ Definition

class RSubConnection : public RSubSessionBase /** * An Sub-Connection, a channel within a Connection. A representation of a channel between this device and remote devices with which we are communicating. This channel will be used by one or more sockets. * Depending on the state of the channel, it may not be possible to bind arbitary sockets into it. Attempts to bind sockets from different protocol families to a single channel is an error, as each channel can only be used by one protocol family. * @note The sub-connection can represent a end-to-end channel and/or a channel from this device to an intermediate device (e.g an access server such as a GGSN which using UMTS and PDP contexts. Properties can be specified simultaneously on protocol and link level. */ { public: enum TSubConnType { EAttachToDefault, ECreateNew }; public: // Sub Connection Management IMPORT_C RSubConnection(); IMPORT_C TInt Open(RSocketServ& aServer, TSubConnType aSubConnType, RConnection& aConnection); IMPORT_C void Close(); // Socket Management IMPORT_C TInt Add(RSocket& aSocket, TRequestStatus& aStatus); IMPORT_C TInt Remove(RSocket& aSocket, TRequestStatus& aStatus); // QoS Properties IMPORT_C TInt SetParameters(const RSubConParameterBundle& aParametersSet); IMPORT_C TInt GetParameters(RSubConParameterBundle& aParametersSet); // Event Notification IMPORT_C void EventNotification(TNotificationEventBuf &aEventBuffer, TBool aGenericEventsOnly, TRequestStatus &aStatus); IMPORT_C void EventNotification(TNotificationEventBuf &aEventBuffer, TEventFilter aEventFilterList[], TUint aEventListLength, TRequestStatus &aStatus); IMPORT_C void CancelEventNotification(); // Generic Control IMPORT_C TInt Control(TUint aOptionLevel, TUint aOptionName, TDes8& aOption); };

## RSocket Class

### Description

In order to allow an RSocket to be associated with an RSubConnection, an additional Open() method has been added to RSocket.  This new method takes the form of one of the original Open() methods but takes an additional RSubConnection& to associate the socket with. A system wide error code is returned, as with the existing Open() methods. The socket will not actually be added to the sub-connection until such time as the socket is connected.

### C++ Definition

For clarity the class definition only shows the additions.

class RSocket : public RSubSessionBase { … public: … IMPORT_C TInt Open(RSocketServ& aServer,TUint addrFamily TUint sockType,TUint protocol, RSubConnection& aSubConnection); … };

# RSubConnection API Related Classes

## RSubConParameterBundle

### Description

This class is intended to hold a collection (or bundle) of parameter families for an individual sub-connection.  An instance of a parameter bundle can only be applied wholly. That is, if one family is added and a call is made to RSubConnection::SetParameters(), to add another family or an additional extension set, it must be added to the original RSubConParameterBundle instance and another call made to RSubConnection::SetParameters(). If the original parameter bundle is not available a call must be made to RSubConnection::GetParameters() to obtain the current settings. When a family is added to the bundle, the bundle will take ownership of that object. All added CSubConParameterFamily objects will be destroyed along with the parameter bundle.

The AddFamilyL() and FindFamily() methods do just as their names suggest. It is unlikely that the AddFamilyL() call will ever be required since the constructor of CSubConParameterFamily adds the family to the passed in parameter bundle automatically. It is present mainly for internal use.

The ClearAllParameters() method will clear parameters of the given parameter set type from all families that the bundle owns.

The Load() and Store() methods are used for serialisation of the parameter bundle, whilst Length() will return the number of bytes the serialised data will occupy.

### C++ Definition

class RSubConParameterBundle /** Container of RSubConnection parameter families. May contain and 0..N parameter families. @publishedAll @released since v9.0 */ { public: IMPORT_C RSubConParameterBundle(); IMPORT_C void Close(); IMPORT_C TUint Length() const; IMPORT_C TInt Load(const TDesC8& aDes); IMPORT_C TInt Store(TDes8& aDes) const; public: IMPORT_C void AddFamilyL(CSubConParameterFamily* aFamily); IMPORT_C CSubConParameterFamily* FindFamily (TUint32 aFamilyId); IMPORT_C void ClearAllParameters(CSubConParameterFamily::TParameterSetType aType); };

## CSubConParameterFamily

### Description

A CSubConParameterFamily holds a generic and zero or more extension parameter sets that relate to a particular family of parameters. At present only two families are defined. These are QoS (KSubConQoSFamily) and Authorisation (KSubConAuthorisationFamily), which are described in more detail in later sections. The parameter sets contained by a CSubConParameterFamily belong to one of the three types defined in TParameterSetType. The three types are ERequested, EAcceptable, and EGranted. The simplest way to explain their meaning is in the context of QoS. An application would add a parameter set of the type ERequested to request an ideal level of bandwidth and latency, and a parameter set type of EAcceptable could be added to indicate the bare minimum required for the application to operate. A parameter set of the type EGranted must never be added by an application. The RSubConnection’s parameter bundle will be updated with parameter sets of this type when the negotiated settings change.

When a generic or extension set is added to the family, the parameter family object will take ownership of that parameter set. All added parameter sets will be deleted along with the parameter family. Before a call to SetParameters() is made on the sub-connection the family must at least contain a parameter set of the type ERequested. If the parameter set type EAcceptable is omitted the requested values will used as the acceptable values. Any parameter sets of the type EGranted will be ignored by a call to SetParameters().

The LoadL() method is used to create a new CSubConParameterFamily from a previously serialised object. It will be added to the given bundle, which will taken ownership of the new family.

The SetGenericSetL() and AddExtensionSetL() methods are used to add parameter sets to the family. If the type of parameter set added/set is the same as a parameter set type already contained by the CSubConParameterFamily the method will leave with KErrAlreadyExists. For example, if a generic set is added for the type ERequested, then another generic set is also added for ERequested the method will leave. For extension sets the method will leave if both the parameter set type and extension type match an existing parameter set.

The GetGenericSet() and FindExtensionSet() methods are used to retrieve a parameter set from the parameter family. If there is no generic set, or the extension set of the specified type NULL is returned.

The ClearAllParameters() method will clear both generic and extension parameter sets of the given parameter set type from the family.

Id() will return the family identifier.

The Load() and Store() methods are used for serialisation of the parameter family, whilst Length() will return the number of bytes the serialised data will occupy.

### C++ Definition

class CSubConParameterFamily : public CBase /** Container of RSubConnection parameter sets. For each Parameter Type (Requested, Acceptable and Granted) it contains one generic and 0..N extended parameter sets. @publishedAll @released since v9.0 */ { public: enum TParameterSetType { ERequested = 0, EAcceptable = 1, EGranted = 2 }; public: IMPORT_C static CSubConParameterFamily* NewL(RSubConParameterBundle& aBundle, TUint32 aFamilyId); IMPORT_C static CSubConParameterFamily* LoadL(RSubConParameterBundle& aBundle, TPtrC8& aBuffer); public: IMPORT_C virtual ~CSubConParameterFamily(); IMPORT_C void SetGenericSetL(CSubConGenericParameterSet& aGenericSet, TParameterSetType aType); IMPORT_C CSubConGenericParameterSet* GetGenericSet(TParameterSetType aType); IMPORT_C void AddExtensionSetL(CSubConExtensionParameterSet& aExtensionSet, TParameterSetType aType); IMPORT_C CSubConExtensionParameterSet* FindExtensionSet(STypeId aSetId, TParameterSetType aType); IMPORT_C void ClearAllParameters(TParameterSetType aType); inline TUint32 Id(); IMPORT_C TUint Length() const; IMPORT_C TInt Load(TPtrC8& aDes); IMPORT_C TInt Store(TDes8& aDes) const; };

## CSubConGenericParameterSet

### Description

All family specific generic parameter sets must derive from this class. It provides no specific functionality.

### C++ Definition

/** Base class for generic RSubConnection parameter sets. @publishedAll @released since v9.0 */ class CSubConGenericParameterSet : public CSubConParameterSet { public: IMPORT_C ~CSubConGenericParameterSet(); };

## CSubConExtensionParameterSet

### Description

All family specific extension parameter sets must derive from this class. It provides no specific functionality.

### C++ Definition

/** Base class for extended RSubConnection parameter sets. @publishedAll @released since v9.0 */ class CSubConExtensionParameterSet : public CSubConParameterSet { public: IMPORT_C ~CSubConExtensionParameterSet(); };

## CSubConNotificationEvent

### Description

Both generic and extension sub-connection events will derive from this class. The rules for generic and extension events are the same as for parameter sets. That is, a generic event should be able to be understood by all technologies.

The IsGeneric() method unsurprisingly identifies whether or not the event is in fact generic.

The GroupId() method returns the UID of the factory that contains the event, and Id() returns the class type Id within that factory. These two pieces of information comprise the STypeId of the event.

An important thing to note about these events is that the notification mechanism used makes them unidirectional. Many of the events contain setter methods for values within the event. Using these setters to change values will in no way affect the values stored within Esock or those used by the network. The setters are present to support the creation of the event notifications and must not be used by applications.

### C++ Definition

class CSubConNotificationEvent : public SMetaDataECom { public: IMPORT_C static CSubConNotificationEvent* NewL(const STypeId& aTypeId); IMPORT_C static CSubConNotificationEvent* NewL(const TNotificationEventBuf& aEventBuffer); IMPORT_C TBool IsGeneric() const; IMPORT_C TInt32 GroupId() const; IMPORT_C TUint32 Id() const; };

# RSubConnection API Events

## CSubConGenEventDataClientJoined, CSubConGenEventDataClientLeft

These two events derive from CSubConGenEventDataClientBase, which provides the functionality for both. The source and destination end points of the data client are presented with this event, along with the IAP ID of the connection it was created on.

### C++ Definition

const TInt32 KSubConnGenericEventsImplUid = 0x10204306; const TUint32 KSubConGenericEventDataClientJoined = 0x2; const TUint32 KSubConGenericEventDataClientLeft = 0x4; /** Data client event base. @internalComponent @released since v9.1 */ CSubConGenEventDataClientBase : public CSubConNotificationEvent { public: inline CSubConGenEventDataClientBase(); inline TUint GetIap() const; inline void SetIap(TUint aIap); inline const TSockAddr& GetSourceAdress() const; inline void SetSourceAddress(const TSockAddr& aSourceAddress); inline const TSockAddr& GetDestAdress() const; inline void SetDestAddress(const TSockAddr& aDestAddress); virtual ~CSubConGenEventDataClientBase(); }; /** Data client joined event. @publishedAll @released since v9.1 */ class CSubConGenEventDataClientJoined : public CSubConGenEventDataClientBase { public: inline static CSubConGenEventDataClientJoined* NewL(); inline CSubConGenEventDataClientJoined(); virtual ~CSubConGenEventDataClientJoined(); }; /** Data client left event. @publishedAll @released since v9.1 */ class CSubConGenEventDataClientLeft : public CSubConGenEventDataClientBase { public: inline static CSubConGenEventDataClientLeft* NewL(); inline CSubConGenEventDataClientLeft(); virtual ~CSubConGenEventDataClientLeft(); };

## CSubConGenEventParamsGranted

### Description

Notification of this event occurs after a request to SetParameters()  has been made and negotiation with the network has been completed. A notification will be received for each family contained in the parameter bundle that was negotiated successfully. This event presents a generic set and zero or more extension sets (providing they are supported by the underlying sub-connection provider technology) of the parameter family identified by the Id return from GetFamily().

### C++ Definition

const TInt32 KSubConnGenericEventsImplUid = 0x10204306; const TUint32 KSubConGenericEventParamsGranted = 0x1; /** Sub-connection parameters granted event. @publishedAll @released since v9.1 */ class CSubConGenEventParamsGranted : public CSubConNotificationEvent { public: inline static CSubConGenEventParamsGranted* NewL(); inline CSubConGenEventParamsGranted(); inline const CSubConGenericParameterSet* const GetGenericSet() const; inline void SetGenericSet(CSubConGenericParameterSet* aGenericSet); inline const CSubConExtensionParameterSet* const GetExtensionSet(TUint aIndex) const; inline void AddExtensionSetL(CSubConExtensionParameterSet* aExtensionSet); inline TInt GetNumExtensionSets(); inline TUint32 GetFamily() const; inline void SetFamily(TUint32 aFamily); virtual ~CSubConGenEventParamsGranted(); };

## CSubConGenEventParamsRejected

### Description

Notification of this event occurs after a request to SetParameters() has been made and negotiation with the network has failed for some reason. It could be an error within the handset software/configuration, or that the network could not provide the acceptable (minimum) level of QoS. The reason for failure and the parameter family are presented by the accessor methods Error() and FamilyId(). Like the CSubConGenEventParamsGranted event, a notification for CSubConGenEventParamsRejected is received for each family in the parameter bundle that could not be negotiated successfully.

### C++ Definition

const TInt32 KSubConnGenericEventsImplUid = 0x10204306; const TUint32 KSubConGenericEventParamsRejected = 0x20; /** Sub-connection parameters rejected event. @publishedAll @released since v9.1 */ class CSubConGenEventParamsRejected : public CSubConNotificationEvent { public: inline static CSubConGenEventParamsRejected* NewL(); inline CSubConGenEventParamsRejected(); inline TInt Error() const; inline void SetError(TInt aError); inline TInt FamilyId() const; inline void SetFamilyId ( TInt aFamilyId ); virtual ~CSubConGenEventParamsRejected(); };

## CSubConGenEventParamsChanged

### Description

This event occurs when the properties of a parameter family has been renegotiated due to some event on the network. It is not sent in response to a request to change the properties. The change could be the result of an error or just that the level of QoS has improved/worsened. If a new set of parameters are available they’ll be presented as with the CSubConGenEventParamsGranted event. The error status is presented via the Error() method.

### C++ Definition

const TUint32 KSubConGenericEventParamsChanged = 0x10; const TInt32 KSubConnGenericEventsImplUid = 0x10204306; /** Sub-connection parameters changed event @publishedAll @released since v9.1 */ class CSubConGenEventParamsChanged : public CSubConGenEventParamsGranted { public: inline static CSubConGenEventParamsChanged* NewL(); inline CSubConGenEventParamsChanged(); inline TInt Error() const; inline void SetError(TInt aError); virtual ~CSubConGenEventParamsChanged(); };

## CSubConGenEventSubConDown

### Description

This event occurs when the underlying sub-connection has been lost. This could be due to request for it to be closed, or some error on the network. The error status is presented via the Error() method.

### C++ Definition

const TInt32 KSubConnGenericEventsImplUid = 0x10204306; const TUint32 KSubConGenericEventSubConDown = 0x8; /** Sub-connection down event. @publishedAll @released since v9.1 */ class CSubConGenEventSubConDown : public CSubConNotificationEvent { public: inline static CSubConGenEventSubConDown* NewL(); inline CSubConGenEventSubConDown(); inline TInt Error() const; inline void SetError(TInt aError); virtual ~CSubConGenEventSubConDown(); };

# QoS Family Classes

These classes are not so much part of the RSubConnection API as they are extensions that build on it. However, they are very closely tied to the API and almost certainly to be used by anyone using it.

## QoS Parameters Description

| Parameter | Description |
| --- | --- |
| Bandwidth | Bandwidth the client requires |
| Maximum Burst Size | Maximum size of a burst of data the client can handle |
| Average Packet Size | Average packet size required (e.g. codec use) |
| Maximum Packet Size | Maximum packet size the client can handle |
| Delay | Acceptable Delay/Latency |
| Delay Variation | Acceptable variation in delay (also known as jitter) |
| Priority | Relative priority the client expects to give this channel compared to it’s other channels |
| Header mode | Specify whether the header size should be calculated by the QoS module or specified by the client. Default is to let the QoS module calculate it. |
| Name | Identity of a “well known” set of QoS Parameters. |

## CSubConQosGenericParamSet

### Description

This class contains a collection of generic QoS family parameters. These parameters should be understood by any technology, such as IP and UMTS. Adding a generic set of parameters to a family is not necessary where one or more extensions sets have been added, providing that one of the extension sets contain conceptually identical parameters that may be mapped by an aware sub-connection provider.

### C++ Definition

const TInt32 KSubConnGenericParamsImplUid = 0x10204304; const TInt32 KSubConnQosGenericParamsType = 1; /** Set of generic QoS parameters. */ class CSubConQosGenericParamSet : public CSubConGenericParameterSet { public: IMPORT_C static CSubConQosGenericParamSet* NewL(CSubConParameterFamily& aFamily, CSubConParameterFamily::TParameterSetType aType); public: inline TInt GetDownlinkBandwidth() const; inline TInt GetUplinkBandwidth() const; inline TInt GetDownLinkMaximumBurstSize() const; inline TInt GetUpLinkMaximumBurstSize() const; inline TInt GetDownLinkAveragePacketSize() const; inline TInt GetUpLinkAveragePacketSize() const; inline TInt GetDownLinkMaximumPacketSize() const; inline TInt GetUpLinkMaximumPacketSize() const; inline TInt GetDownLinkDelay() const; inline TInt GetUpLinkDelay() const; inline TInt GetDownLinkDelayVariation() const; inline TInt GetUpLinkDelayVariation() const; inline TInt GetDownLinkPriority() const; inline TInt GetUpLinkPriority() const; inline TBool GetHeaderMode() const; inline const TName& GetName() const; inline void SetDownlinkBandwidth(TInt); inline void SetUplinkBandwidth(TInt); inline void SetDownLinkMaximumBurstSize(TInt); inline void SetUpLinkMaximumBurstSize(TInt); inline void SetDownLinkAveragePacketSize(TInt); inline void SetUpLinkAveragePacketSize(TInt); inline void SetDownLinkMaximumPacketSize(TInt); inline void SetUpLinkMaximumPacketSize(TInt); inline void SetDownLinkDelay(TInt); inline void SetUpLinkDelay(TInt); inline void SetDownLinkDelayVariation(TInt); inline void SetUpLinkDelayVariation(TInt); inline void SetDownLinkPriority(TInt); inline void SetUpLinkPriority(TInt); inline void SetHeaderMode(TBool); inline void SetName(const TName&); };

## CSubConQosR99ParamSet

### Description

This class contains the QoS parameters as defined by section 6.4.3 of the 3GPP Quality of Service (QoS) concept and architecture document [R3]. Getter and setter methods are provided for each parameter.

Note: This class is called CSubConQosIPLinkR99ParamSet in Symbian OS v9.2 and previous releases, and the constants used for it are KSubConIPParamsUid and KSubConQosIPLinkR99ParamsType.

### C++ Definition

const TInt KSubCon3GPPExtParamsFactoryUid = 0x1020D460; const TInt KSubConQosR99ParamsType = 1; class CSubConQosR99ParamSet : public CSubConExtensionParameterSet { public: inline static CSubConQosR99ParamSet* NewL(CSubConParameterFamily& aFamily, CSubConParameterFamily::TParameterSetType aType); inline static CSubConQosR99ParamSet* NewL(); inline CSubConQosR99ParamSet(); inline RPacketQoS::TTrafficClass GetTrafficClass() const; inline RPacketQoS::TDeliveryOrder GetDeliveryOrder() const; inline RPacketQoS::TErroneousSDUDelivery GetErroneousSDUDelivery() const; inline RPacketQoS::TBitErrorRatio GetResidualBitErrorRatio() const; inline RPacketQoS::TSDUErrorRatio GetSDUErrorRatio() const; inline RPacketQoS::TTrafficHandlingPriority GetTrafficHandlingPriority() const; inline TInt GetTransferDelay() const; inline TInt GetMaxSduSize() const; inline TInt GetMaxBitrateUplink() const; inline TInt GetMaxBitrateDownlink() const; inline TInt GetGuaBitrateUplink() const; inline TInt GetGuaBitrateDownlink() const; inline void SetTrafficClass(RPacketQoS::TTrafficClass aTrafficClass); inline void SetDeliveryOrder(RPacketQoS::TDeliveryOrder aDeliveryOrder); inline void SetErroneousSDUDelivery(RPacketQoS::TErroneousSDUDelivery aDeliveryOfErroneusSdu); inline void SetResidualBitErrorRatio(RPacketQoS::TBitErrorRatio aResidualBer); inline void SetSDUErrorRatio(RPacketQoS::TSDUErrorRatio aErrorRatio); inline void SetTrafficHandlingPriority(RPacketQoS::TTrafficHandlingPriority aPriority); inline void SetTransferDelay(TInt aTransferDelay); inline void SetMaxSduSize(TInt aMaxSduSize); inline void SetMaxBitrateUplink(TInt aMaxBitrateUplink); inline void SetMaxBitrateDownlink(TInt aMaxBitrateDownlink); inline void SetGuaBitrateUplink(TInt aGuaBitrateUplink); inline void SetGuaBitrateDownlink(TInt aGuaBitrateDownlink); };

## CSubConQosR5ParamSet

### Description

This class contains the QoS parameters as defined by section 6.4.3 of the 3GPP Quality of Service (QoS) concept and architecture document [R4]. They inherit from the release 4/99 set CSubConQosR99ParamSet. Although it is possible to add both this parameter set and the R4/R99 one, it is not necessary and shouldn’t be done.

Note: This class is only available in releases after Symbian OS v9.2

### C++ Definition

const TInt KSubCon3GPPExtParamsFactoryUid = 0x1020D460; const TInt KSubConQosR5ParamsType = 3; class CSubConQosR5ParamSet : public CSubConQosR99ParamSet { public: inline static CSubConQosR5ParamSet* NewL(CSubConParameterFamily& aFamily,CSubConParameterFamily::TParameterSetType aType); inline static CSubConQosR5ParamSet* NewL(); inline CSubConQosR5ParamSet(); inline TBool GetSignallingIndicator() const; inline void SetSignallingIndicator(TBool aSignallingIndicator); inline RPacketQoS::TSourceStatisticsDescriptor GetSourceStatisticsDescriptor() const; inline void SetSourceStatisticsDescriptor(RPacketQoS::TSourceStatisticsDescriptor aSrcStatsDescType); };

## CSubConIMSExtParamSet

### Description

This class contains the IM CN Signalling Indicator flag. This is defined in the section 6.4 of the 3GPP End-to-end Quality of Service (QoS) concept and architecture document [R5] and section 4.2.6 of the 3GPP IP Multimedia Subsystem document [R6].

Note: This class is only available in releases after Symbian OS v9.2

### C++ Definition

const TInt KSubCon3GPPExtParamsFactoryUid = 0x1020D460; const TInt KSubConIMSExtParamsType = 4; class CSubConIMSExtParamSet : public CSubConExtensionParameterSet { public: inline static CSubConIMSExtParamSet* NewL(CSubConParameterFamily& aFamily,CSubConParameterFamily::TParameterSetType aType); inline static CSubConIMSExtParamSet* NewL(); inline CSubConIMSExtParamSet(); // New IM CN Signalling Indicator inline TBool GetIMSSignallingIndicator() const; inline void SetIMSSignallingIndicator(TBool aIMSSignallingIndicator); };

# Authorisation Family Classes

These classes are not so much part of the RSubConnection API as they are extensions that build on it. However, they are very closely tied to the API and almost certain to be used by anyone using it.

## CSubConAuthorisationGenericParamSet

### Description

[TODO: Awaiting Information]

### C++ Definition

const TInt KSubConnGenericParamsImplUid = 0x10204304; const TInt KSubConnAuthorisationGenericParamsType = 2; class CSubConAuthorisationGenericParamSet : public CSubConGenericParameterSet { public: inline static CSubConAuthorisationGenericParamSet* NewL(CSubConParameterFamily& aFamily, CSubConParameterFamily::TParameterSetType aType); inline static CSubConAuthorisationGenericParamSet* NewL(); inline CSubConAuthorisationGenericParamSet(); inline TInt GetId() const; inline void SetId(TInt); };

## CSubConSBLPR5ExtensionParamSet

### Description

[TODO: Awaiting Information]

### C++ Definition

const TInt KSubCon3GPPExtParamsFactoryUid = 0x1020D460; const TInt KSubConnSBLPR5ExtensionParamsType = 2; class CSubConSBLPR5ExtensionParamSet : public CSubConExtensionParameterSet { public: inline static CSubConSBLPR5ExtensionParamSet* NewL(CSubConParameterFamily& aFamily, CSubConParameterFamily::TParameterSetType aType); inline static CSubConSBLPR5ExtensionParamSet* NewL(); /** Media Authorization Token setter and getter functions. */ inline const TAuthToken& GetMAT() const; inline void SetMAT(const TAuthToken& aAuthToken); /** Flow identifires setter and getter functions. */ inline TInt GetNumberOfFlowIds() const; inline const TFlowId& GetFlowIdAt(TInt aIndex) const; /** Adding of Flow Identifires into an array. */ inline void AddFlowIdL(const TFlowId& aFlowId); /** public constructors so that it can be accessed by factory. */ inline CSubConSBLPR5ExtensionParamSet(); };

# Code Examples

## Attaching to the default Sub-Connection

In the following example, the application wants to connect to the default sub-connection to set its properties.  The application simply tries to connect a socket over the connection (which uses the default sub-connection) after it has set properties on the default sub-connection. An example of parameter creation can be found in section 8.4.

RSocketServ ss; RConnection conn; RSubConnection subconn; RSocket sock; TRequestStatus status; // Connect to ESOCK ss.Connect(); // Open an Connection conn.Open(ss, KAfInet); // Start the connection conn.Start(status); User::WaitForRequest(status); // Attach to the default sub-connection subconn.Open(ss, RSubConnection::EAttachToDefault, conn); // Set Properties of the default sub-connection subconn.SetParameters(…); // Open a TCP socket on the connection (this is the same as using the default sub-connection) sock.Open(ss, KAfInet, KSockStream, KProtocolInetTcp, conn); _LIT(KRasAddr,"10.159.24.13"); const TInt KEchoPort = 7; TInetAddr destAddr; destAddr.Input(KRasAddr); destAddr.SetPort(KEchoPort); // Request the Socket to connect to the destination over the default sub-connection sock.Connect(destAddr, status);

Note: Error handling is not included to aid clarity.

## Creating a new sub-connection - Socket connected over the sub-connection

The following example shows how an application can use a sub-connection explicitly via an RSubConnection instance, and tie an RSocket to the sub-connection that has had its properties set. The socket is connected over the sub-connection in this case (if the underlying technology allows this).

RSocketServ ss; RConnection conn; RSubConnection subconn; RSocket sock; TRequestStatus status; // Connect to ESOCK ss.Connect(); // Open an Connection conn.Open(ss, KAfInet); // Start the connection conn.Start(status); User::WaitForRequest(status); // Create a new sub-connection subconn.Open(ss, RSubConnection::ECreateNew, conn); // Set Properties of the sub-connection subconn.SetParameters(…); // Open a TCP socket on the sub-connection sock.Open(ss, KAfInet, KSockStream, KProtocolInetTcp, subconn); _LIT(KRasAddr,"10.159.24.13"); const TInt KEchoPort = 7; TInetAddr destAddr; destAddr.Input(KRasAddr); destAddr.SetPort(KEchoPort); // Request the Socket to connect to the destination over the sub-connection sock.Connect(destAddr, status);

Note: Error handling is not included to aid clarity.

## Creating a new sub-connection - Adding an already connected socket

The following example shows how an application can use a sub-connection explicitly via an RSubConnection instance, and tie an already connected RSocket to the sub-connection that has had its properties set (parameter creation is described in separate section):

RSocketServ ss; RConnection conn; RSubConnection subconn; RSocket sock; TRequestStatus status; // Connect to ESOCK ss.Connect(); // Open an Connection conn.Open(ss, KAfInet); // Start the connection conn.Start(status); User::WaitForRequest(status); // Open a TCP socket on the connection sock.Open(ss, KAfInet, KSockStream, KProtocolInetTcp, conn); _LIT(KRasAddr,"10.159.24.13"); const TInt KEchoPort = 7; TInetAddr destAddr; destAddr.Input(KRasAddr); destAddr.SetPort(KEchoPort); // Connect the Socket to the destination over the connection (default sub-connection) sock.Connect(destAddr, status); // Create a new sub-connection subconn.Open(ss, RSubConnection::ECreateNew, conn); // Set Properties of the sub-connection subconn.SetParameters(…); // Move the connected socket onto the new sub-connection TRequestStatus status; subconn.Add(sock, status); // Wait for socket to added User::WaitForRequest(status);

Note: Error handling is not included to aid clarity.

## Creating and setting properties for a Sub-Connection

The following example shows how an application can create Quality of Service properties and assign them to a sub-connection:

// Create the container for all sub connection parameters RSubConParameterBundle subconParams; CleanupClosePushL(subconParams); // Create a container for QoS sub connection parameters (Param bundle takes ownership) CSubConParameterFamily* qosFamily = CSubConParameterFamily::NewL(subconParams, KSubConQoSFamily); // Create the requested generic parameter set for QoS (Qos family takes ownership) CSubConQosGenericParamSet* reqGenericParams = CSubConQosGenericParamSet::NewL(*qosFamily, CSubConParameterFamily::ERequested); // Set the requested Generic Parameters reqGenericParams->SetDownlinkBandwidth(128); reqGenericParams->SetUplinkBandwidth(64); // Create the acceptable generic parameter set for QoS (Qos family takes ownership) CSubConQosGenericParamSet* accGenericParams = CSubConQosGenericParamSet::NewL(*qosFamily, CSubConParameterFamily::EAcceptable); // Set the acceptable Generic Parameters accGenericParams->SetDownlinkBandwidth(48); accGenericParams->SetUplinkBandwidth(32); // Create a requested technology specific parameter set for QoS (Qos family takes ownership) CSubConQosR99ParamSet* reqRel99Params = CSubConQosR99ParamSet::NewL(*qosFamily, CSubConParameterFamily::ERequested); // Set the requested Technology Specific Params reqRel99Params->SetMaxSDUSize(1024); // Create a acceptable technology specific parameter set for QoS (Qos family takes ownership) CSubConQosR99ParamSet* accRel99Params = CSubConQosR99ParamSet::NewL(*qosFamily, CSubConParameterFamily::EAcceptable); // Set the acceptable Technology Specific Params accRel99Params->SetMaxSDUSize(512); // Now open the sub-connection as normal… ……… ……… // Create a new sub-connection subconn.Open(ss, RSubConnection::ECreateNew, conn); // Set Properties of the sub-connection subconn.SetParameters(subconParams); // Destroy parameters CleanupStack::PopAndDestroy(); // subconParams // Open a TCP socket on the sub-connection sock.Open(ss, KAfInet, KSockStream, KProtocolInetTcp, subconn); _LIT(KRasAddr,"10.159.24.13"); const TInt KEchoPort = 7; TInetAddr destAddr; destAddr.Input(KRasAddr); destAddr.SetPort(KEchoPort); // Connect the Socket to the destination over the sub-connection sock.Connect(destAddr, status); User::WaitForRequest(status); // Fetch the granted qos RSubConParameterBundle grantedParams; subconn.GetParameters(grantedParams);

Note: Error handling is not included to aid clarity.

## Registering for events – Simple case

The following example shows the simplest case of how an application can register for events occurring on a sub-connection. In this example the application registers for notification of all events.

// Create the container for all sub connection parameters RSubConParameterBundle subconParams; CleanupClosePushL(subconParams); ……… ……… // Create and initialise parameters sets as above ……… ……… // Create a new sub-connection subconn.Open(ss, RSubConnection::ECreateNew, conn); TNotificationEventBuf eventBuffer; TRequestStatus eventStatus; subconn.EventNotification(eventBuffer, EFalse, eventStatus); // Set Properties of the sub-connection subconn.SetParameters(subconParams); // Destroy parameters CleanupStack::PopAndDestroy(); // subconParams // Open and connect a TCP socket on the sub-connection sock.Open(ss, KAfInet, KSockStream, KProtocolInetTcp, subconn); sock.Connect(destAddr, status); User::WaitForRequest(status); // Negotiation may not occur until a socket is assigned to the sub-connection // First event should be cSubConGenEventDataClientJoining User::WaitForRequest(eventStatus); // Next we’d expect a CSubconGenEventParamsGranted/ CSubconGenEventParamsRejected subconn.EventNotification(eventBuffer, EFalse, eventStatus); User::WaitForRequest(eventStatus);

Note: Error handling is not included to aid clarity.

## Registering for events – Using filters

The following example shows how to register for specific events by using filters. In this example the application registers for notification when sub-connection parameters have been granted or rejected. Each TEventFilter contains the factory Uid of the events and a mask of event Ids bitwise OR’d together.

// Create the container for all sub connection parameters RSubConParameterBundle subconParams; CleanupClosePushL(subconParams); ……… ……… // Create and initialise parameters sets as above ……… ……… // Create a new sub-connection subconn.Open(ss, RSubConnection::ECreateNew, conn); // Create event filter TEventFilter filter; filter.iEventGroupUid = KSubConnGenericEventsImplUid; filter.iEventMask = KSubConGenericEventParamsRejected ¦ KSubConGenericEventParamsGranted; // Register for event TNotificationEventBuf eventBuffer; TRequestStatus eventStatus; subconn.EventNotification(eventBuffer, &filter, 1, eventStatus); // Set Properties of the sub-connection subconn.SetParameters(subconParams); // Destroy parameters CleanupStack::PopAndDestroy(); // subconParams // Open and connect a TCP socket on the sub-connection sock.Open(ss, KAfInet, KSockStream, KProtocolInetTcp, subconn); sock.Connect(destAddr, status); User::WaitForRequest(status); // Event should be CSubconGenEventParamsGranted/CSubconGenEventParamsRejected User::WaitForRequest(eventStatus);

Note: Error handling is not included to aid clarity.

## Extracting information from received events

The following example shows how to extract the information contained within an event notification once it has been received.

// Create the container for all sub connection parameters RSubConParameterBundle subconParams; CleanupClosePushL(subconParams); ……… ……… // Create and initialise parameters sets as above ……… ……… // Create a new sub-connection subconn.Open(ss, RSubConnection::ECreateNew, conn); // Create filter, register for events, and set parameters as above …… subconn.EventNotification(eventBuffer, &filter, 1, eventStatus); …… // Open and connect a TCP socket on the sub-connection …… // Receive the event notification User::WaitForRequest(eventStatus); CSubConNotificationEvent* event; event = CSubConNotificationEvent::NewL(eventBuffer); CleanupStack::PushL (event); if (event->GroupId() == KSubConnGenericEventsImplUid && event->Id() == CSubConGenEventParamsRejected) { CSubConGenEventParamsRejected* rejectedEvent = static_cast< CSubConGenEventParamsRejected*>(event); TInt error = rejectedEvent->Error(); …… // Do something with the error …… } CleanupStack::PopAndDestroy (event);

Note: Error handling is not included to aid clarity.

## Working Code Examples

# Further Information

## Glossary

| Term | Meaning |
| --- | --- |
| APN | Access Point Name. |
| CommDB | Symbian OS Communications Database |
| Connection | Synonymous with “Interface” |
| DNS | Domain Name System |
| Explicit connection | A connection which is started explicitly by the use of RConnection API. See also “Implicit connection”. |
| GPRS | General Packet Radio Service |
| IAP | Internet Access Point |
| Implicit connection | A connection which is started implicitly via an RSocket or RHostResolver request. See also “Explicit connection”. |
| Interface | Provides a connection to an external network. Synonymous with “connection”. |
| IPv4 | Internet Protocol Version 4 |
| IPv6 | Internet Protocol Version 6 |
| NIF | Symbian OS Network Interface |
| PDP | Packet Data Protocol |
| QoS | Quality of Service |
| Sub-session | Within this document, an RSocket, RHostResolver or RConnection object. |
| TCP/IP | Transmission Control Protocol / Internet Protocol |
| TFT | Traffic Flow Template |

## People

| Role | Person / People |
| --- | --- |
| Reviewers | Richard Collin, Viki Turner, Steve Butler, Dino Livanos |
| Contributors | Richard Collin, Nadeem Wahid, Barry Alcock |
| Distribution | Networking TA, Anyone using or maintaining the RSubConnection API. |

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [R1] | N/A | v2.0 | RConnection API |
| [R2] | N/A | v1.2 | RConnection API for Connection Management |
| [R3] | 3GPP TS 23.107 | v4.6.0 | Quality of Service (QoS) concept and architecture. (Release 4/99) |
| [R4] | 3GPP TS 23.107 | v5.13.0 | Quality of Service (QoS) concept and architecture. (Release 5) |
| [R5] | 3GPP TS 23.207 | v5.10.0 | End-to-end Quality of Service (QoS) concept and architecture (Release 5) |
| [R6] | 3GPP TS 23.228 | v5.13.0 | IP Multimedia Subsystem (IMS) (Release 5) |

## Open Issues

- Information needed to describe how SBLP and authorisation classes work.

- This document will need further updates as part of GT280, PREQ201.

## Document History

| Date | Version | Status | Author | Description |
| --- | --- | --- | --- | --- |
| 29-07-2004 | 0.1 | Draft | Richard Collin | New Document |
| 10-08-2004 | 0.2 | Draft | Richard Collin | Updated to reflect new API changes |
| 14-08-2004 | 0.3 | Draft | Richard Collin | Updated after comments from QoS GT205 team |
| 10-09-2004 | 0.4 | Draft | Richard Collin | Added parameter structure |
| 18-10-2005 | 0.5 | Draft | Barry Alcock | Significant updates for currently available classes. Added more examples/descriptions. |
| 24-10-2005 | 0.6 | Draft | Barry Alcock | Post review changes. Added test apps. |
| 07-11-2005 | 1.0 | Issued | Barry Alcock |  |