Contents

1	Introduction	3

1.1	Purpose and Scope	3

2	API Overview	4

2.1	Registering support for DMA with the Local Media Subsystem	4

2.2	Querying the availability of Physical Memory	4

2.3	Retrieving Physical Addresses	5

3	Detailed Break down of the Migration Tasks	6

3.1	Task 1 - Register support with the Local Media Subsystem	6

3.2	Task 2 – Enable Read support for Physical Addresses	6

3.2.1	Detecting the use of physical memory	8

3.2.2	Transferring data directly to Physical Memory	11

3.2.3	Eliminate Inter-Process Copies	11

3.3	Task 3 – Enable Write support for Physical Addresses	12

4	Implementation Considerations	12

4.1	Media Addressing Scheme	12

4.2	Scatter/Gather DMA Controllers	13

5	Testing	17

6	Further Information	18

6.1	References	18

6.2	Open Issues	18

6.3	Glossary	18

# Introduction

To facilitate the use of a DMA controller within media drivers, the driver must transfer data via an intermediate DMA-safe physical buffer, which it uses as the source/destination for data transfers before performing an Inter-Process-Copy from/to the user-side process.  For standard File I/O requests this is required as the memory provided by a user processes will nominally be virtual mapped memory. However for functionality such as File Caching and Demand Paging (which already make direct use of Physical memory) this introduces an unnecessary over head.

The Local Media Subsystem has been modified such that where physical memory objects (e.g. Shared Chunks[R1], File Server Cache & Demand Paging) are used to access media, the physical memory address is made available.

By making the physical address available, a media driver can access the physical memory directly (using DMA or Mem::Copy) rather than copying through use of an intermediate physical memory buffer.  By removing the overhead of additional Inter-Process-Copy calls, an increase in throughput and a reduction of CPU utilisation can be achieved for functionality that makes use of physical memory objects.

## Purpose and Scope

This document describes the recommended changes to media drivers to support direct memory addressing.

This document assumes that the reader is familiar with Symbian OS device driver model, Base Port layering and associated components.

# API Overview

To facilitate the use of physical memory within a media driver, various new API’s have been provided which shall be described before discussing the porting process in more detail:

## Registering support for DMA with the Local Media Subsystem

The LocDrv::RegisterDmaDevice(…)API is used by the media driver to inform the Local Media Subsystem that the media driver is capable of Direct Memory Addressing, and its requirements for physical memory:

/**

Registers a media device with physical memory addressing capabilities with the

Local Media Subsystem.

@param aPrimaryMedia      A pointer to the primary DPrimaryMedia object associated

with the media device.

@param aMediaBlockSize    The Minimum transfer size (bytes) for the media device.

@param aDmaMaxAddressable The Maximum Addressing Range for the media device's DMA controller,

0 if None.

@param aDmaAlignment      The required memory alignment for the media device's DMA controller.

e.g. word alignment required = 2

@return KErrNone, Always;

*/

EXPORT_C TInt LocDrv::RegisterDmaDevice(	DPrimaryMediaBase* aPrimaryMedia,

TInt aMediaBlockSize,											TInt aDmaMaxAddressable,

TInt aDmaAlignment)

Once registered, the Local Media Subsystem will recognise requests using physical memory, and extract the necessary information required by the driver for read/write requests. This information is made available through the TLocDrvRequest object [sections 2.2, 2.3].

The Local Media Subsystem makes use of the parameters provided by the driver via LocDrv::RegisterDmaDevice(…)to determine if physical memory is valid for the media driver.

- aMediaBlockSize - The minimum number of bytes that can be transferred by the media device (for example, SD memory cards support block addressing so the minimum granularity is 512 bytes). For requests where the transfer is less than aMediaBlockSize the Local Media Subsystem will not enable direct memory access.  This is done to prevent media drivers accidentally accessing memory outside of the bounds of the request.

- aDmaMaxAddressable – The maximum number of bytes that can be transferred in one DMA request. If the total size of a request requires exceeds this value, the Local Media Subsystem will issue multiple requests so not to exceed the maximum transfer size.

- aDmaAlignment – The required DMA controller memory alignment. For example if the DMA controller uses a 16 bit addressing scheme then it requires 2 byte or word alignment. The Local Media Subsystem will not support direct memory addressing for requests that are not correctly aligned to this value.

## Querying the availability of Physical Memory

The API TLocDrvRequest::IsPhysicalAddress() is provided to allow the media driver to determine if a physical address is available for the current request:

/**

Returns ETrue if Physical memory is available for this request

*/

inline TBool TLocDrvRequest::IsPhysicalAddress()

This API should only be called in response to requests with ID’s of DLocalDrive::ERead or DLocalDrive::EWrite.  For all other request types EFalse shall be returned.

## Retrieving Physical Addresses

The TLocDrvRequest::GetNextPhysicalAddress(…) API allows the media driver to retrieve the physical addresses and lengths for the physically contiguous fragments making up the current request:

/**

* Returns Address and Length of next contiguous Physical memory fragment

*

* @param aAddr On success, populated with the Physical Address of the next fragment.

* @param aLen  On success, populated with the length in bytes of the next fragment.

*

* @return 	KErrNone, if successful;

* 		KErrNoMemory, if no more memory fragments left.

* 		KErrNotSupported, if Physical Memory addressing is not supported by this Media.

*/

EXPORT_C TInt TLocDrvRequest::GetNextPhysicalAddress(TPhysAddr& aAddr, TInt& aLen)

The physical address returned via aAddr always represents a contiguous physical block (or fragment) of physical memory.  If the length of this fragment is less than the total length of the request then it can be assumed that the underlying memory is fragmented.  Once the fragment has been completed, this API must be called again to retrieve the next fragment until the total transfer is complete.

The underlying physical memory can only be in 1 of 2 states:

- Contiguous – Physical memory is allocated in one continuous block

Virtual Memory

Physical Memory

- Fragmented – Physical memory is allocated in multiple contiguous blocks (fragments) that may not be located sequentially.

Virtual Memory

Physical

Fragment #2

Unused

Unused

Unused

Physical

Fragment #3

Physical

Fragment #1

Physical

Fragment #4

# Detailed Break down of the Migration Tasks

To enable the use of Direct Memory Addressing, the media must be modified in the following manner:

- Register support for Physical Addressing with the Local Media Subsystem

- Enable Read support for Physical Addresses

- Enable Write support for Physical Addresses

## Task 1 - Register support with the Local Media Subsystem

As described in section 2.1 the media driver must register itself as capable of handling physical addresses.

Find the function that contains the call to LocDrv::RegisterMediaDevice(…).  This will usually be found in the extension entry point (DECLARE_STANDARD_EXTENSION) of your driver.  After the driver has been successfully registered, add a call to LocDrv::RegisterDmaDevice(…), specifying the capabilities of your driver.

The following code shows an example use of LocDrv::RegisterDmaDevice(…)for the MMC controller:

EXPORT_C TInt TMMCardControllerInterface::Create()

{

…

TInt err = LocDrv::RegisterMediaDevice(mdi.iDevice,

drivesPerSlot,

mdi.iDriveList+driveListIndex,

pMedia,

numMediaPerSlot,

*mdi.iDeviceName);

if (err != KErrNone)

return err;

err = LocDrv::RegisterDmaDevice(pMedia,

KMMCardHighCapBlockSize, // Block Addressing 512 Bytes

MaxDataTransferLength(), // 1024 Byte address range

DmaAlignment() );        // 2 Bytes - 16 bit register

if (err != KErrNone)

return err;

…

}

This example informs the local media subsystem that the MMC controller requires Physical memory to be Word aligned and cannot address more that 1MB in any given request.  It also notes that for this particular implementation the minimum transfer size for the media is 512 Bytes.

These values should be obtained from the documentation for the platform on which the media driver to be used.  If this method is to be called from a generic (platform-independent) layer of the driver, then you should arrange for these values to be reported from the variant’s Platform Specific Layer

Before moving onto the next task, run your regression tests to ensure that the media driver is still operating correctly. Although the Local Media subsystem is now processing physical addresses, the media driver read/write functionality has not yet been modified so operation should not be affected.

## Task 2 – Enable Read support for Physical Addresses

The modification of Read functionality is broken down further into 3 stages:

- Detecting the use of physical memory

- Transferring data directly to Physical Memory

- Eliminate Inter-Process Copies

The following sections provide a simplified overview of the migration process for read handling and do not take into account any specific media driver specific functionality (you will need to consider the specific impacts on your media driver before beginning implementation). Section 4 contains some additional topics for implementation consideration.

### Detecting the use of physical memory

Locate the functionality or method that handles Read Requests (DLocalDrive::ERead) within the media driver and where appropriate make a call to the IsPhysicalAddress()API in order to provide the new data path for direct memory access.

..<snip>..

iMediaStartPos = iCurrentReq->Pos();

iTotalLength   = I64LOW(iCurrentReq->Length());

iDoPhysicalAddress = iCurrentReq->IsPhysicalAddress();

if(iDoPhysicalAddress)

{

..< Physical memory specific code >..

}

else

{

..< Existing Code >..

### Transferring data directly to Physical Memory

Within the new code path the physical address and length must be retrieved and used to issue the read command.

TPhysAddr physAddr;

TInt physLength;

TInt err = KErrNone;

err = iCurrentReq->GetNextPhysicalAddress(physAddr, physLength);

if(err == KErrNone)

{

if (physLength < iTotalLength)

{

// Memory is fragmented, note remainder for next iteration

iRemaining    = iTotalLength – physLength;

iTotalLength -= physLength;

}

// Start data transfer into the current fragment
      DoDataTransfer(iMediaPos, physLength, physAddr);

}

In this example, the length of the current fragment is compared against the total request length.  If less than the total length then it can be determined that the physical memory is fragmented and another iteration of the functionality will be required.

However, should you not wish to support fragmented memory requests, then this comparison can also be used to revert back to the original read handling code.

Your media driver may perform a Linear to Physical memory address translation, and if so must be modified so that this it is not performed for physical memory requests as shown below.

…

TPhysAddr dest;

if (iCurrentReq->IsPhysicalAddress())

dest = (TPhysAddr) aDes.iDest;

else

dest = Epoc::LinearToPhysical(aDes.iDest);

### Eliminate Inter-Process Copies

Finally, read request handling must be modified to eliminate any unnecessary calls to WriteRemote(…) as we have already transferred our requested data directly to the appropriate physical address.

if (!iCurrentReq->IsPhysicalAddress())

{

if( (id == DMediaPagingDevice::ERomPageInRequest)||

(id == DMediaPagingDevice::ECodePageInRequest) )

{

r = iCurrentReq->WriteToPageHandler((TUint8 *)(& iBufPtr [0]), len, usrOfst);

}

else if(id==DLocalDrive::ERead)

{

r = iCurrentReq->WriteRemote(&iBufPtr,usrOfst);

}

}

The media driver should now be capable of handling physical memory requests for read operations.  Before moving onto the next task, run your regression tests to ensure that the media driver operates correctly [see section 5].

## Task 3 – Enable Write support for Physical Addresses

Enabling write support for Physical Addressing follows the same principles as described in the previous task so shall not be described in more detail here.

# Implementation Considerations

## Media Addressing Scheme

Before you begin migration of your media driver, consideration must be given to the addressing scheme used by the hardware.

For example, MMC cards have a block based addressing scheme where all media access is performed in a multiple of 512 bytes.  For example:

- Read 1 byte - 512 Bytes must be read and the single byte extracted.

- Write 1 Byte - 512 Bytes must be read, the single byte modified then the whole 512 Bytes written back to the media (read/modify/write).

With transfers less than specified minimum transfer size the local media subsystem will not make a physical address available as it is considered dangerous to allow access to the surrounding memory.

However transfers will occur if they are greater then minimum transfer size but not media block aligned (i.e. do not start/end on a multiple of the block size).  It is the responsibility of the media driver to handle this situation, and the following example looks at a possible method for handling this issue.

Consider a read request of 1024 bytes that begins at address 256:

2048

1024

512

0

1536

Start 256

End 1280

1024

The media driver can handle this in the following manner:

- The first 256 bytes cannot be read directly to physical memory as it does not start on a multiple of 512 bytes (and doing so would corrupt the 256 bytes of memory preceding the start of the physical buffer).  Therefore the whole of the first block must be read (address 0-511) must be read into an intermediate buffer before copying out the required 256 bytes to physical memory.

- Similarly, the final 256 bytes cannot read directly to physical memory (as doing to would corrupt the 256 bytes of memory following on from the end of the physical buffer), so again the last block (1024-1535) must be read into an intermediate buffer before copying out the required 256 bytes  physical memory.

- The middle 512 bytes are media aligned so can be read directly into physical memory without the need for an intermediate buffer.

The same considerations must also be made for writing.

## Scatter/Gather DMA Controllers

Some DMA controllers can support the Scatter/Gather mode of operation (sometimes referred to as Advanced DMA).  Such controllers can be programmed with multiple physical addresses and lengths per transfer request.  If your DMA controller does support this mode of operation, then the GetNextPhysicalAddress(…)can be called multiple times to populate the scatter-gather list in advance.  The following example code shows how this can be achieved (note that for clarity this example assumes an infinite scatter-gather list is supported by the DMA controller):

TPhysAddr physAddr;

TInt physLength;

TInt err = KErrNone;

while (iRemaining > 0)
      {

err = iCurrentReq->GetNextPhysicalAddress(physAddr, physLength);

if(err != KErrNone)

return err;

iRemaining -= physLength;

PopulateScatterGatherList(physAddr, physLength);

}

return DoDataTransfer(pos, length);

# Testing

The standard E32 and F32 automated tests should be run to verify the basic operation of the media driver.  It is also recommended that any media driver specific unit tests should also be run (if available) to exercise media specific functionality.

To improve code coverage, your media driver specific tests should be extended to consider the following test cases:

- Reads and writes greater than the maximum DMA addressable range.  For example if the maximum addressable range is 128KBytes, transfers should be made using 192Kbytes, 256Kbytes etc...

- Reads and writes to misaligned physical memory.  For example, if the DMA controller requires memory to be Word aligned (16 bits) issue transfers with physical memory starting at an odd address.

- Reads and writes to misaligned media addresses.  For example, if your hardware supports a minimum addressable boundary of 512 bytes, issue transfers starting at media offsets within this boundary.

For performance measurements, T_FSYSBM should be executed with and without DMA enabled; the following table lists the test arguments required for the different modes of operation.

| File Caching | +r +f +w |
| --- | --- |
| DChunk (Shared Memory – Contiguous) | -r –w –f +s |
| DChunk (Shared Memory – Dis-Contiguous) | -r –w –f +x |

It is recommended that the same Media and Hardware platform is used for all performance measurements such that any resulting measured difference is only due to software changes.

# Further Information

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
|  | Base_How_To_Shared_Chunks.doc |  | Details use of Shared Chunk memory objects. |

## Open Issues

## Glossary

| Term | Definition |
| --- | --- |