Contents

1	Introduction	3

1.1	Overview	3

2	Current mode of operation	4

2.1	Interrupts	4

2.2	Kernel Side Threads	4

2.3	SMP Enhanced	4

3	Main Changes for the SMP Kernel	5

3.1	Atomic Operations	5

3.2	Read-Write locks around shared data access	8

3.2.1	Spin Locks	8

3.2.2	Spin Lock Macros	9

3.2.3	Spin Lock TOrder	10

3.3	Memory Barriers	11

3.4	DFC Queues	13

3.4.1	Dedicated DFC Queue	13

3.4.2	DFC Queue Re-use	13

3.4.3	DFC Signalling	13

3.5	Obsolete Scheduling Patterns	14

3.5.1	Use of NKern::Lock	14

3.5.2	Use of Interrupt Disabling	14

3.6	Interrupt Handling	14

3.6.1	Empty Interrupts	14

3.6.2	Disappearing Interrupts	15

3.7	Hardware Accessor Methods	16

3.7.1	Register Access	16

3.7.2	Register Access Ordering	16

3.8	Thread Affinity	17

3.9	Context Awareness	18

3.10	Device Driver Shut Down	18

3.11	Finalising a Device Driver	18

3.12	Revisited: Spinlocks	19

3.12.1	The spinlock API	19

3.12.2	Core Deadlocking	21

3.12.3	More deadlocks and wasting time with spinlocks.	23

4	Further Information	25

4.1	References	25

Appendix A - Additional Information	26

A.1	Old Atomic APIs	26

A.2	TSpinLock	27

A.3	TRWSpinLock	29

Appendix B - Case Studies	30

B.1	Btrace	30

B.2	Sound Driver (Shared Chunk)	30

B.3	MMC Driver	31

B.4	I2S Driver	31

B.5	PCI Driver	31

B.6	LCD Driver	32

B.7	Serial Driver	32

B.8	Camera LDD	33

B.9	USB Host Stack	33

B.9.1	Host Driver for Reference Hardware	33

B.9.2	OTGDI	33

B.9.3	USB Host stack	34

B.9.4	T_USBDI tests	34

B.10	DMA Driver	35

B.11	Ethernet	35

B.12	GPIO	35

B.13	Power Resource Manager (PRM)	36

B.14	GCE	36

B.15	Watch Dog Timer	36

B.16	Local Media Subsystem	36

B.17	Virtual Serial Keyboard	36

B.18	Key Pad	36

B.19	Digitizer	37

B.20	NAND	37

B.21	Internal RAM Driver	40

# Introduction

This document is intended to highlight some of the areas of device driver code which will need review and rework to ensure they function correctly on an SMP system, it is intended to stimulate the developer into thinking about concurrency in their code so they may better analyse potential failure cases.

This document will be updated throughout the SMP programme as development progresses.

## Overview

This document describes the changes necessary in order to make the Symbian provided device drivers ‘SMP functional’. It is intended that all Symbian provided device drivers be made ‘SMP functional’ from the first instance (i.e. no need to lock driver threads and ISRs to a single CPU) and there is to be no set of changes which simply make drivers ‘SMP safe’.

This document looks in detail at SMP related problem areas for device drivers.

One issue is likely to affect any driver that uses interrupts. This relates to the mechanisms that such drivers currently use to synchronise access to shared data or h/w between code executing in ISR and DFC context. The most common scheme used is the disabling of interrupts at the critical points within the DFC. To effectively use a multi-core device, drivers must be free to execute on a different CPU from their associated ISR. Disabling interrupts on a multi-core system will not provide the driver with exclusive access to device memory as the interrupt will only be disabled on one CPU. Hence an alternative synchronization mechanism must be used instead such as use of a spin-lock. Note that we are talking about disabling interrupts at the CPU level here. Synchronization by disabling specific interrupts in the h/w device itself may still be used on an SMP system with appropriate use of memory barriers.

An alternative scheme used by drivers for ISR/thread synchronization on uni processor systems is to use the NKern:: atomic operations within a DFC for safe access to shared memory. This scheme still appropriate to use for SMP although the NKern:: atomic operations should be replaced with the new atomic operations (in e32atomics.h) added for SMP. Note that the __e32_atomic_* operations (and the NKern:: atomic operations they replace) are only suitable for use on normal memory, not on I/O addresses.

Locks are already required around any data that is shared between driver threads. However with SMP these threads can run in parallel so any omissions in locking may be exposed where it has remained undetected on a single core platform. All drivers that involve multiple threads need to be reviewed to ensure locks are used in code that accesses shared data or h/w. Drivers with potential problems in this areas include any which service power handler events directly in the event callback functions or drivers which queue DFCs on timer related threads (e.g. use of TTickLink). Fundamentally any driver which cannot be absolutely certain of the context in which one of its callbacks will be executed is considered a problem area.

Similarly, drivers that make assumptions about thread priority ordering will almost certainly not operate correctly for SMP. For example, a driver which queues a DFC on another DFC thread of known high priority and relies on this running to completion without any explicit synchronisation would be a problem. Typically, any such code should be changed to communicate using a fast semaphore or the critical areas in the two threads concerned changed to make use of a lock etc.

Certain drivers may be using NKern::Lock() as their locking mechanism on access to shared data or hardware between DFCs. On single-core platforms this prevents the currently running thread from being pre-empted by another thread, DFC or IDFC. However the kernel lock should not be used in this way on SMP platforms and an alternative locking scheme should be used such as a fast mutex. (However it may still be valid to acquire the kernel lock where this is a pre-condition for using other kernel services).

Another problem area concerns drivers which use shared memory such as shared chunks or user accessible h/w chunks in order to communicate between threads. On SMP platforms, write and read ordering is not guaranteed between CPUs, meaning that any communication between threads or processes requires explicit synchronisation using barriers. ‘Official’ IPC mechanisms (e.g. signalling a thread using a request semaphore or fast semaphore) will make appropriate use of barriers but any drivers which use shared memory and don’t use such ‘official’ IPC mechanisms must be changed to make use of memory barriers or completion barriers as appropriate (or modified to use one of the ‘official’ IPC mechanisms).

# Current mode of operation

## Interrupts

All interrupts are currently tied to CPU0. The interrupts handling subsystem as we shall see is quite different. The nanokernel now provides an API and data structures encapsulating an ARM Generic Interrupt Controller [1]. The GIC is a superset of that present in the Arm11MPCore. In the current implementation, the GIC is initialised so that all interrupts target CPU0. This is done by NKern::Init0 which is called just after the bootstrap has completed.

## Kernel Side Threads

As with interrupts all kernel side threads are currently tied to the CPU0. During thread creation, if the thread creation information block indicates that the thread is user side, the affinity is set to allow scheduling on any CPU.  Otherwise, it is set to target CPU0. This change, along with the interrupt CPU0 targeting is a temporary measure to protect device drivers from concurrent access. This simplifies base port migration, especially at a time when the SMP kernel is itself under development.

It should be noted however that client side threads are not tied to any CPU, therefore if a device driver executes any of its code in a client thread context, that code could potentially be executed on any CPU and possibly the code might be executed on multiple CPUs simultaneously.

## SMP Enhanced

In the future the drivers provided by Symbian will be enhanced for SMP, the forced affinity of interrupts and kernel side threads will be removed [section 3.10]. It is worth noting that even in the early versions of the kernel it is possible to dynamically change which CPU a thread is running on using the thread affinity APIs [section 3.8].

# Main Changes for the SMP Kernel

## Atomic Operations

A wide variety of atomic operations are now provided in both in euser.dll and in the kernel. These replace the old atomic operations functions provided in the NKern and User classes. The list of replaced functions is provided in Appendix A.1, the old APIs are still provided for source compatibility purposes. However old versions are implemented in terms of the new atomic APIs.

The new functions can be found in e32atomics.h. The functions here take the following format:

__e32_atomic_xxx_yyyZ

Z denotes the type size and can be 8, 16, 32, 64 or _ptr to denote operations on pointers.

xxx specifies the operation performed. Available operations are:

load	read memory atomically

store	write memory atomically

swp	write to a memory location and return the original value of the

memory location

add	add a value to a memory location and return the original value

of the memory location

and	bitwise AND a value with a memory location and return the

original value of the memory location

ior	bitwise OR a value with a memory location and return the

original value of the memory location

xor	bitwise XOR a value with a memory location and return the

original value of the memory location

axo	atomic AND followed by XOR. Function takes three parameters,

address p, value u and value v and atomically does the following:

{ orig_v = *p; *p = (orig_v & u) ^ v; } return orig_v;

cas	Compare and Swap. Function takes the following three parameters

address  aAddress, address of an expected value aExpected  and a value

aValue. It then does the following:

orig_v = *aAddress;

if (orig_v == *aExpected) {*aAddress = aValue; return ETrue}

else {*aExpected = orig_v; return EFalse}

if the value of a memory location matches a specified expected

value, write a specified new value and return ETrue, otherwise

update the expected value with the actual value seen and return

EFalse

tau	Threshold and add (unsigned). This one takes four parameters:

aAddress, aThreshold, aValue1, aValue2

It proceeds as follows:

orig_v = *aAddress;

if (TUint(orig_v) >= TUint(aThreshold) *aAddress += aValue1

else*aAddress += aValue2;

return orig_v;

if the value of a memory location is >= a specified threshold,

considered as an unsigned integer, add a specified value to it

otherwise add a different specified value to it; return the

original value of the memory location

tas	Threshold and add (signed).This proceeds in the same way as tau, but here the comparison considers the memory location value and threshold to be signed integers

yyy denotes the type of memory ordering enforced (with memory barriers) when the memory is accessed. There are four types:

rlx 	relaxed memory ordering there is no guarantee on the order in which the

atomic operation is observed relative to preceding or following memory accesses

acq 	acquire semantics, the atomic operation is guaranteed to be observed before any

following memory accesses. In other words, a memory barrier follows the operation.

rel 	release semantics, the atomic operation is guaranteed to be observed after any

preceding memory accesses. In other words, a memory barrier is placed before the

operation.

ord 	fully ordered the atomic operation is guaranteed to be observed after any preceding

memory accesses and before any following memory accesses. Memory barriers

before and after the operation

The memory barriers used equate to arm dmb instructions. Here is an example of the code generated for a __e32_atomic_tau_ord32() in the ARM11MPCore:

extern "C" EXPORT_C __e32_atomic_tau_ord32(volatile TAny* /*a*/, __TYPE__ /*t*/, __TYPE__ /*u*/, __TYPE__ /*v*/)

{

// R0=a, R1=t, R2=u, R3=v

// return value in R0

asm("mov r12, #0");

asm("mcr p15, 0, r12, c7, c10, 5 ");		// data memory barrier

asm("stmfd sp!, {r4-r5} ");			// save some register on the stack

asm("mov r12, r0 ");

asm("1: ");					// in exclusive monitor

asm("ldrex r0, [r12] ")			//  orig=*a

asm("cmp r0, r1 ");				//  orig > t

asm("addcs r4, r0, r2 ");			// yes orig += u

asm("addcc r4, r0, r3 ");			// no  orig += v

asm("strex r5, r4, [r12] ");			// *p = orig

asm("cmp r5, #0 ");				// storage succed?

asm("bne 1b ");				// no so try again

asm("mov r5, #0");

asm("mcr p5, 0, r12, c7, c10, 5 ");		// data memory barrier

asm("ldmfd sp!, {r4-r5} ");			// restore regs

asm("mov pc,lr");				// return with orig in R0

}

The atomic operation functions are where possible implemented in terms of load and store exclusive instructions. Therefore they can only be used in normal memory regions. For sizes of 32bit and below the only exceptions to this are the load and store (__e32_atomic_load_yyyZ and __e32_atomic_store_yyyZ). __e32_atomic_load_yyy64 does use LDREXD and STREXD if available and therefore should only be used in read/write memory regions.

Note that for reading, writing and modifying hardware registers, it remains the recommendation to use the Hardware Accessor Methods, as described in section 3.7.

## Read-Write locks around shared data access

A Multiprocessor system allows the possibility of concurrent threads simultaneously accessing shared memory which can in turn lead to memory in a corrupt or unknown state. In order to provide mutual exclusion among processors, spin lock functionality has been introduced.

### Spin Locks

In simple terms, a spin lock is a mutex where the thread simply waits in a loop ("spins"), repeatedly checking until the mutex becomes available. As the thread remains active but isn't performing a useful task, the use of such a lock is a kind of busy waiting.  Such a spin lock can be locked, unlocked and flashed.  Flashing can be used to give other waiting spin locks a chance to take the lock, before proceeding with more code needing the lock.  This is faster than the equivalent unlock/lock combination that it represents, as it can skip the operations if nothing is waiting for the lock.  It will also unlock a reschedule or IRQ on the current processor, to avoid needlessly blocking something, while this thread isn’t running.

As spin locks are a form of busy waiting, they really shouldn't be thought of as a replacement, or the equivalent of a mutex.  Mutexes don't waste any time spinning needlessly, and ensures the CPU keeps doing useful work.   A spin lock becomes useful when you cannot block (i.e., when in interrupt context), or/and when you are never going to have to wait more than a very short time (such as setting a few registers), and as such a mutex would be too slow (or at least, very wasteful of time). Commonly, a spin lock would be used in place of locking the kernel or disabling pre-emption by disabling global interrupts on a single core kernel.  One should be wary when calling unfamiliar subroutines inside a spinlock (as when locking the kernel or disabling interrupts), as it may be lengthy in execution, or It might also acquire other spin locks (or even mutexes) and thereby violate the lock ordering rules.

The Spin lock API has been provided in the form of a class called TSpinLock and can be found in nkern.h. In 3.2.2 we will describe why you would rarely, if at all, use these directly, but until then we will just consider the macro equivalents.

Spin locks can be nested and in a similar manner to DMutexes have an order number that is checked in debug builds to prevent deadlocks. In a nested situation, a spin lock can be acquired if its order number is lower than the preceding locks.  One should never be tempted to exclude its order number, and circumvent this mechanism, unless there is no other option (This is rare.).  Mutexes and spin locks are complicated, and even if you believe you fully understand the interactions of your code, the next person that changes your code, or something that your code uses, may well not. (They may be unaware that your routine even calls theirs, as they add a spin lock.)

An additional complexity is brought about by the fact that spin locks do not protect against any interrupts on the same core.  Although there are situations where you may not need to disable interrupts (maybe you know that they are already disabled), these situations are rare, and generally you should always use the IRQ disabling methods, such as those listed below.

As a result of having interrupts disabled, it naturally follows that there are many operations that can’t be done within a spinlock, such as allocating memory.  (Although, this example would clearly not be done within a spinlock anyway, as it is a very slow and unbounded operation.) The need for interrupt disabling is explained further in 3.12.3.

Following is a list of macros and constructor, that can be used to take a spin lock, and a description of their function on an SMP kernel.  Note that the macros have been written as if they were functions, for descriptive reasons only.

| IMPORT_C TSpinLock(TUint aOrder); |  |
| --- | --- |
| Description | Constructor for TSpinLock object, needed for the following macros. |
| Parameters | aOrder – order value associated with TSpinLock (see TSpinLock::TOrder). A spin lock can only be acquired if any previously acquired locks have a higher order number. This condition is checked in UDEB builds to prevent deadlocks but should always hold true. |

| TInt __SPIN_LOCK_IRQSAVE(lock) |  |
| --- | --- |
| Description | Disable interrupts and acquire the lock. |
| Parameter: lock | The TSpinLock object to be used to aquire the lock. |
| Return | Original interrupt state. |

| __SPIN_UNLOCK_IRQRESTORE(lock, Tint irq) |  |
| --- | --- |
| Description | Release the lock, and then restore interrupt state. |
| Parameter: lock | The TSpinLock object to be used to aquire the lock. |
| Parameter: irq | Interrupt state. |

| TBool __SPIN_FLASH_IRQRESTORE(lock, Tint irq) |  |
| --- | --- |
| Description | If someone else is waiting for the lock or there is an interrupt pending, UnlockIrqRestore() then LockIrq() |
| Parameter: lock | The TSpinLock object to be used to aquire the lock. |
| Parameter: irq | Interrupt state |
| Returns | ETrue if the Unlock/Lock sequence happened, EFalse otherwise. |

A more extensive list TSpinlock methods can be found in 3.12.   These can be used for coping with more complex situations, such as where you already disable IRQs a few instructions earlier.

More details of the TSpinLock declaration, including the TOrder parameter used in construction, are included in Appendix A.2.

### Spin Lock Macros

For compatibility purposes with the non-SMP nanokernel, a set of macros is supplied which work both in the SMP and non-SMP kernel. For the latter, the macros will call the nanokernel interrupt enabling and disabling functions or compile to nothing otherwise as appropriate. The TSpinLock API should therefore never be used directly and in fact it is marked as an internal component, instead the macros should be used.

The table below shows each macro and the equivalent functions in SMP and non-SMP kernels that it equates to:

| Macro | SMP | Non-SMP |
| --- | --- | --- |
| TInt __SPIN_LOCK_IRQSAVE (TSpinLock lock) | LockIrqSave | NKern::DisableAllInterrupts |
| __SPIN_UNLOCK_IRQRESTORE (TSpinLock lock, Tint irq) | UnLockIrqRestore | NKern::RestoreInterrupts |
| TBool __SPIN_FLASH_IRQRESTORE (TSpinLock lock, Tint irq) | FlashIrqRestore | NKern:: RestoreInterrupts NKern::DisableAllInterrupts |

Example usage of the Spin Lock macros can be seen in the Btrace driver’s TBTraceBufferK::Reset function:

void TBTraceBufferK::Reset(TUint aMode)

{

#ifdef __SMP__

TSpinLock* sl = BTrace::LockPtr();

#endif

TInt irq = __SPIN_LOCK_IRQSAVE(*sl);	// guarantees handler can't run at the same time

… … …  // code to reset buffer

__SPIN_UNLOCK_IRQRESTORE(*sl,irq);

…

}

### Spin Lock TOrder

When a spinlock object is created, a TOrder value must be provided.  In a nested situation, a spin lock can be acquired if its order number is lower than the preceding locks.  This value is checked in UDEB builds and is the primary defence against deadlock, which must never be allowed to happen.  Care should be taken in choosing an order value, based on the developer’s sound judgment.   You should always provide a TOrder value, even if you don't think the function will be nested, as it will not hurt, and will protect you from future changes, and may demonstrate any errors in your logic.

Spinlocks TOrders which are used with interrupts disabled (most of them in device drivers) are shown below.  For the full list, including TOrders used with interrupts enabled (but pre-emption disabled), see A.2.

| Value | Enum | Description |
| --- | --- | --- |
| 0x0 … 0x3 | EOrderGenericIrqLow0 … EOrderGenericIrqLow3 | Device driver spin locks, low range. A device driver should pick one of these if they do not need to use method involving values 0x4 – 0x16 (eg BTrace or NTimer) If writing a BTrace handler, needing a spinlock, it would need a lock order within this range. |
| 0x4 | EOrderBTrace | BTrace lock |
| 0x7 | EOrderEventHandlerList | Per-CPU event handler list lock |
| 0xA | EOrderNIrq | NIrq lock |
| 0xD | EOrderGenericIPIList | Generic IPI list lock |
| 0x10 | EOrderNTimerQ | Nanokernel timer queue lock |
| 0x13 | EOrderExIDfcQ | Per-CPU exogenous IDFC queue list lock |
| 0x16 | EOrderIdleDFCList | Idle DFC list lock |
| 0x18 … 0x1B | EOrderGenericIrqHigh0 … EOrderGenericIrqHigh3 | Device driver spin locks, high range A device driver should pick one of these if they need to use method involving values 0x4 – 0x16 (eg BTrace or NTimer) |
| 0xFF | EOrderNone | This disables order checking and should only be used with EXTREAM CARE, when it becomes impossible to use an ordering. |

For your device driver, you will most likely want to use a EOrderGenericIrqHigh enum or less likely a EOrderGenericIrqLow enum, depending on whether you intend to use methods that involve values 0x4 – 0x16, such as BTrace.

## Memory Barriers

When a program runs on a single processor system, the hardware/Kernel ensures that a program behaves as if reads and writes are executed in order.  In reality, the core uses all sorts of tricks to speed up execution. These optimisations involve reading and writing the memory out of order, delaying some writes to memory, and using cached copies of others and then, with appropriate logic, ensures that the overall effect is the same.  However, this means that when memory is shared with CPUs in a multiprocessor system, write and read ordering between the CPUs is not guaranteed; this is referred to as Relaxed Memory Consistency. For example a second CPU may or may not see memory changes made by the first CPU in a sequence which differs from program order.

Consider the following 2 functions; each is executed on a different core:

Core#1:			Thread#1

Volatile TUint iStatus = 0;

Volatile TUint iData = 0;

...

TInt16 x = 0;

do

{

x = iStatus;

}

while (x == 0);

TInt16 f = iData;

Kern::Printf(“f1 = 0x%x”,f);

Core#2:			Thread#2

iData = 0x10;

iStatus = 0x1;

If both threads were executed on a single processor system then “f1 = 0x10” would always be printed; however this may not be the case for a multiprocessor system, if core #2's store operations are executed out-of-order, it is possible that iStatus would be updated before iData, and therefore "f1 = 0x0” might be printed; this is not acceptable.

This means that any communication between threads or processes will require explicit synchronisation using memory barriers, or official IPC mechanisms that use memory barriers internally. Memory barriers constrain the order of access and guarantee that accesses are observed in a certain order.

Two barrier functions are provided:

| __e32_memory_barrier() |  |
| --- | --- |
| Description | Ensures all preceding explicit memory accesses are observed before any following explicit memory accesses. Does not however guarantee the order of completion of the preceding memory accesses. Equates to the ARM DMB instruction |

| __e32_ io_completion_barrier() |  |
| --- | --- |
| Description | Ensures all preceding explicit memory accesses are complete before any following instructions execute. For example, it ensures that writes to I/O devices have actually occurred before execution continues. Equates to the ARM DSB instruction. |

From the example code above a memory barrier has been inserted before core #2's setting of iStatus to ensure that the new value of iData is visible to other cores at or prior to the change in the value of iStatus:

Core#2:			Thread#2

iData = 0x10;

__e32_memory_barrier();

iStatus = 0x1;

Generally __e32_memory_barrier() rather than __e32_io_completion_barrier() is used to ensure the order of accesses to normal memory. __e32_io_completion_barrier() is normally only used within the kernel, example usage includes page table manipulation.

Since iData may have been read in advance on Core#1, before iStatus was read and checked, its also necessary to insert a memory barrier after the while loop.

Core#1:			Thread#1

Volatile TUint iStatus = 0;

Volatile TUint iData = 0;

...

TInt16 x = 0;

do

{

x = iStatus;

}

while (x == 0);

__e32_memory_barrier();

TInt16 f = iData;

Kern::Printf(“f1 = 0x%x”,f);

The official Inter-Process (Thread) Communication (IPC) mechanisms (such as mutexes and spinlocks) use the memory barrier API and take care of ordering; however any code that uses non-standard IPC mechanisms are likely to be broken and will require the addition of barriers (at the very least). It is recommended that this code is evaluated to establish whether the use of non-standard IPC is essential, and:

- If it is not essential the code should be reworked to use one of the standard mechanisms and inherit the safe / portable mechanisms exposed by the kernel.

- If it is essential (because of performance characteristics or data requirements for example), the code should be evaluated for adoption as a standard mechanism to be included in the Symbian OS product line.

Special attention must be given to drivers that make use of Shared chunks or Static variables to provide signaling or data transfer between threads/processes. In some cases a combination of spin locks and memory barriers may be required to ensure data integrity, the Btrace driver analysis [section B.1] provides a good example of this issue.

## DFC Queues

### Dedicated DFC Queue

All drivers that use DFCs should make use of a dedicated DFC queue to service them, in preference to using the Kernel’s shared DFC queues (Kern::DfcQue0 and Kern::DfcQue1).  This is possible by using Kern::DfcQueueCreate for kernel extensions, or the TDynamicDfcQue class for dynamically loaded drivers.

All DLogicalChannel-derived drivers should use a dedicated DFC queue to process requests (this is set by calling DLogicalChannel::SetDfcQ).

This provides a degree of isolation between drivers, and prevents page faults in one driver’s DFC thread affecting the execution of another driver’s DFCs.

### DFC Queue Re-use

It is advisable not to run driver call-backs in a thread other than the DFC thread associated with that driver, the following example illustrates the issues with this practice:

Two device drivers are both clients of a data bus driver in order to receive status from their respective devices (for example similar structure to PCI or USB); the data bus driver makes use of the standard ISR/DFC pattern. When registering as a data bus client the drivers both provide a call-back method in order to be notified of any events received.

Within the data bus driver the DFC issues a call-back to each device driver to indicate an event (i.e. data received) has occurred, when the device driver receives its call-back, it accesses the data bus to determine if the event was for it or another driver.

This means that until the 1st call-back is complete the DFC is blocked and no further events can be reported. During this blocking additional events could occur which would be missed or in some devices the minimum response time to an event is not achieved.

Data Bus Driver

Device Driver 1

Device Driver 2

X

Call-back

Data Rx

Call-back

Data Rx

A solution to this example is for each data bus driver client to supply a DFC to be queued when the requested operation completes, this allows data bus to continue operation unblocked.

### DFC Signalling

Some device drivers that support interrupts only have 1 or 2 actual interrupt signals, therefore instead of maintaining a “status” variable to record which interrupt occurred, the DFC itself is used. For example a driver creates two DFC call-backs, one for timeout interrupts and one for Busy/ready interrupts. This removes the need to disable interrupts as no memory is shared between ISR and DFC’s.

It is not advisable to make excessive use of this pattern.

## Obsolete Scheduling Patterns

### Use of NKern::Lock

NKern::Lock can no longer be used as a mechanism to stop scheduling as it only affects the core that the calling code is running on. Therefore use of this lock in areas where there is a possibility of multi-processor access should be replaced by a DMutexes, NFastMutex, spin lock [section 3.2] or atomic operation [section 3.1].

Currently this would only affect kernel side code that can be accessed by a user side thread through an exec, for example in a DLogicalChannelBase derived driver or in a Hal handler implementation. This can also effect drivers which contain multiple threads or has code which is called back in the context of an unrelated thread e.g. TTickLink or BSP bus drivers which use call-backs.

### Use of Interrupt Disabling

Like NKern::Lock, Interrupt disabling functions (NKern::DisableInterrupts,  NKern::DisableAllInterrupts) only affect the core that the calling code is running on. As highlighted in the section 3.2, nominally use of this kind of locking should be replaced with spin locks using the portable macros supplied (__SPIN_LOCKIRQ). These macros automatically revert to interrupt disabling in single core platforms.

Current use of NKern::DisableInterrupts and NKern::DisableAllInterrupts within drivers should be evaluated. Consider the following:

- Could the same functionality be accomplished by disabling specific interrupts in the hardware device itself (with appropriate use of memory barriers of course), to ensure that concurrent access to shared memory never occurs.  Be aware that if this approach is used the driver thread could be pre-empted while it has disabled the interrupt thus leave a specific interrupt disabled for a potentially long time.

- Are the NKern::DisableInterrupts and NKern::DisableAllInterrupts functions required at all? Some drivers may have mistakenly guarded memory that is not accessed in ISR routines and as such could instead be replaced by spin locks or atomic operations.

- Consider use of DFC signalling [section 3.4.3].

## Interrupt Handling

Handling interrupts correctly is even more important on an SMP system, although effectively the same problems apply on both single processor and multiprocessor systems; interrupt debugging is more difficult on multiprocessor systems.

The following examples detail common issues with interrupt handling, it can be assume that memory barriers and spin locks are used as appropriate.

### Empty Interrupts

- An Interrupt is received/processed & Variable x is set with the interrupt reason and the DFC is queued.

- DFC is executed but before Variable x can be read, another interrupt is received (on a single-core this would pre-empt the DFC, on SMP the DFC and ISR could be executed concurrently).

- The Interrupt is received/processed and Variable x is updated again with the interrupt reasons and the DFC is queued for a 2nd time.

- The DFC reads variable x; both interrupt reasons are processed and call-backs are issued. Variable x is then cleared as these reasons have been dealt with.

- DFC is executed, Variable x is read and found to be empty (i.e. no reason for the DFC), code then could ASSERTS as this condition shouldn’t normally arise.

2.)

1.)

DFC

Time

DFC Q’d

Set var x

IRQ

Set var x

Process var x

DFC Q’d

DFC

var x empty!

IRQ

3.)

4.)

5.)

There can be various solutions to this problem all of which depend on the device driver’s functionality and its use of Interrupts. Solutions may include:

- Only queue a DFC once all outstanding interrupts have been received.

- Serialise the interrupts such that only one interrupt signal is un-masked at a time, although this can affect performance.

- Silently “swallow” the empty interrupt.

### Disappearing Interrupts

- A driver is in the process of retrieving data, it masks all interrupts except “Command received”.

- An ISR is received and the Interrupt status register is checked to see if “Command received” is set, which it is.

- After reading the Interrupt status register “Data Ready” is set, interrupt status can still be set even though an actually interrupt is not generated or is still in ISR context.

- Before exiting the ISR the interrupt status register is cleared.

- The driver then masks all interrupts except “Data Ready”, it is at this point the driver effectively hangs because the device has already reported this signal.

This issue can be simply resolved by specifically only clearing those interrupts which are currently un-masked.

## Hardware Accessor Methods

As noted by section 3.2 in this document, guarding is required around any shared memory access, nominally hardware registers are mapped to system memory in effect making them shared memory as any Kernel side process/thread could access them at any time. Therefore register access must also be guarded.

### Register Access

The Modular Hardware Adaptation (MHA) layer has introduced standardised accessor classes & methods for register access; for SMP platforms Write and Modify methods are already implemented with use of spin locks to guard register access. It is recommended that all device drivers use the AsspRegister class for all such register accesses.

These accessor methods allow registers to be read, written, and modified, for registers of size 8, 16, 32 and 64 bits.  The modify(…) method is particularly useful, as it allows part of a register to be modified, without affecting the other bits – an operation that would otherwise require a read and a write operation, surrounded by a spinlock.

Where a device driver cannot make use of the MHA, for example a NAND driver might need to be capable of executing outside of the Symbian OS for the OS loading functionality and therefore cannot rely on any Symbian OS methods/Classes, its specific accessor functions for shared peripheral registers, which do any of the following, must be protected with a spin lock rather than by simply disabling interrupts:

- Read-modify-write operations.

- 64-bit wide register operations.

- A RAM variable used to represent the state of a write only register.

### Register Access Ordering

It is not uncommon for device drivers to perform register operations that require multiple uninterrupted device accesses; an example of this is a media driver writing data to a store:

- Set Address Register (0x1234)

- Set Data Register (“ABCD”)

- Set Command Register (WRITE)

Although individually each register operation is protected as a group this may not be the case, e.g. another thread could change the address register before the command register is set, this may result in “ABCD”  not be written to location 0x1234 . For these cases an additional spin lock could be used to encompass the register operations e.g.

i.) __SPIN_LOCK_IRQSAVE ( )

- Set Address Register (0x1234)

- Set Data Register (“ABCD”)

- Set Command Register (WRITE)

ii.) __SPIN_UNLOCK_IRQRESTORE ( )

In effect this is “nesting spin locks”, where each register operation itself utilises a spin lock.  As a result, you would have to ensure your TOrder value was greater then the one used in the MHA. (At the time of writing, the reference MHA used a TOrder of EOrderGenericIrqLow1)

## Thread Affinity

APIs have been provided to control the affinity of a thread. There are no process level equivalents, specifically only APIs which affect all thread/s in a process exist at the moment. These new APIs do not really have any impact on the current base port but are mentioned here for completeness.

Some functions have also been provided to allow groups of threads to be treated as a single unit in terms of affinity, i.e. to guarantee that all threads belonging to a group are scheduled on the same CPU. Note that a group as a whole can migrate from one CPU another. We are still at a very early stage in the development of the scheduling area in general and therefore no details on those functions are provided here.  Generally the thread affinity APIs should not be used except to work around limitations of the prototype kernel or for testing proposes - e.g. to force scheduling of device drivers on CPUs other than CPU 0.

The more salient functions are:

| IMPORT_C TUint32 NKern::ThreadSetCpuAffinity(NThread* aThread, TUint32 aAffinity) |  |
| --- | --- |
| Description | Change the CPU affinity of a thread |
| Preconditions | Call in a thread context. @param The new CPU affinity mask @return The old affinity mask |
| Parameters | aThread pointer to NThread whose affinity needs changing aAffinity The new CPU affinity mask |
| Return | The old affinity mask |

| IMPORT_C TUint32 NKern::CurrentCpu(); |  |
| --- | --- |
| Return | Return the CPU number of the calling CPU. |

| IMPORT_C Tint NKern::NumberOfCpus() |  |
| --- | --- |
| Return | The number of available CPUs |

| IMPORT_C Tint NKern::FreezeCpu () |  |
| --- | --- |
| Description | After this the current thread will not migrate to another processor Note: There are kernel APIs which will not work if called after calling NKern::FreezeCpu() – in particular Cache:: functions may crash since they rely on being able to migrate to other CPUs in some situations. |
| Return | A cookie to be passed to NKern::EndFreezeCpu() to allow nesting |

| IMPORT_C Tint NKern::EndFreezeCpu (TInt aCookie) |  |
| --- | --- |
| Description | After this the current thread will again be eligible to migrate to another processor |
| Parameter | Cookie value returned by NKern:: FreezeCpu() |

It should be noted that the NKern::ThreadSetCpuAffinity() API should only be used for driver development and should not be present in production drivers.

## Context Awareness

As seen in the previous chapters, an SMP system requires that greater care is taken with memory access and the ordering thereof.  The difference in code intended for an ISR, IDFC and that of a DFC has now larger, and as such, it is more important to understand the context your method/function is executing in.  Also, if you expect code to be running in a thread context, (such as a DFC), you want to ensure that you understand whether this is the only thread involved in the drivers functionality, or whether there are multiple threads.  It is recommended to document any such assumptions, so that there is no misunderstanding in future.  This advice becomes especially important for exported methods i.e. those used by base ports.   The pattern of creating functions/methods which can be used in any context, for anything other than the most trivial of actions, is to be discouraged!

When it comes to knowing whether the code is to be run as an ISR or thread, further steps can be taken to prevent confusion (and disasters incurred) by adding assertion macros to the start of any function that makes such assumptions.  The following macro is defined in UDEB builds:

#define __ASSERT_NOT_ISR	__NK_ASSERT_DEBUG(NKern::CurrentContext()!=NKern::EInterrupt)

Should a method containing __ASSERT_NOT_ISR, be mistakenly used within an ISR, a panic will be raised.

## Device Driver Shut Down

When a device driver is closed, nominally resources are released and outstanding requests are cancelled, on an SMP system there is a problem that it may not be guaranteed that outstanding requests are actually cancelled.

Any call-backs, DFCs or IDFCs to a device driver from a thread that are not owned by that driver are effectively in an undetermined state. For example use of NTimer within a device driver, a function pointer is passed at construction time which can be executed from the timer ISR or from the nanokernel timer dfc1 thread:

- At driver close down a Cancel() is issued for an NTimer object.

- However just before the Cancel() the NTimer expired and caused the function to begin execution on a different core, inside the dfc1 thread or within an ISR.

- Driver close down continues and all driver resources are released.

- The NTimer function then tries to access memory/pointers that no longer exist.

It should be noted that Interrupts are not considered an issue at shutdown because when unbinding an interrupt it will block until its outstanding ISR are complete.

We are still working on a solution to this problem, in the mean-time any drivers with this issue should indicate so within the respective drivers SMP safe analysis.

## Finalising a Device Driver

Once a Device driver has been migrated and declared SMP safe, the mmp files for the effected driver’s executables must be updated with the keyword SMPSAFE.

Currently all binaries and associated interrupts are locked to CPU0 by default [section 2]; however any binaries with the SMPSAFE keyword will be unlocked from CPU0 such that they can be executed on any core.

It should be noted that at the moment the SMPSAFE keyword has no effect, therefore for interim testing of device driver migration the CPU affinity API’s maybe used to provide the same effect.

NKern::ThreadSetCpuAffinity(NKern::CurrentThread(),KCpuAffinityAny);

## Revisited: Spinlocks

In 3.2.1 spinlocks where introduced.  Here we expand on what was said, showing the larger set of options that are available.  Chapter 3.2.1 gave all of the necessary information for spinlocks, and this chapter is only intended for those who want a greater understanding of spinlocks.   Section 3.12.1 describes some extra macros that can be used, and sections 3.12.2 and 3.12.3 go into more detail about problems that can occur if they are used badly.

### The spinlock API

The spinlock API has more forms then was previously described, to cope with different IRQ requirements.  A list of available methods is shown below:

| IMPORT_C TSpinLock(TUint aOrder); |  |
| --- | --- |
| Description | Constructor |
| Parameters | aOrder – order value associated with TSpinLock (see TSpinLock::TOrder). A spin lock can only be acquired if any previously acquired locks have a higher order number. This condition is checked in UDEB builds to prevent deadlocks but should always hold true. |

| IMPORT_C void LockOnly(); |  |
| --- | --- |
| Description | acquire the lock, assuming interrupts/preemption already disabled |

| IMPORT_C void UnlockOnly(); |  |
| --- | --- |
| Description | release the lock, don't change interrupt/preemption state |

| IMPORT_C TBool FlashPreempt(); |  |
| --- | --- |
| Description | if another piece of code is waiting for the lock, UnlockOnly() then LockOnly(), after the unlock NKern::PreemptionPoint is called |
| See also | NKern::PreemptionPoint |
| Returns | ETrue if the Unlock/Lock sequence happened EFalse otherwise |

| IMPORT_C void LockIrq(); |  |
| --- | --- |
| Description | Disable interrupts and acquire the lock |

| IMPORT_C void UnlockIrq(); |  |
| --- | --- |
| Description | release the lock, then enable interrupts |

| IMPORT_C TBool FlashIrq(); |  |
| --- | --- |
| Description | if another piece of code is waiting for the lock or there is an interrupt pending, UnlockIrq() then LockIrq() |
| Returns | ETrue if the Unlock/Lock sequence happened EFalse otherwise |

| IMPORT_C Tint LockIrqSave(); |  |
| --- | --- |
| Description | Disable interrupts and acquire the lock |
| Return | Original interrupt state |

| IMPORT_C void UnlockIrqRestore(Tint); |  |
| --- | --- |
| Description | release the lock, then restore interrupt state |
| Parameter | Interrupt state |

| IMPORT_C TBool FlashIrqRestore(Tint); |  |
| --- | --- |
| Description | if another piece of code is waiting for the lock or there is an interrupt pending, UnlockIrqRestore() then LockIrq() |
| Parameter | Interrupt state |
| Returns | ETrue if the Unlock/Lock sequence happened EFalse otherwise |

The spin lock macros associated with these calls are listed below:

| Macro | Parameter | Return | SMP | Non-SMP |
| --- | --- | --- | --- | --- |
| __SPIN_LOCK_IRQ(lock) | TSpinLock |  | LockIrq | NKern::DisableAllInterrupts |
| __SPIN_UNLOCK_IRQ(lock) | TSpinLock |  | UnlockIrq | NKern::EnableAllInterrupts |
| __SPIN_FLASH_IRQ(lock) | TSpinLock | TBool | FlashIrq | NKern::EnableAllInterrupts NKern::DisableAllInterrupts |
| __SPIN_LOCK_IRQSAVE(lock) | TSpinLock | Tint (interrupt state) | LockIrqSave | NKern::DisableAllInterrupts |
| __SPIN_UNLOCK_IRQRESTORE(lock,irq) | TSpinLock Tint (interrupt state) |  | UnLockIrqRestore | NKern::RestoreInterrupts |
| __SPIN_FLASH_IRQRESTORE(lock,irq) | TSpinLock Tint (interrupt state) | TBool | FlashIrqRestore | NKern:: RestoreInterrupts NKern::DisableAllInterrupts |
| __SPIN_LOCK(lock) | TSpinLock | TBool | LockOnly | Nothing |
| __SPIN_UNLOCK(lock) | TSpinLock |  | UnlockOnly | Nothing |
| __SPIN_FLASH_PREEMPT(lock) | TSpinLock | TBool | FlashPreempt | NKern::PreemptionPoint |

If a TSpinLock object is used within an ISR context (As well as the current use in question) and the ISR state is not known (this is generally the case for device drivers) then only the following macros are to be used with that TSpinLock object, even within ISRs:

- __SPIN_LOCK_IRQSAVE(lock)

- __SPIN_UNLOCK_IRQRESTORE(lock, irq)

- __SPIN_FLASH_IRQRESTORE(lock, irq)

Some may question the need for these versions of the locks within an ISR, but the use of these becomes important if the code were to be used in an environment with multi-level IRQs. Since the use of this is harmless in a non-multi-level system, it is recommended to always use these as if it were multi-level.  Generally, if in doubt, these are the macros to use.

However, if you need a spin lock, in a block of code where you have already disabled interrupts, then you could use the following macros:

- __SPIN_LOCK(lock)

- __SPIN_UNLOCK(lock)

- __SPIN_FLASH_IRQRESTORE(lock, irq)

This saves you needlessly re-disabling interrupts. If using __SPIN_FLASH_IRQRESTORE, then the irq parameter should be that returned from NKern::DisableAllInterrupts (Which you would have used to disable the interrupts with, before the spinlock.).

If you do not use this TSpinLock object in an ISR, you could also use these locks inside a block protected with NKern::Lock. If doing so, one must use a different TOrder value when creating the TSpinlock object, such that bit 7 is set (See Appendix A.2Error: Reference source not found for the full list of TOrder enums).  You should also use the following within the block, instead of __SPIN_FLASH_IRQRESTORE.

- __SPIN_FLASH_PREEMPT(lock)

The final set of Macros is for when a TSpinLock object is used within an ISR context (and therefore IRQs must be disabled) and the ISR state is known (Usually only the Kernel is aware of IRQ state, and therefore these are rarely of use).  The macros are as follows:

- __SPIN_LOCK_IRQ(lock)

- __SPIN_UNLOCK_IRQ(lock)

- __SPIN_FLASH_IRQ(lock)

One should take care not to mix uses of the different sets of macros, within any one instance of lock/unlock.  For example, if one took a spinlock with __SPIN_LOCK_IRQSAVE(lock),then one should use __SPIN_UNLOCK_IRQRESTORE to release it, and not say __SPIN_UNLOCK_IRQ. Also, if you have opted to protect the spin lock with NKern::Lock (instead of disabling interrupts), then you must lock this, this way, on every usage of your TSpinLock object. (This is enforced)

A read write TSpinLock is also available. This optimisation allows multiple readers to acquire the lock simultaneously. Thus an attempt to acquire the lock for read is allowed to proceed if the lock is either free or just held for read, but blocked if the lock is held for write. On the other hand an attempt to lock for write will proceed if the lock is not held for read or for write and block otherwise. The API looks identical except every function is duplicated and has a postfix of R for read accesses and W for write accesses. Equally there is a set of macros which have a postfix of _R or _W for read and write access respectively. An overview of the class and macros can be found in Appendix A.3.

### Core Deadlocking

If acquiring a spin lock in a DFC or thread context that is also used in an ISR, it is necessary to disable interrupts first. The spin lock API provides functions that will disable interrupts before the lock is acquired (LockIrqSave) and restore them after the lock is released (UnlockIrqRestore). Note that the action of disabling interrupts only changes the state of interrupts on the core which is acquiring the lock.

The following illustration explains why this is important:

1.)  DFC (Core 0)

ISR (Core 0)

__SPIN_LOCK(a)

Update Var x

Update Var x

__SPIN_LOCK(a)

2.)

3.)

4.)

Loop until spinlock is unlocked.

(Never)

(Never Reached)

(Never Reached)

The above illustration shows the following events, which can happen if interrupts are left enabled:

- A DFC is to update variable x, it issues a spin lock for ‘a’ and acquires the lock.

- However, before the DFC can update variable x, an interrupt is received.

- The ISR also wants to update variable x and tries to issue a spin lock for ‘a’.

- The ISR can not acquire the lock and this causes the core to spin forever.

This is obviously unacceptable.  To avoid this, we disable interrupts, and get the following situation:

1.)  DFC (Core 0)

ISR (Core 1)

__SPIN_LOCK_IRQSAVE (a)

__SPIN_LOCK_IRQSAVE(a)

2.)

3.)

Loop until spinlock is unlocked.

(Never long)

__SPIN_UNLOCK_IRQRESTORE(a,i)

Update Var x

Update Var x

__SPIN_UNLOCK_IRQRESTORE(a,i)

4.)

5.)

The diagram above shows the following events:

- A DFC is to update variable x, it issues a spin lock IRQ for ‘a’ and acquires the lock

- However before the DFC can update variable x an interrupt is received (as interrupts are disabled on Core 0 execution is moved to Core 1)

- The ISR also wants to update variable x and tries to issue a spin lock for ‘a’

- The DFC continues execution, eventually releasing the lock.

- The ISR acquires the lock and can then execute as normal.

Using a spinlock in an ISR can appear to go against the golden rules of ISR’s, in that you should never block, or do anything that will take any large amount of time.  However, a spinlock doesn't technically block, and it nothing should ever hold a spinlock for very long, and so should not be a problem.

### More deadlocks and wasting time with spinlocks.

A common misconception is that if you are not using your TSpinLock object in an ISR, then you don't need to disable interrupts.  This is not true!  If you tried, you would hopefully hit a debug assertion, protecting you against this dangerous pattern.  For those that have an inquisitive mind, here follows an explanation of why it cannot be allowed.

Figure a) shows one example of what could happen with unguarded spinlocks, on a system with many threads running.  Each column represents a thread, and the vertical axis is time, running down.  Note that the time axis is not linear, and just shows the order of events.

The following events happen in the diagram above:

  - Thread A takes a spinlock.

  - Thread A’s time slice expires, and thread B starts to run.

  - Thread B wants to hold the spinlock, and spins until it is free.
The spinlock cannot be unlocked until after thread A gets to run again!

  - Eventually, thread B’s timeslice expires, and thread C gets to run

  - Thread C has nothing to do with the spinlock, but has plenty to do.

  - Thread D that is running on a different core, attempts to take the spinlock, and has to spin until thread A leases it.

  - Thread C either blocks, or its time slice expires, and thread A gets to run again.

  - Thread A ‘sets flags’, and releases the spin lock.

  - Thread A’s time slice hasn't expired and it does more operations not requiring a lock.

  - Thread either blocks or time slice expires, and thread B finally gets to set its flags, before unlocking the spinlock.

This scenario shows how two threads on the same core, both taking the same spinlock, can cause the system to spin until the thread is pre-empted.  In computer terms, this is a long time!    Also, if this happens, or any other thread (which may be doing something totally unrelated)  pre-empts the one with the spin lock, the lock will be held for the entire duration of the timeslice, which is far longer than desirable.  Furthermore, any other core, wanting to hold this lock, will end up spinning, needlessly, waiting for the first thread, on the first core, to be re-scheduled.  This is obviously a tremendous waste of computing time and power, and goes against the whole point of spinlocks being a quick short lock.  (A fast mutex would be much faster)

The scenario above could be made far worse, by thread B being made to disable pre-emption (such as by disabling interrupts), or given a higher priority then thread A.  In this case, thread A could never get to run again on this core.  As thread B would never get pre-empted, and would spin forever, this would deadlock the core. (And any other core waiting for the spin lock).

Simply disabling interrupts before taking the spinlock is clearly a far quicker and safer alternative!

Spinlock

Set flags

Spin Unlock

Do lots of other stuff

not needing a alock

Set flags

Spin Unlock

Thread spins, waiting

for the lock.

Inadvertently delaying that very event!

It keeps spinning until its time slice expires.

The thread is prempted

The thread is prempted

Do lots of operations

not needing a lock.

Either finishes doing things, or blocks.

The thread is prempted

Spinlock

The thread is prempted

Thread A (Core 0)

Thread B (Core 0)

Thread C (Core 0)

Spinlock

Thread D (Core 1)

Another core attempts to take the spin lock, and has to spin, until thread A is finished.

a)

b)

c)

d)

e)

f)

g)

h)

i )

j)

Figure a) Using spinlocks with pre-emption enabled

# Further Information

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [1] | ARM IHI 0048 |  | ARM Generic Interrupt Controller Architecture Specification |

- Additional Information

  - Old Atomic APIs

Nanokernel:

TInt LockedInc(TInt& aCount)

TInt LockedDec(TInt& aCount)

TInt LockedAdd(TInt& aDest, TInt aSrc)

TInt64 LockedInc(TInt64& aCount)

TInt64 LockedDec(TInt64& aCount)

TInt64 LockedAdd(TInt64& aDest, TInt64 aSrc)

TUint32 LockedSetClear(TUint32& aDest, TUint32 aClearMask, TUint32 aSetMask)

TUint16 LockedSetClear16(TUint16& aDest, TUint16 aClearMask, TUint16 aSetMask)

TUint8 LockedSetClear8(TUint8& aDest, TUint8 aClearMask, TUint8 aSetMask)

TInt SafeInc(TInt& aCount)

TInt SafeDec(TInt& aCount)

TInt AddIfGe(TInt& aCount, TInt aLimit, TInt aInc)

TInt AddIfLt(TInt& aCount, TInt aLimit, TInt aInc)

TAny* SafeSwap(TAny* aNewValue, TAny*& aPtr)

TUint8 SafeSwap8(TUint8 aNewValue, TUint8& aPtr)

TUint16 SafeSwap16(TUint16 aNewValue, TUint16& aPtr)

TBool CompareAndSwap(TAny*& aPtr, TAny* aExpected, TAny* aNew)

TBool CompareAndSwap8(TUint8& aPtr, TUint8 aExpected, TUint8 aNew)

TBool CompareAndSwap16(TUint16& aPtr, TUint16 aExpected, TUint16 aNew)

TUint32 SafeSwap(TUint32 aNewValue, TUint32& aPtr)

TUint SafeSwap(TUint aNewValue, TUint& aPtr)

TInt SafeSwap(TInt aNewValue, TInt& aPtr)

TBool CompareAndSwap(TUint32& aPtr, TUint32 aExpected, TUint32 aNew)

TBool CompareAndSwap(TUint& aPtr, TUint aExpected, TUint aNew)

TBool CompareAndSwap(TInt& aPtr, TInt aExpected, TInt aNew)

User

TInt LockedInc(TInt& aValue);

TInt LockedDec(TInt& aValue);

TInt SafeInc(TInt& aValue);

TInt SafeDec(TInt& aValue);

  - TSpinLock

/** Spin lock

Used for protecting a code fragment against both interrupts and concurrent

execution on another processor.

List of spin locks in the nanokernel, in deadlock-prevention order:

A	NEventHandler::TiedLock (preemption)

B	NFastMutex spin locks (preemption)

C	Thread spin locks (preemption)

D	Thread group spin locks (preemption)

E	Per-CPU ready list lock (preemption)

a	Idle DFC list lock (interrupts)

b	Per-CPU exogenous IDFC queue lock (interrupts)

c	NTimerQ spin lock (interrupts)

d	Generic IPI list locks (interrupts)

e	NIrq spin locks (interrupts)

f	Per-CPU event handler list lock (interrupts)

z	BTrace lock (interrupts)

z must be minimum since BTrace can appear anywhere

interrupt-disabling spinlocks must be lower than preemption-disabling ones

Nestings which actually occur are:

A > C

B > C > D > E

c > f

Nothing (except possibly z) nested inside a, b, d, f

e is held while calling HW-poking functions (which might use other spinlocks)

@publishedPartner

@prototype

*/

class TSpinLock

{

public:

enum TOrder

{

// Bit 7 of order clear for locks used with interrupts disabled

EOrderGenericIrqLow0	=0x00u,	// Device driver spin locks, low range

EOrderGenericIrqLow1	=0x01u,	// Device driver spin locks, low range

EOrderGenericIrqLow2	=0x02u,	// Device driver spin locks, low range

EOrderGenericIrqLow3	=0x03u,	// Device driver spin locks, low range

EOrderBTrace	=0x04u,	// BTrace lock

EOrderEventHandlerList	=0x07u,	// Per-CPU event handler list lock

EOrderNIrq	=0x0Au,	// NIrq lock

EOrderGenericIPIList	=0x0Du,	// Generic IPI list lock

EOrderNTimerQ	=0x10u,	// Nanokernel timer queue lock

EOrderExIDfcQ	=0x13u,	// Per-CPU exogenous IDFC queue list lock

EOrderIdleDFCList	=0x16u,	// Idle DFC list lock

EOrderGenericIrqHigh0	=0x18u,	// Device driver spin locks, high range

EOrderGenericIrqHigh1	=0x19u,	// Device driver spin locks, high range

EOrderGenericIrqHigh2	=0x1Au,	// Device driver spin locks, high range

EOrderGenericIrqHigh3	=0x1Bu,	// Device driver spin locks, high range

// Bit 7 of order set for locks used with interrupts enabled, preemption disabled

EOrderGenericPreLow0	=0x80u,	// Device driver spin locks, low range

EOrderGenericPreLow1	=0x81u,	// Device driver spin locks, low range

EOrderReadyList	=0x88u,	// Per-CPU ready list lock

EOrderThreadGroup	=0x90u,	// Thread group locks

EOrderThread	=0x91u,	// Thread locks

EOrderFastMutex	=0x98u,	// Fast mutex locks

EOrderEventHandlerTied	=0x9Cu,	// Event handler tied lock

EOrderGenericPreHigh0	=0x9Eu,	// Device driver spin locks, high range

EOrderGenericPreHigh1	=0x9Fu,	// Device driver spin locks, high range

EOrderNone	=0xFFu	// No order check required (e.g. for dynamic ordering)

};

public:

IMPORT_C TSpinLock(TUint aOrder);

IMPORT_C void LockIrq();

IMPORT_C void UnlockIrq();

IMPORT_C TBool FlashIrq();

IMPORT_C void LockOnly();

IMPORT_C void UnlockOnly();

IMPORT_C TBool FlashOnly();

IMPORT_C TInt LockIrqSave();

IMPORT_C void UnlockIrqRestore(TInt);

IMPORT_C TBool FlashIrqRestore(TInt);

IMPORT_C TBool FlashPreempt();

…

};

  - TRWSpinLock

/** Read/Write Spin lock

@publishedPartner

@prototype

*/

class TRWSpinLock

{

public:

IMPORT_C TRWSpinLock(TUint aOrder);

IMPORT_C void LockIrqR();

IMPORT_C void UnlockIrqR();

IMPORT_C TBool FlashIrqR();

IMPORT_C void LockIrqW();

IMPORT_C void UnlockIrqW();

IMPORT_C TBool FlashIrqW();

IMPORT_C void LockOnlyR();

IMPORT_C void UnlockOnlyR();

IMPORT_C TBool FlashOnlyR();

IMPORT_C void LockOnlyW();

IMPORT_C void UnlockOnlyW();

IMPORT_C TBool FlashOnlyW();

IMPORT_C TInt LockIrqSaveR();

IMPORT_C void UnlockIrqRestoreR(TInt);

IMPORT_C TBool FlashIrqRestoreR(TInt);

IMPORT_C TInt LockIrqSaveW();

IMPORT_C void UnlockIrqRestoreW(TInt);

IMPORT_C TBool FlashIrqRestoreW(TInt);

IMPORT_C TBool FlashPreemptR();

IMPORT_C TBool FlashPreemptW();

private:

…

};

Macros:

__SPIN_LOCK_IRQ_R(lock)

__SPIN_UNLOCK_IRQ_R(lock)

__SPIN_FLASH_IRQ_R(lock)

__SPIN_LOCK_IRQ_W(lock)

__SPIN_UNLOCK_IRQ_W(lock)

__SPIN_FLASH_IRQ_W(lock)

__SPIN_LOCK_R(lock)

__SPIN_UNLOCK_R(lock)

__SPIN_FLASH_R(lock)

__SPIN_LOCK_W(lock)

__SPIN_UNLOCK_W(lock)

__SPIN_FLASH_W(lock)

__SPIN_LOCK_IRQSAVE_R(lock)

__SPIN_UNLOCK_IRQRESTORE_R(lock,irq)	(lock).

__SPIN_FLASH_IRQRESTORE_R(lock,irq)

__SPIN_LOCK_IRQSAVE_W(lock)

__SPIN_UNLOCK_IRQRESTORE_W(lock,irq)	(lock).

__SPIN_FLASH_IRQRESTORE_W(lock,irq)

__SPIN_FLASH_PREEMPT_R(lock)

__SPIN_FLASH_PREEMPT_W(lock)

- Case Studies

This appendix is intended to capture Device driver SMP analysis performed by Symbian engineers. It is hoped that these practical applications will provide an insight into the required thought patterns and potential issues caused by the different types of drivers available.

  - Btrace

The Btrace driver has been most affected by the SMP migration. This is because it makes use of a shared chunk to pass the Btrace data and more importantly metadata to the user side. Key information such as addresses from which Btrace data is available, or the Btrace data itself, can be accessed simultaneously by more than one CPU. This is particularly the case when the Btrace data is being read out. Adding new Btrace records is protected by a spin lock, and is therefore safe. However reading of data does not require using the spin lock. This optimisation allows user side threads on one CPU to read out the data from the Btrace buffer whilst another CPU is writing data into it. The code is more complex as appropriate flags need to be set and tracked in this situation, to avoid corruption of the data. When kind of approach is used, it is essential that judicious use is made of memory barriers and atomics to ensure the other CPU always observes a valid state.

  - Sound Driver (Shared Chunk)

For SMP functional both playback and recording will work on the same DFC queue. Driver creates and owns this DFC queue and provides it to the Sound LDD. (LDD and PDD will work on the same DFC queue). The DMA framework is used to control the data coming out and to the hardware. For each unit (either playback or record) DDmaRequest objects are used. Each unit during the initialization creates a number of these objects (defined as KMaxRxDmaRequests and KMaxTxDmaRequests – in the NaviEngine PDD both of them are 2). This is to make use of the DMA double-buffering support.

DDmaRequest objects need the callback functions to be provided during their creation. For each unit the same callback function is used for all request objects created. Request objects are also configured to use the same DFC queue that was used for the driver. This all means that the operation of the DFC thread will be the same for both uniprocessor and SMP systems.

Interrupts are not used directly by the driver. DMA request objects, even though they are owned by the driver – are issuing callbacks triggered by the Interrupts handled by the DMA framework. There is no driver shutdown problem - provided, that the DMA framework don’t issue the callback after all requests were cancelled and request objects destroyed.

Audio playback and record hardware is functionally distinct enough to so separate playback and recording *but only*, if the same settings are used for playback and record. In this case it can be considered to use separate DFC queues (Kernel threads) to process each of them.

  - MMC Driver

Generic PIL

MMC Stack and media driver runs in 1 thread context, it creates 1 owned DFC Queue,

depending on the platform 1 or 2 interrupts maybe used for card and media change events.

The MMC State Machine makes use of “disableallinterrupts()” to prevent pre-emption when setting status flags or manipulating the session queue.

The stack variables themselves aren’t actually accessed by any ISR’s, although the session queue maybe manipulated in an ISR context {again depending on the platform}.

Each MMC stack session makes use of 3 NTimer classes:

NTimer iPollTimer

NTimer iRetryTimer

NTimer iProgramTimer

These are of concern with regards to driver close down, see section 3.10 for details.

A shared memory buffer is used for DMA purposes however access is restricted to the MMC media driver, stack and PSL so there are no concurrency issues, also the driver make use of normal IPC API's and should automatically be safe for client I/O messaging.

With some additional changes and some correct IRQ management, DisableInterrupts() can be removed completely from Programmatic IO (non-DMA).

Testing

General Test coverage can be achieved through use of F32tests on the MMC drive

The MMC specific unit tests (T_MMCDRV & T_PWSTR) should also be executed.

No SMP focused tests may currently be run, mainly due to the hardware configuration of the SMP reference device. Ideally the MMC Stack should be tested with multiple sessions (i.e. CPRM support or execution with SDIO drivers), to increase the chances of multiple sessions being queued and manipulated.

Because the generic layer required changing a full regression run must be performed on all available platforms as well.

  - I2S Driver

I2S MHA is a kernel extension, exporting the static library for clients to link against it. It exposes channels, and is designed for one user (thread) per channel. (although this is not enforced by the API).

All methods which can be also used from the ISR context were provided in locking mechanisms.

Also, all points, where the uninterrupted sequence of the configuration is needed were covered by using locking mechanism.

No further work needs to be done both for SMP functional and SMP optimized.

  - PCI Driver

The PCI driver was written to be thread safe, and is also SMP safe:

As such:

- It does not make use of interrupt disabling or NKern::Lock for synchronisation

- It uses the MHA API for register access.

- Spinlocks are not used as there is no need to access shared data in ISR context

- Missing/Empty interrupt issues will not arise as the driver only uses ISRs to handle bus errors, and does not schedule DFCs in response

- Shared driver data may be accessed from multiple client threads but is protected by DMutexes

- Where multiple registers must be modified consistantly, NFastMutex is used for protection.

- The driver does makes use of DFC queue 0 (when chunks get closed they schedule a DFC which calls into the driver to remove their memory mapping) it seems acceptable to use this queue as the latency of the operation is not critical, and it shouldn't hold up the dfc queue itself.

With regards to SMP testing it is not necessary to modify the CPU affinity of any threads within the PCI driver. This is because the driver does not have any threads of its own - it works solely in its client threads' context, and these may be running on any CPU.

  - LCD Driver

Authors Note: This analysis was performed for both SMP and WDP changes concurrently and as such contains notes on both. SMP specific points of interest have been highlighted.

Generally fine as most operations access the display buffer, secure display buffer or palette buffer, none of which will be paged. Most other user memory accesses are in HAL calls. Config buffers are the exception. Can probably fix with client-side stack-based copies. The public nature of the HAL calls means that there could potentially be multiple clients changing attributes such as colour depth simultaneously. These calls should invoke a kernel message to be processed in a separate kernel thread to synchronise/serialise such calls.

Details:

- Needs to be moved to own kernel thread to avoid potential HAL call sync problems

- No uses of Kern::DfcQue0() or Kern::DfcQue1() but does use OMAP::PlatformSpecificDfcQ()

- NFastMutex x2 - iLock doesn't access user memory, iUpdateLock doesn't seem to be used anywhere

- Kern::InfoCopy - x3 HAL functions

- kumemget32 - x2 HAL functions

- kumemput32 - x11 HAL functions

  - Serial Driver

Authors Note: This analysis was performed for both SMP and WDP changes concurrently and as such contains notes on both. SMP specific points of interest have been highlighted.

The EComm LDD is a DLogicalChannel driver where user-mode data is sent and received on kernel-side DFC threads. This means a bit of real work to shift the user-memory accesses into the client threads.

Details:

- DChannelComm::Complete() should use Kern::QueueRequestComplete instead of Kern::RequestComplete.

- DChannelComm::DoDrainRxBuffer should use TClientBuffer + Kern::ThreadBufWrite instead of Kern::ThreadDesWrite.

- DChannelComm::DoFillTxBuffer shoul use TClientBuffer + Kern::ThreadBufRead instead of Kern::ThreadDesRead.

- DChannelComm::HandleMsg() should use Kern::QueueRequestComplete instead of Kern::RequestComplete.

- DChannelComm::DoRequest() has a number of places where single 32-bit values are read from the client

- thread with Kern::ThreadRawRead. These can easily be passed by value instead.

- RBusDevComm - corresponding changes needed for channel requests

- DChannelComm::InitiateRead() uses Kern::ThreadGetDesMaxLength() and Kern::ThreadDesWrite() to check the

- user-mode buffer is big enough and to empty it ready to receive. I see no reason this cant be done in

- user-mode (i.e. by RBusDevComm) before the Read request is issued to the LDD.

- DChannelComm::UpdateSignals() should use TClientBuffer + Kern::ThreadBufWrite + Kern::QueueRequestComplete

- DChannelComm::DoControl() - although Kern::ThreadDesRead and Kern::ThreadDesWrite are used, the control

- Messages that use them are always in client context, not DFC context. No action needed.

  - Camera LDD

Makes no use of disabling interrupts to synchronise access to data, implements a call-back function that the PDD calls from thread context via a Dfc on its DfcQ.

Memory sharing between the kernel and the user is involved since the driver uses a shared chunk for transferring images to the user. However, the driver uses an image buffer locking and synchronisation mechanism which makes it SMP safe.

Finally, for an SMP enabled platforms that uses multiple sensors, a DfcQ per camera h/w unit is recommended, which is an optimisation rather than required functionality for SMP.

  - USB Host Stack

    - Host Driver for Reference Hardware

A spinlock, sc_spinlock was introduced in the OHCI controller to protect shared variables and registers (accessed in both ISR and thread context). When binding the OHCI's interrupt, Interrupt::Clear is called and its return value is checked against KErrNone.  On SMP platforms Interrupt::Clear currently returns KErrNotSupported so this is also allowed as a return value to cope with this case.

    - OTGDI

The OTGDI maintains circular lists (implemented as fixed size arrays with head and tail indexes) of pending OTG events/states/messages from the client which are to be processed.  Any additions/removals to these lists is now protected by use of the mutex the OTGDI has for enforcing single streaming.

OTGDI Message Queue (OTG events and debug strings)

The OTGDI deals with buffering and outputting debug from both ISR's and thread contexts.  ISR debug is added to a fixed circular buffer to prevent the need for allocation/free'ing which is not allowed in ISR context.  A spinlock was introduced for protecting access to the circular debug buffer used by ISR's to output debug.  This prevents clashes between the ISR's adding debug to the buffer and the OTG DFC emptying it and printing the debug.

The OTGDI maintains a circular list (implemented as a fixed size array with head and tail indexes) of OTG events which include debug messages and events such as notifications of HNP/SRP states etc. which the USB Host stack and the driver's client interface populate during its operation.  These events are dealt with by the OTG DFC (which is queued each time an event is added).  Debug events have a pointer to an allocated string containing the debug to be printed.  This is allocated by the thread that adds the debug message to the circular list.

Items are added whilst holding the OTGDI single streaming mutex.  Once added the OTG DFC is queued to deal with the pending messages.  If the list is full at the time of adding, it is wrapped around and the oldest item is overwritten.  If this element being overwritten is a debug message, the debug string is free'd.

The OTG DFC processes the list of events and in the case of debug events, delete's the associated string after it has been printed.  When the OTG DFC is processing the items in the list, it only holds the OTG single streaming mutex long enough to modify the head and tail indices (to prevent holding up the driver from processing time critical USB/OTG events).  The event is then processed in-situ in the list (meaning its debug string could be leaked if another item overwrote it and also non-debug events could be processed out of order).

In the process of making the OTGDI SMP safe, a flag was introduced in the OTG elements in the list to signify whether they are currently being processed by the OTG DFC thread.  Whilst the OTG DFC is holding the single streaming mutex, it takes a local copy of the element to prevent the data being overwritten and flags the original element as being processed.  If the element being overwritten has this flag set, the debug string will not be deleted by the thread adding further events (as the OTG DFC will do this after it has dealt with the debug string).

    - USB Host stack

The implementation of spinlocks in the USB Host stack was based on disabling and enabling interrupts.  It has been changed to use the __SPIN_LOCK_IRQXXXX macros.  Spin locks get allocated/initialised and deleted each time the stack starts/stops.  Initially and when the stack is stopped, these spinlocks are set to NULL to help catch misuse of the lock and unlock functions.  There is a spinlock order map introduced to allow the correct spinlock order to be allocated to spinlocks.

When grabbing and releasing the giant mutex, the USB Host stack uses a spinlock (in debug mode) to modify some flags relating to the giant mutex (whether it is held, current thread holding it etc.) for diagnostic purposes.  Some of the code referencing this diagnostic data didn't do so under spinlock so for SMP safeness the spinlock is now held and released whenever referencing or changing this data.

Interrupts

The USB Host stack maintains a list of ISR's and DSR's (Deferred Service Routine) that are bound to each interrupt.  When an interrupt occurs, each ISR in the list for that interrupt is called in turn and it has the option of queuing its associated DSR if further processing is required (by flagging it in the list of ISR's/DSR's).  At the end of the generic ISR function, the generic DFC is added if any ISR flagged for its DSR to be called.  If the generic DFC is called it walks the ISR/DSR list and calls in turn each DSR that is flagged as to be called and clears each flagged DSR in turn.  To make the interrupts SMP safe, a spinlock was introduced gIntSpinLock around any code walking the list or modifying the call DSR flags.

A DFC was introduced to remove ISR/DSR combinations from the list.  The call that triggers the queuing of this DFC then waits on a newly introduced cleanup semaphore to confirm the ISR/DSR has been safely removed and there are no pending DFC's scheduled to be called for that ISR/DSR combination.  In the remove DFC, the ISR/DSR combination is flagged as disabled and it is removed from the list under gIntSpinLock to prevent the ISR from interfering.  Any cleanup necessary is performed and then the cleanup semaphore is signalled to indicate the ISR/DSR combination has been successfully and cleanly removed.

USBDI

The USBDI maintains a list of pipes for each channel.  If a channel is being closed (from either client side or driver side), the giant mutex is held and then all the pipe's in the channel's list are deleted.  During deletion of a pipe, the giant mutex has to be released for a short period before being retaken.  If both the client side and the driver side were simultaneously trying to close a channel, it was possible for both sides to be modifying the list of pipes.  The local copy of the list was then inconsistent and caused crashes.   A flag was introduced to signify a channel is being closed.  This flag is set whilst the giant mutex is held and before deleting of the pipes begins.  The close channel function now checks this flag and if it is set, does nothing (as the channel is already being closed).

Other

The USB Host stack used to maintain a current context global variable which could be queried to see whether the stack was currently in ISR context or thread context as in some cases e.g. debug processing, different things had to be done depending on the context.  This global variable was removed and replaced by calls to NThread::CurrentContext as each core could be in a different context.

    - T_USBDI tests

Test 0473

This test assumed that 3 notifications it was waiting for would happen in a specific order.  This no longer held true on an SMP enabled platform so the test was changed to tick off notifications as they happened and only proceed once all 3 notifications had been received (regardless of the order in which they occurred).

Test 0495 and Test 0498

Both of these tests used a local descriptor variable as a wrapper for a result field in an asynchronous call.  This was changed to be a member variable to prevent a KErrBadDescriptor error.

Test 1229 and Test 1230

On SMP platform, the delay between the EP0 transfer complete being signalled in the controltransferrequest.cpp RunL and the time that the RecordTime was being called in the 1229 and 1230 testcases was either very quick or very slow.  This meant the tests failed  even though the times recorded at the RunL were well within the boundaries.  The tests were changed to record the completion times as they happened in the controltransferrequest.cpp RunL rather than waiting further down the line until RecordTime was called.

  - DMA Driver

The majority of the logic implemented by the platform-independent layer was rewritten with SMP safety in mind some time ago. Platform-specific layer mostly performs hardware access (using AsspRegister) and some buffer management which runs protected by fast mutexes. The detailed analysis is given below:

Architecture and use of DFC queues

DMA driver has a number of DMA channels corresponding to the hardware DMA channels. Each channel has an interrupt code and a DFC queue associated with it. The DFC queue is supplied by the channel’s client in the Open() function. Currently the comment for the function doesn’t make it clear that the clients should not use shared Kernel DFC queues for DMA transfer call-backs – probably the comment should explain this limitation for SMP platforms.

Interface between ISR/DFC

DFC and ISR use a shared 32 bit buffer to communicate with each other and they use atomic ordered operations to access this buffer thus avoiding pre-empted access.

Protection of the data shared between threads

Synchronization between DFC threads and the code executed in the client’s thread is performed using fast mutex objects

Interface between the driver and the hardware

The PSL uses the AsspRegister interface to get the state of the channels and to indicate that an interrupt has been handled.

Driver shutdown

When requested to shut down, the DMA driver would set a cancel flag in the buffer used by ISR – this prevents the ISR from queuing a new DFC. Then the driver would wait for any outstanding DFCs to complete.  These 2 steps ensure that no call-backs are made into the client after the driver has been closed down.

  - Ethernet

The Ethernet PDD for the reference hardware uses one driver spinlock (member variable iDriverLock) accessed through the inline functions DriverLock()/DriverUnlock() which use the SPINLOCK_IRQSAVE/RESTORE macros to protect from simultaneous access from multiple threads or interrupt handlers.

This lock is used to serialise access to the device registers as some of these are required to be accessed atomically or are order dependent:

- TX DATA FIFO should only be written to one packet at a time otherwise data from multiple packets will get intermingled

- MAC and PHY registers are accessed as command/data pairs (i.e. you write the command to execute to one register and the data to operate on to another, this need to be done atomically)

Each member function is commented as to whether or not the iDriverLock should be held on entry to the function.

  - GPIO

The GPIO extension for the reference hardware uses one driver spinlock (static variable GpioSpinLock) accessed through the GpioLock()/GpioUnlock() inline functions which use the SPINLOCK_IRQSAVE/RESTORE macros to protect from simultaneous access from multiple threads or interrupt handlers.

This is required to serialise accesses to the internal GpioPins array and the device registers:

- Several read/modify/write operations to GPIO device registers need to occur atomically

- The GPIO hardware does not fully implement some of the facilities defined in the GPIO API, some of these facilities are handled by recording additional information in the GpioPins array, this needs to be updated atomically and kept consistent with the physical hardware.

  - Power Resource Manager (PRM)

Most of the PRM API's are execute in the context of the client thread but synchronisation is taken care using a mutex. All access to long latency hardware resources takes place from single DFC thread and also access to internal data structures from various threads is also synchronised using a mutex. No code is executed in ISR context and call-backs are executed in client context.

No changes were made to PRM kernel side functionality as it is already SMP safe.

The following changes were made to the user side PRM functionality; the driver used kernel lock (Kern::Lock()) and kernel unlock (Kern::UnLock()) while acquiring and releasing the fast mutex, simply to prevent other threads from running. Kernel lock and unlock have been removed, Also changed the fast mutex release and acquire API's from NFastMutex::Wait and NFastMutex::Signal to NKern::FMWait and NKern::FMSignal.

  - GCE

Most of the driver operations are being executed in the single DFC queue created for the GCE driver, so no synchronisation issue could arise. However, a spinlock is needed in order to synchronise access to the structure that holds the call-back functions information and which is accessed in the context of a vsync interrupt ISR and the context of whichever thread registers or deregisters a call-back function.

  - Watch Dog Timer

No SMP specific changes are required.

The Watchdog timer is a single NTimer that resets the Watchdog hardware reset counter every 5 seconds. The destructor for the watchdog is never called. The Watchdog makes use of MHA methods for register access and is the only driver to access these set of registers.

  - Local Media Subsystem

The Local Media subsystem was already designed to operate in a multi-thread environment; it deals with multiple drive and client threads. It makes use of NKern::Lock() and NKern::Unlock() to ensure that certain code paths are not executed concurrently or pre-empted; these have been replace with the new ‘Atomic’ methods and semaphores were necessary (NKern::Lock() will only ‘lock’ the current CPU core on SMP platforms and will not provide satisfactory protection).

  - Virtual Serial Keyboard

No SMP specific changes are required.

The Virtual Serial Keyboard driver makes use of the existing Serial Driver; which is already SMPSAFE (see B.7).

It makes use of a single interrupt to receive characters from the input device, once received the interrupt is disabled until the character is processed and then interrupts are re-enabled. Use of interrupts in this manner prevents synchronisation issues and data corruption with in the event queue.

  - Key Pad

No SMP specific changes are required.

The Key pad driver has been designed as a simple single thread that uses an NTimer object to poll for key events once every 100 milliseconds. Once a Key event has been received and processed the timer is re-queued. No Interrupts/DFC/Shared memory are used therefore synchronisation issues and data corruption should not occur as a result of execution on an SMP platform.

  - Digitizer

No SMP specific changes are required.

Digitizer Driver makes use of NKern::LockSystem() & NKern::UnlockSystem() when updating calibration data within the Persistent Machine Configuration to guard against multiple access and data corruption. Note use of NKern::LockSystem() is due to the Persistent Machine Configuration object is stored within shared memory such that it can be access by multiple device drivers and services (i.e. HAL).

It also utilises an Interrupt to receive pen position updates, once received the interrupt is disabled until the position coordinates are processed and then interrupts are re-enabled. Use of interrupts in this manner prevents synchronisation issues and data corruption within the pen status variable.

  - NAND

Q1) Does the media driver disable all interrupts at critical points within DFCs so as to synchronise access to shared data or hardware between code executing in DFC and ISR context?  If so have these disable interrupt calls been replaced with SMP safe alternatives?

A1) The media driver doesn’t disable all interrupts to synchronise access between DFC and ISR.

Q2) Are there any existing NKern::atomic calls being made within the media driver DFCs so as to synchronise access to shared data or h/w between code executing in DFC and ISR context? If so have you replaced these calls with suitable calls to new atomic operations added for SMP?

A2) The media driver doesn’t use any NKern::atomic calls.

Q3) Are locks provided around accesses to any data shared between media driver threads? If not have they been added? Have you considered ISRs and timer callbacks as well as main driver threads?

A3) The main driver thread, the ISR and the two timer callback functions have been considered. It is assumed that the NTimer & TDfc objects are SMP safe as SMP versions of these objects appear to be provided in the SMP kernel.

NandIsr

NandInterrupt::Service

No problem with the following Boolean operations:

Set iTimeoutActive

Read iTimeoutActive

Set iDfcQueued

For the reference hardware calls to PAM_ClearInt() & PAM_DisableInt() call the underlying functions are EXPORT_C TInt Interrupt::Clear(TInt aId) & EXPORT_C TInt Interrupt::Disable(TInt aId). As these functions are implemented as SMP specific versions it’s assumed they are SMP safe.

DMediaDriverNand::IdleTimerCallBack

No issues with this simple callback function.

DMediaDriverNand::TimeoutTimerCallBack

No problem with Boolean operation, read iDfcQueued.

Writing Tint iNotBusyReason can’t be an issue as this variable doesn’t get used anywhere else.

The reference hardware calls to PAM_IsNotBusy() & PAM_ProgError() are hard coded to return false so calls to these functions are  SMP safe.

Q4) Are any assumptions being made about the order of execution of media driver threads? If so has the code been modified so that such assumptions no longer apply or alternatively does analysis suggest that such assumptions won’t cause a problem in an SMP environment in this particular case?

A4) The media driver uses one main thread and no assumptions are made about its order of execution in relation to the execution of the ISR and two timer callback functions and no assumptions are made about the relative order of execution of the ISR and two callback functions either.

Q5) Does the media driver call NKern::Lock()? If so have these calls been replaced with an appropriate locking mechanism or reconciled as not causing a problem in an SMP environment in this particular case?

A5) The media driver doesn’t call NKern::Lock().

Q6) Does the media driver use unofficial IPC mechanisms? If it does, have these been replaced with official mechanisms?

A6) I would assume that all the following mechanisms are “official”:

- TLocDrvRequest::WriteToPageHandler()

- TLocDrvRequest::WriteRemote()

- LocDrvRequest::ReadFromPageHandler()

- TLocDrvRequest::ReadRemote()

- Kern::ThreadRawWrite()

- Kern::ThreadRawRead()

The NAND driver makes use of TLocDrvRequest::RemoteDes().

Q7) Are the media driver threads impacted by any Relaxed Memory Consistency issues? If so have these been dealt with? Have you considered ISRs and timer callbacks as well as main driver threads?

A7) The main driver thread, the ISR and the two timer callback functions have been considered. It is assumed that in itself the use of NTimer and TDfc objects won’t cause consistency issues between threads, as SMP versions of these objects are provided in the SMP kernel.

DMediaDriverNand::IdleTimerCallBack

Can’t see how this simple callback function could cause a consistency issue in other threads or be impacted by a consistency issue in another thread.

DMediaDriverNand::TimeoutTimerCallBack

Read accesses can’t cause consistency issues in other threads. Writing Tint iNotBusyReason can’t cause consistency issues in other threads as this variable doesn’t get used anywhere else. Calls to the reference hardware functions PAM_ClearInt() & PAM_DisableInt() call the underlying functions EXPORT_C TInt Interrupt::Clear(TInt aId) & EXPORT_C TInt Interrupt::Disable(TInt aId). As these functions are implemented as SMP specific versions it is assumed they have been coded to avoid causing consistency issues for other threads by just being called. What about the impact of calls to PAM_DisableInt(),PAM_ClearInt() & md.iTimeoutIDfc.Add() getting reordered? There is no need to worry about PAM_ClearInt() as on the reference hardware this amounts to a no op. The callback listing disables the NAND interrupt before queuing the dfc function. The compiler probably wouldn’t queue the function before the interrupt was disabled but even if it did, this would be unlikely to cause any problems as the NAND interrupt would still be disabled rapidly and this would certainly happen before the function could run and schedule a new operation on the NAND flash.

DMediaDriverNand::TimeoutTimerCallBack tests the md.iDfcQueued flag and, if this is not set, the callback function could add the DMediaDriverNand::NandDfcFunction to the dfc queue more than once for one hardware operation. See both DMediaDriverNand::TimeoutTimerCallBack and NandIsr, NandInterrupt::Service add DMediaDriverNand::NandDfcFunction below.

NandIsr

NandInterrupt::Service

Read accesses can’t cause any consistency issues in other threads. What about the impact of calls to PAM_ClearInt(),PAM_DisableInt(),pMDNand->iNandIreqDfc.Add() & pMDNand->iTimeoutTimer.Cancel() and lines setting pMDNand->iTimeoutActive & pMDNand->iDfcQueued getting reordered. There is no need to worry about PAM_ClearInt() as on the NaviEngine this amounts to a no op. The compiler would probably disable the interrupt first but even if it didn’t this would be unlikely to cause any problems as the NAND interrupt would still be disabled rapidly and this would certainly happen before the main thread could schedule a new operation on the NAND flash. The compiler would probably cancel the timeout timer before queueing the dfc but even if this is not the case it makes little difference as it is impossible to ensure that the timeout timer doesn’t expire once the ISR is triggered. If the timeout timer does expire once the ISR is triggered then the DMediaDriverNand::NandDfcFunction may be added to the DFC queue more than once for one hardware operation (see both DMediaDriverNand::TimeoutTimerCallBack and NandIsr, NandInterrupt::Service add DMediaDriverNand::NandDfcFunction below). The compiler would probably reset the status flags iTimeoutActive & iDfcQueued close to cancelling the timer and queuing the DFC function but, if this behaviour were relaxed, would this cause problems? Updating  iTimeoutActive out of sequence is unlikely to cause problems. If the flag were updated too soon this would stop DMediaDriverNand::TimeoutIDfcFunction from cancelling the timer but this wouldn’t matter as the ISR would do this soon enough anyhow. If the flag were updated too late this could result in DMediaDriverNand::TimeoutIDfcFunction cancelling the timer a second time but this wouldn’t be a problem. Updating iDfcQueued out of sequence could have the following impact. If the flag were updated too late this could result in DMediaDriverNand::TimeoutTimerCallBack queueing its dfc function. Hence DMediaDriverNand::NandDfcFunction might be added to the DFC queue more than once for one haedware operation (see DMediaDriverNand::TimeoutTimerCallBack and NandIsr, NandInterrupt::Service add DMediaDriverNand::NandDfcFunction  below). If the flag were updated too early this would stop DMediaDriverNand::TimeoutIDfcFunction from queueing its dfc function timer but this wouldn’t matter as the ISR would queue its DFC function soon enough anyhow.

DMediaDriverNand::TimeoutTimerCallBack and NandIsr, NandInterrupt::Service add DMediaDriverNand::NandDfcFunction

In the unlikely event that the interrupt fires close to the timeout then it is possible for the media driver to try and add DMediaDriverNand::NandDfcFunction to the DFC queue twice for one NAND operation.  However, this may not cause any difficulty as TDfc::Add () does nothing if the DFC is already queued. This could only cause a problem if the DFC gets added and scheduled and then added again. The driver needs to be analysed to see if a fix is needed and to provide one if need be.

Q8) Does the media driver make use of Shared chunks or Static variables to provide signaling or data transfer between threads/processes?  If so has a suitable combination of spin locks and memory barriers been applied to ensure data integrity, where appropriate? Have you considered ISRs and timer callbacks as well as main driver threads?

A8) The media driver doesn’t make use of shared chunks at all. The media driver’s main thread, ISR and timer callback functions don’t make use of static variables to provide signaling or data transfer between themselves.

Q9) Does the media driver use a dedicated DFC queue? If not has this been fixed?

A9) The media driver uses its own dedicated DFC queue.

Q10) Has the media driver ISR been checked for the ‘Early Interrupt’ or ‘Disappearing Interrupt’ problems? If either were found, has the ISR been fixed?

A10) The media driver ISR cannot manifest the ‘Early Interrupt’ problem; it doesn’t use an interrupt reason variable. The media driver ISR cannot manifest the ‘Disappearing Interrrupt’ problem: it doesn’t use an interrupt status register.

Q11) Is the media driver using MHA accessor methods to access hardware registers? Where such methods can’t be used are spinlocks used when registers are accessed as appropriate?

A11) The media driver doesn’t use MHA accessor methods to access hardware registers. MHA accessor methods may be appropriate when the media driver gets compiled in the Symbian OS environment but wouldn’t be aoppropriate when the media driver gets compiled in an OS less environment. Perhaps MHA could be used or spinlocks could provided around hardware accesses to allow for unlikely possibility that another driver might implement access to the same hardware sometime in t the future. It is recommended that general work in this area would result in effort being expended for no real gain. It is not known what the performance impact of such general work would be. (Andrei’s code accesses controller registers and SRAM buffer at many different places through a variety of accessor functions. See calls to NcRegWrite16(), NcRegRead16(), NcSetPageAddr(), NcClearBit(), NcClearBit,  NcClearBit,  NcSetCmd() and PAM_memcpy(). These accessor functions could have spinlocks added. Alternatively MHA access could be used in these calls.)

Q12) Does he media driver exhibit any register access ordering issues and, if so, have they been resolved by adding spin locks?

A12) There aren’t any register access ordering issues since the media driver always accesses NAND controller registers from the single main thread.

Q13) Have any media driver shutdown issues been documented?

A13) There is some work to do here. Verify that DMediaDriverNand::Close() gets called on formatting a NAND drive or as the result of some other operation. If it does consider calling the following in the Close() routine.

- TDfc::Cancel() for TDfc objects iNandIreqDfc, iTimeoutIDfc & iTimerDfc

- NTimer::Cancel for Ntimer objects iTimeoutTimer & IdleTimeout

- Media driver  complete routine

- DMediaDriver::Close()

Note that the use of NTimer objects in a media driver cause an SMP safe issue for which there is currently no available solution.

Q14) Where the media driver uses spinlocks, are they being used appropriately?

A14) The media driver doesn’t currently use spinlocks.

Q15) Does the media driver make any unsuitable assumptions about P&S message ordering?  If so has this been fixed?

A15) The media driver doesn’t use publish and subscribe messages.

  - Internal RAM Driver

No SMP specific changes are required.

The RAM Driver is a simple synchronous device driver that makes use of the Kernel MMU object TInternalRamDrive. TInternalRamDrive is fundamentally a shared chunk of memory used to represent the RAM drive; in addition the object provides a Mutex that can be used to guard access to the memory.

The RAM driver acquires the TInternalRamDrive Mutex before performing any operation; once an operation is complete it releases the Mutex, this prevents synchronous access to the Shared memory chunk.

For read/write access the RAM driver makes use of the official IPC mechanisms ReadRemote() & WriteRemote().

The Ram driver does not access any hardware directly, nor make use of Interrupts, Dfc’s or call-backs;