Base How To BTrace

| Status: | Released |
| --- | --- |
| Version: | 1.0 |
| Team/Department : | Kernel/Core Development |

Contents

1	Introduction	3

1.1	Architecture	3

1.2	Secondary Filter	3

2	Generating Traces Within Source Code	3

2.1	Basic Trace Macros	3

2.2	Traces With Context ID and PC Values	4

2.3	Traces filtered by UID value	4

3	Trace Record Structure	4

3.1	Header	4

3.1.1	Size	4

3.1.2	Flags	5

3.1.3	Category	5

3.1.4	Sub-Category	5

3.2	Header Extensions	5

3.2.1	Header2	5

3.2.2	Timestamp	5

3.2.3	Timestamp2	5

3.2.4	Context ID	5

3.2.5	Program Counter	6

3.2.6	Extra	6

3.3	Data	6

3.4	Multipart Traces	6

4	Configuration	6

4.1	Compiling the Kernel	6

4.2	Including Symbian's BTrace Handler	7

4.2.1	Full OS ROMs	7

4.2.2	Base Text-Shell ROMs	7

4.3	Configuring BTrace During ROM Building	7

4.4	BTrace on the Emulator	7

5	Extracting Trace Data From a Device	8

6	Base Porting Information	8

6.1	Implementing a BTrace Handler	8

6.1.1	IMPORTANT NOTES.	10

6.2	Implementing a BTrace Control Handler.	10

7	Kernel Performance logging macros	10

8	Further Information	10

8.1	References	10

8.2	Open Issues	11

8.3	Glossary	11

8.4	Document History	11

Appendix A -	12

A.1	Multipart Trace Structure	12

# Introduction

BTrace is a kernel service which provides a means of generating and capturing trace information in a manner designed to minimise the impact on a running system.

## Architecture

Both the Kernel and EUSER implement the BTrace class which is used for generating traces, (see e32btrace.h). Each trace has an 8 bit Category value, and the kernel implements a filter which enables traces in each category to be either immediately discarded or passed on to the BTrace handler. This handler is then responsible for further dealing with the trace data; typically this will involve storing it in a memory buffer or transmitting it off the device via a fast communication link.

Symbian provides a BTrace handler which stores traces in a ring buffer located in a Shared Chunk. This handler is split into two parts.

BTRACEX.LDD - This is an LDD/kernel-extension which adds timestamps to each trace then stores them in a memory buffer.

BTRACEC.DLL - This is a user-side client DLL which provides an interface for extracting trace data from the memory buffer. This interface is the class RBTrace, which is defined in d32btrace.h.

A primitive Symbian OS command-line utility is also provided to control and extract BTrace data. See 5

## Secondary Filter

In addition to filtering each trace based on its category (primary filtering) a secondary run-time filter may be optionally applied to traces. (See 2.3.) This examines the first 32 bits of data in the trace and if this value isn't present in the list maintained in the secondary filter, the trace is discarded. Values used for secondary filtering must be Symbian Unique Identifiers (UIDs) allocated using the normal UID allocation process. Note, the following non-valid UID value ranges are reserved.

0x00000000..0x007fffff		Reserved for platform specific use.

0x00800000..0x00ffffff		Reserved for use by Symbian.

The secondary filter may be tested independently of trace output methods by using the BTrace::CheckFilter2 method. This enables the filter to be checked before any costly pre-processing of trace data.

# Generating Traces Within Source Code

To include trace generation into source code, there are a family of C pre-processor macros defined in e32btrace.h. All of these macros take a Category and Sub-Category value as their first two arguments, the other arguments form the data for the trace record to be generated.

The Category value indicates the trace type and is a value from the enumeration BTrace::TCategory, the Sub-Category value is an enumeration specific to the category.

## Basic Trace Macros

Here are the basic macros used for generating traces:

BTrace0 (aCategory,aSubCategory)

BTrace4 (aCategory,aSubCategory,a1)

BTrace8 (aCategory,aSubCategory,a1,a2)

BTrace12(aCategory,aSubCategory,a1,a2,a3)

These output traces with arbitrary data of 0, 4, 8 or 12 bytes as contained in the 32bit integer arguments a1-a3.

For longer traces use:

BTraceN (aCategory,aSubCategory,a1,a2,aData,aDataSize)

This generates trace data of the 8 bytes specified by a1 and a2, followed by the aDataSize bytes stored at address aData. If aDataSize is greater than 80 (KMaxBTraceDataArray), then the data is truncated to this size and the trace record header has a flag set to indicate that this has occurred. See 3.1.2

For traces without size restrictions the 'Big' macro is provided.

BTraceBig(aCategory,aSubCategory,a1,aData,aDataSize)

This is similar to the previous BTraceN except that rather than truncating data which is too big, it is split up into multiple separate traces. See Multipart Traces in section 3.4

## Traces With Context ID and PC Values

There are variants of each of the basic macros described in the previous section which generate traces with additional context information added. This context information can be either, or both, of the following.

- Context ID - This indicates which thread generated the trace, it's value is the address of the kernel's NThread object for the thread. If the trace came from an Interrupt Service Routine (ISR) or Immediate Delayed Function Call (IDFC) then the Context ID has a special value which has non-zero bits in the two least-significant bit positions. See 3.2.4

- Program Counter (PC) Value - This gives the address of the CPU instruction which will be executed next, after the call to the trace output function. This address should normally be inside the function in which the trace macro was used but this cannot be guaranteed as the compiler may perform tail recursion or out-of-line optimisations.

The macro names to generate traces with context information are of the form BTraceContextX, BTracePcX, and BTraceContextPcX.. Where X is '0', '4', '8', '12', 'N' or 'Big'.

## Traces filtered by UID value

Traces may also be optionally sent through a second level of run-time filtering. This treats the first 32 bits of data in the trace as a Unique Identifier (UID) value and, if this UID isn't present in the list maintained by the secondary filter, it is discarded.

The macros used to generate such traces follow the same form as those mentioned above but they have the 'Filtered' added to their names; these are BTraceFilteredX, BTraceFilteredContextX, BTraceFilteredPcX, and BTraceFilteredContextPcX.. Where X is '4', '8', '12', 'N' or 'Big'.

For example:

BTraceFiltered8(aCategory,aSubCategory,aUid,a2)

This would generate a trace containing 8 bytes of data: the two 32bit values aUid  and a2.

# Trace Record Structure

Each trace generates a record consisting of three parts: a header, header extensions, and the trace data itself.

## Header

The header consists of four bytes containing, in this order, Size, Flags, Category and Sub-Category,

### Size

The size is the total number of bytes in the trace record including the header, header extension and trace data. This will not exceed the value KMaxBTraceRecordSize.

When trace records are stored in memory they are stored word (32 bit) aligned. Therefore the size must be rounded up to a multiple of four when calculating the address of the next record. E.g.

TUint8* record;   // pointer to a trace record

TInt size = record[BTrace::ESizeIndex];

record += (size+3)&~3;   // move record pointer on to next record.

The BTrace::NextRecord() method is provided to do this operation.

### Flags

This is a bitmask of values from the BTrace::TFlags enumeration.

Bits 0 to 5 indicate which of the 6 extension header words are present. (See 3.2)

Bit 6 (ERecordTruncated) is set if the trace data for this record was truncated to keep the size within the maximum permissible.

Bit 7 (EMissingRecord) is set to indicate that trace record(s) before this one are missing. This can happen if the trace buffer was full when a trace output was attempted.

### Category

This indicates which category the trace falls into. This is a value from the BTrace::TCategory enumeration.

Each category has its own 'filter' bit, which if set to zero means that all traces that category are not recorded. These filter bits can be manipulated by the kernel function BTrace::SetFilter. They can also have their initial values set when a ROM is  built, (see 4.3).

### Sub-Category

The meaning of this value is dependent of the Category value.

## Header Extensions

Following the header are optionally a number of 32 bit 'header extension' values. These are present in the order shown below but only exist if the appropriate flag bit is set in the Header.

### Header2

Contains flag values from the BTrace::TFlags2 enumeration.

This value is only present if the EHeader2Present flag is set.

### Timestamp

A timestamp value indicating when the trace was generated. The format and resolution of this value are platform dependant.

Symbian's BTrace handler uses the same values as would be returned by User::FastCounter() or NKern::FastCounter().

This value is only present if the ETimestampPresent flag is set.

### Timestamp2

Additional timestamp information. E.g. the most significant half of a 64bit timestamp value. Note, it is valid for a Timestamp2 value to be present even if the previous Timestamp is absent.

This value is only present if the ETimestamp2Present flag is set.

### Context ID

This value indicates the context in which the trace was generated. The meaning of the id is dependant on the contents of the two least significant bits:

- indicates the value is the address of the NThread object for the currently executing thread.

- indicates Fast Interrupt (FIQ) context. Other bits of the value are currently reserved for future use.

- indicates Interrupt (IRQ) context. Other bits of the value are currently reserved for future use.

- indicates Immediate Delayed Function Call (IDFC) context. Other bits of the value are currently reserved for future use.

This value is only present if the EContextIdPresent flag is set.

### Program Counter

This is the memory address of the CPU instruction after the location the trace was output.

This value is only present if the EPcPresent flag is set.

### Extra

An extra value used for different purposes depending on the trace type.

This value is only present if the EExtraPresent flag is set.

## Data

Following the header extensions are 0 or more bytes of trace data specified when the trace was output.

E.g. if the trace was generated with the macro

BTrace8(aCategory,aSubCategory,a1,a2)

the data will contain the 8 bytes specified by the two 32bit integers a1 and a2. (Data is always little-endian.)

## Multipart Traces

When a 'Big' trace is generated which doesn't fit into a single trace record its data is split into several separate trace records; a multipart trace. The structure of these trace records is described here.

- Header2 is present, with the bits contained by bitmask EMultipartFlagMask holding a value from the enumeration TMultiPart. This indicates whether a trace is the first, last or a middle part of a multipart trace.

- The Extra value is present in the header extension, this contains a unique identifier which is the same in all parts of the trace.

- The first 4 bytes of trace data contain a 32bit integer giving the size of the original trace data before it was split into multiple parts. (This is the same value as the aDataSize argument in the BTraceBig macro.)

- The second 4 bytes of trace data contain another 32bit integer. In the first trace part, this is the value of the a1 argument given to BTraceBig. In subsequent parts, this value is the byte offset within the original trace data from which the data in this particular trace was taken.

- The rest of the bytes of trace data contain a section from the original data.

A more formal description of the contents is contained in Appendix A.1

# Configuration

## Compiling the Kernel

The BTrace APIs are provided by the kernel in both debug and release builds, however, traces generated by the kernel in release builds may be configured using conditional compilation macros. The mechanism for this is to include 'macro' statements into the variant.mmh file for the base port. These macros have names in the form BTRACE_XXX where XXX is a name describing a trace category. (See e32\include\nkern\nk_trace.h.)

A single macro, BTRACE_KERNEL_ALL, can be used to enable all supported trace categories. Symbian's base-ports include this macro, however, on production devices it is assumed that this will be omitted.

## Including Symbian's BTrace Handler

### Full OS ROMs

Symbian's BTrace handler can be added to ROMs as a kernel extension, which means it is loaded and activated early during boot. To achieve this, the BTRACE macro needs to be defined when preprocessing the OBY file(s), e.g. by specifying –DBTRACE on the buildrom command line.

To include the required binaries as normal loadable files the file BTRACE.IBY should be included from another IBY file. In this case the handler will not be loaded until RBTrace::Open() is called.

### Base Text-Shell ROMs

The BTrace binaries are always included in Text-Shell ROMs. To exclude the client DLL, define the macro EXCLUDE_BTRACE, e.g.

ROM rom -variant=syborg -inst=armv5 -build=udeb -d=EXCLUDE_BTRACE

## Configuring BTrace During ROM Building

ROMBUILD supports additional keywords for configuring BTrace behaviour. These can be added to any OBY file.

| Keyword | Description |
| --- | --- |
| BTrace N0 [N1 .. N7] | Sets the initial state of the fast trace filter; arguments are between one and eight 32bit integers. Each bit in these values is a flag, which if set, indicates that a trace category is enabled. The flag for trace category C is contained in integer N[C/32] at bit position C%32. |
| BTraceMode N | Set the initial mode for the BTrace handler to the integer N. For Symbian's handler use values from RBTrace::TMode |
| BTraceBuffer N | Set the initial size for any memory buffer used to store trace data. |

## BTrace on the Emulator

Symbian's BTrace handler is always loaded and initialised at Emulator boot.

The behaviour of the handler can be configured using keywords in the emulator's INI file. These settings have the same syntax as ROMBUILD settings described in the previous section, except that the trace filter is set by eight separate keywords, BTrace0 through BTrace7, rather than a single keyword taking 8 arguments.

# Extracting Trace Data From a Device

BTRACE.EXE is primitive Symbian OS command-line utility which enables control and extraction of BTrace data from Symbian's BTrace handler. It is included in all Base Text-Shell ROMs but must be manually added to other kinds of ROM.

The syntax for usage is:

BTRACE [-fLIST] [-mN] [-bN] [-d] [filename]

-fLIST   Set category filter to a LIST of comma separated category numbers.

This argument may be used more than once e.g. -f1,22,3 -f44

-mN      Set capture mode to value N. (See RBTrace::TMode).

-bN      Set capture buffer size to N kBytes.

-d       Dump contents of trace buffer to debug port.

filename File to dump contents of trace buffer to.

Example:

BTRACE -f1,2,3 -m3 -b1024

MyTestProgram.exe

BTRACE d:\MyLog.txt

The first line of this:

- Enables trace categories 1, 2 and 3.

- Sets the buffer to 'free running' mode (and enabled).

- Sets the trace buffer size to 1024 kBytes.

The second line runs a test program MyTestProgram.exe.

The third line extracts all of the trace data from the buffer and writes it to file d:\MyLog.txt.

Trace can also be sent to the devices debug port (in addition to, or instead of, a file.) by adding the -d option. E.g.

BTRACE d:\MyLog.txt -d

or

BTRACE -d

# Base Porting Information

This section deals with issues involved in Base Porting and kernel-side programming.

## Implementing a BTrace Handler

The kernel passes all trace data, which has not been filtered out, onto the BTrace handler via a callback. The prototype for this callback function is BTrace::THandler, which takes eight TUint32 arguments listed below.

| Argument | Description |
| --- | --- |
| aHeader | The 4 bytes for the trace header. |
| aHeader2 | The second header word. (If EHeader2Present is set in the header flags.) |
| aContext | The context id. (If EContextIdPresent is set in the header flags.) |
| a1 | The first four bytes of trace data. (Only if the header size indicates that this is present.) |
| a2 | The second four bytes of trace data. (Only if the header size indicates that this is present.) |
| a3 | This is either the third four bytes of trace data, if the header size indicates there is between 9 and 12 bytes of trace data. If more than 12 of trace data are indicated, then a3 contains a pointer to the remaining trace data, bytes 8 trough to N. This trace data is word aligned. |
| aExtra | The 'extra' value. (If EExtraPresent is set in the header flags.) |
| aPc | The Program Counter value. (If EPcPresent is set in the header flags.) |

The trace handler should return True, if the trace handler is enabled and handling the trace data, False otherwise.

Here is an example implementation of a trace handler, (where the Output() function is defined to store/transmit 32 bits of data.)

TInt size = (aHeader>>BTrace::ESizeIndex*8)&0xff;

TInt flags = (aHeader>>BTrace::EFlagsIndex*8)&0xff;

Output(aHeader), size -= 4;

if(flags&BTrace::EHeader2Present)

Output(aHeader2), size -= 4;

if(flags&BTrace::EContextIdPresent)

Output(aContext), size -= 4;

if(flags&BTrace::EPcPresent)

Output(aPc), size -= 4;

if(flags&BTrace::EExtraPresent)

Output(aExtra), size -= 4;

if(size<=0)

return ETrue;

Output(a1), size -= 4;

if(size<=0)

return ETrue;

Output(a2), size -= 4;

if(size<=0)

return ETrue;

if(size<=4)

Output(a3);

else

{

TUint32* data = (TUint32*)a3;

TUint32* end = (TUint32*)(a3+size);

do Output(*data++); while(data<end);

}

return ETrue;

The trace handler may add timestamp values to the trace it outputs, in which case it should modify the flags and size in aHeader accordingly, before outputting this value. The Timestamp and/or Timestamp2 should be output, in that order, between the Header2 and Context ID values.

There can only be one handler in the system and this must be registered with the kernel by using the function:

BTrace::SetHandler(BTrace::THandler aHandler);

### IMPORTANT NOTES.

- The handler must not make use of any kernel APIs, apart from TDfc::RawAdd(). (This is 	because kernel APIs may themselves generate tracing and this would result in deadlock of the system.)

- The trace handler must not access or modify arguments which are not indicated as being present by their flag in aHeader or aHeader2. In particular, on ARM CPUs, the values a2, a3, aExtra and aPc are stored on the stack and the caller of this function may not have created these arguments before calling the trace handler. This breaks the ARM Procedure Call Standard, so it is theoretically possible that if the trace handler is written in C/C++ the compiler may generate code which violate the above conditions, even if the source code itself doesn't.

- The handler may be called in any context and in a re-entrant manner. Implementations must be designed to cope with this.

- The interrupt disable status must not be reduced by the trace handler during its operation. E.g. if IRQs are disabled on entry, the handler must not cause them to become enabled at any point.

## Implementing a BTrace Control Handler.

The BTrace APIs also have the concept of a separate 'control handler', which is used to communicate with a particular BTrace implementation. See BTrace::SetHandlers,  BTrace::Control , and BTrace::TControl.

This handler has callbacks so a crash debugger can request the contents of any memory buffers used to store BTrace records. If an implementation doesn't make use of memory for storing trace data it does not need to implement this feature, or can choose to do so and return KErrNotSupported.

This control API may be extended in future to support new features.

# Kernel Performance logging macros

A set of macros PERF_LOGXXX was created to enable simple logging of the kernel-side code performance.

These macros are just wrappers around original BTraceXXX ones. The macros are:

PERF_LOG0(aSubCategory)

PERF_LOG1(aSubCategory,aUserData)

PERF_LOG2(aSubCategory,aUserData,aUserData2)

Where aSubCategory is a user-defined subcategory for BTrace::EKernPerfLog logging category; aUserData and aUserData2 parameters represent arbitrary user data words which will get to the logging trace.

The resultant trace will consist of: Context Id, PC value, Tick Count and optional aUserData and aUserData2. Depending on the concrete implementation of the tracing mechanism the trace may contain Fast Counter Timestamp value.

All these macros are defined in \e32\include\kernperflogger.h header file.

# Further Information

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |

## Open Issues

## Glossary

The following technical terms and abbreviations are used within this document.

| Term | Definition |
| --- | --- |

## Document History

| Date | Version | Status | Description |
| --- | --- | --- | --- |
| 07-08-2007 | 1.10 | Released | Added description of BTrace::Control |
| 05-07-2006 | 1.00 | Released | Added description of the secondary filter. (1.2 and 2.3). |
| 02-03-2006 | 0.11 | Draft | Added a chapter about Kernel Performance logging. |
| 14-12-2005 | 0.10 | Draft | Initial draft |

  - Multipart Trace Structure

The structure of the data part of each trace record in a multipart trace is described below. In this description, the following labels are used.

A is the initial 4 bytes of data; the a1 argument of BTraceBig.

D is the array of bytes of additional data; the aData argument of BTraceBig.

N is the size of D; the aDataSize argument of BTraceBig

X is the maximum number of additional bytes which will fit into a trace record. This is usually KMaxBTraceDataArray but this should not be assumed, instead the size and other information present in each trace record should be examined.

For the first part of a multipart trace, the data in a trace record has the following structure:

- 4 bytes containing N.

- 4 bytes containing A.

- X bytes containing D[0..X-1]

If the parts are numbered 0 through to 'j', then a middle part of a multipart trace is numbered 'i' where 0<i<j. The data in these parts has the structure:

- 4 bytes containing N.

- 4 bytes containing X*i. I.e. the offset within D for the data in this trace record.

- X bytes containing D[X*i..X*i+X-1]

For the last part of a multipart trace, the data has the structure:

- 4 bytes containing N.

- 4 bytes containing X*j. I.e. the offset within D for the data in this trace record.

- N modulo X  bytes containing D[X*j..N-1]. I.e. the final bytes of the trace data.